MSP - 016

PROCEDURE FOR THE RELEASING OF PATIENT RESULTS

- TABLE OF CONTENTS

TABLE OF CONTENTS	2

APPROVAL AUTHORITY	3

RECORD OF AMENDMENTS	4

DISTRIBUTION LIST	5

LIST OF ABBREVIATIONS	6

1.	Objective	7

2.	Scope	7

3.	Responsibilities	7

4.	Definitions	7

5.	Procedure	8

5.1	Report Format	8

5.2	Authorized Signatories	8

5.3	Result Transfer & Approval	9

5.4	Test Report Dispatch	9

5.5	Notification of Delayed Examination to Requester	10

5.6	Communicating Normal, Urgent, Abnormal & Critical results	10

5.7	Releasing Preliminary/Provisional Report	12

5.8	Task-wise Responsibilities	12

6.	Quality Control	13

7.	Related Documents	13

8.	Reference Regulations & Guidelines	13

9.	Records Management	13

- APPROVAL AUTHORITY

| Prepared by: | Quality Manager | Dr. Sai Varun Name | Date: 14/11/2024 |
| --- | --- | --- | --- |
|  |  | Signature |  |
| Reviewed & Issued by: | Laboratory Head | Dr. Abdul Aleem Name | Date: 14/11/2024 |
|  |  | Signature |  |
| Approved by: | Chairman & Managing Director | Mr. Venkata Cherukuri Name | Date: 14/11/2024 |
|  |  | Signature |  |

- RECORD OF AMENDMENTS

| S. No. | Ver. No. | Ver. Date | Rev. No. | Rev. Date | Description of Changes |
| --- | --- | --- | --- | --- | --- |

| Document Copy No. | Name / Designation of the holder of the document copy | Type of Copy | Version No. | Revision No. |
| --- | --- | --- | --- | --- |
| 01/10 | Managing Director | Controlled Copy | 01 | 00 |
| 02/10 |  |  |  |  |
| 03/10 |  |  |  |  |
| 04/10 |  |  |  |  |
| 05/10 |  |  |  |  |
| 06/10 |  |  |  |  |
| 07/10 |  |  |  |  |
| 08/10 |  |  |  |  |
| 09/10 |  |  |  |  |
| 10/10 |  |  |  |  |

- DISTRIBUTION LIST

- LIST OF ABBREVIATIONS

| S. No. | Abbreviations | Expansion |
| --- | --- | --- |
| 1 | CSD | Customer Service Department |
| 2 | HIV | Human Immunodeficiency Virus |
| 3 | ID | Identification |
| 4 | FOM | Form |
| 5 | GE | General |
| 6 | ISO | International Organization for Standardization |
| 7 | LIMS | Laboratory Information Management System |
| 8 | LIP | Licensed Independent Practitioner |
| 9 | MAN | Manual |
| 10 | MD | Doctor of Medicine |
| 11 | MSP | Management System Procedure |
| 12 | NABL | National Accreditation Board for Testing and Calibration Laboratories |
| 13 | OTP | One Time Password |
| 14 | PhD | Doctor of Philosophy |
| 15 | REG | Register |
| 16 | QR Code | Quick Response Code |

Objective

- To provide the laboratory staff with guidelines for releasing the patient results while ensuring that the result reporting process is performed adequately and with appropriate quality control and quality assurance, in a manner that reduces errors or misreporting of results while maintaining patient confidentiality.

Scope

- This procedure applies to all patient reports released for tests conducted across all departments of TRUSTlab.

Responsibilities

  - Laboratory Head/Designee

  - Laboratory Supervisor/Senior

  - Laboratory Quality Manager

  - Applicable to: All Laboratory Staff

Definitions

Sample: A biological specimen of a patient/test-requestor that is received by TRUSTlab for specific diagnostic testing.

Normal test result: Test result that falls within the normal parameters for the particular test established by the laboratory. Requires patient notification but not on an immediate basis.

Abnormal test result: Test result that requires the ordering provider’s attention as soon as possible but is not as urgent or life-threatening as a critical result. Abnormal findings are values that are above or below the established norms for a particular test.

Critical test result: Test result for a condition that if left untreated may be life-threatening or place the patient at serious risk. Patients require urgent clinical attention.

Urgent test result: An urgent test result is a laboratory finding that requires immediate attention due to its potential impact on a patient’s health. The urgent test results are requested as they may require prompt medical intervention to prevent serious complications. These results are neither life threatening nor high risk to patients care.

Electronic communication: Communication of test results by e-mail, WhatsApp, online through TRUSTlab website or TRUSTlife App or other electronic means.

Preliminary Report: Certain tests in some departments require the release of an initial report, e.g. culture reports that require a preliminary 24-hour report, before a later and final report is released, say after 72 hours.

Provisional Report: A laboratory report is considered Provisional until it is reviewed, edited, and signed by an interpreting physician. In many laboratories, a printed or electronic version of the Provisional report serves as a template for the final report. This typically includes the relevant patient information, indications for the test, a description of the test performed, a summary of the results, and a preliminary interpretation. The interpreting physician can then edit or add to this version to create the final report.

Procedure

Report Format

- Each Test Result Report Form (TDPL/GE/FOM-045) includes (where relevant):

  - Patient Information: UHID, Patient/Client Name, age, gender, SRF #, Aadhar #

  - Sample Information: Visit ID, sample drawn date, received date, reported date, report status, barcode

  - Client/Doctor Information: Referral Doctor, Client Code, Client Name

  - Laboratory Information: Name and address of the processing laboratory

  - Department Information: Department name

  - Test Information: Test name, clear and concise results of tests analyses, units, biological reference range, method

  - Comments section

  - Signature and title of the authorized reviewer/s

  - Page number (In the format: page <current page #> of <total #> pages)

  - QR code which when scanned will lead to the electronic records to authenticate the report

  - Electronic round-seal of TRUSTlab.

Authorized Signatories

  - Every TRUSTlab report is signed by a maximum of two (2) authorized signatories, either two MD’s or one MD and one PhD.

  - A controlled copy of the list of Approved Signatures is maintained in the Authorised Specimen Signatures Form (TDPL/QA/FOM-017) which is in the custody of the Quality Manager, so that the approving signatures for Reports can be identified.

Result Transfer & Approval

  - At TRUSTlab, the interfacing of our LIMS with most equipment enables the automatic transfer of test results to LIMS. If any equipment is not interfaced, results yielded from such instruments are manually entered into the Results/Report section in the LIMS software.

  - Most tests conducted at TRUSTlab have a predefined “reference range”, derived from the kit insert or reference medical books/journals.

  - If the patient result falls within the predefined “reference range”, the patient report is automatically released within two (2) minutes of the results being saved in the LIMS software (wherever applicable).

  - If the patient results fall beyond the normal reference range, the report will be released ONLY after review by the Head of the Department or by a technically competent department authorized signatory.

  - If no “reference range” is set in the LIMS software, the test result will not be automatically released and will instead require manual release.

Test Report Dispatch

- At TRUSTlab, all results are reported in a printable format, except for critical values, which are first communicated via telephone to authorized persons at the patient’s end, followed by an electronic report.

- Under normal circumstances, TRUSTlab releases the results directly to patients by physically handing over a printed report or through electronic means such as WhatsApp, email, authorized access to LIMS, the TRUSTlab web account, or the TRUSTlife app.

- When providing a printed report to a patient or customer, TRUSTlab staff will request a government-issued identification, verify the details against the test report, make a photocopy of the identification for records, and obtain a signed duplicate photocopy from the patient or customer and a signature in the Test Report Issue Register (TDPL/GE/REG-004) before releasing the report.

- An electronic format of a test report will be sent only to the WhatsApp number registered in the Test Requisition Form (TDPL/GE/FOM-040) signed by the patient or customer. The electronic copy of the report is also available on the TRUSTlab website or TRUSTlife platform which is secured with a password set by the patient or customer during registration and/or a one-time password (OTP) sent to their registered mobile number.

- In exceptional cases where the law prohibits TRUSTlab from releasing the report directly to the patient, it is provided to the treating physician or a counselor (e.g., A report that states an HIV positive result).

Notification of Delayed Examination to Requester

- TRUSTlab strives to minimize turnaround time for tests, ensuring that patients or customers can begin care without delay. The turnaround time of each test is listed in the TRUSTlab Directory of Services, available online at edos.mytrustlab.com.

- In cases where the results have been delayed to an extent that there is a significant compromise to patient care the lab manager will notify the requester giving reasons for the delay and the expected date of release of the report.

Communicating Normal, Urgent, Abnormal & Critical results

- Normal Report

- It is ensured that normal results are communicated within a reasonable time frame. The following points are to be noted:

- Normal results are communicated to the patient or customer by physical and/or electronic communication only.

- If the patient does not have a WhatsApp number, a text (SMS) message is sent to inform them of the test result’s readiness.

- Results are not conveyed verbally in-person or by telephone or by leaving a message on voicemail/answering machine.

- Abnormal results

- Abnormal results may be communicated to the test requestor by direct verbal communication or electronic communication once the report is released if the HoD deems it necessary based on certain conditions, e.g. if the results do not clinically correlate with the patient details.

- If TRUSTlab Customer Service Department (CSD) staff are unable to contact the physician to communicate verbally, the information about the abnormal result is relayed electronically via a WhatsApp/SMS message.

- Urgent results

  - Urgent results are not communicated via verbal communication to either the requestor or the patient.

  - Urgent results are given high priority, department supervisor will notify the head of the department once the results become available and upon approval will send the same to the requestor. A follow-up call will be made to the requestor, to announce the release of the report, within 30 minutes of releasing the report.

  - All communication or attempts to communicate must be documented.

- Critical results

  - Critical results must be communicated immediately by direct verbal communication from TRUSTlab to the doctor/physician only (wherever possible).

  - When reporting via verbal communication, TRUSTlab staff will identify himself/herself, clearly read out the following and will request a readback of the same:

    - Name of the Patient

    - Date of Birth

    - Test Name

    - Test Results

    - Measurement units

  - TRUSTlab employee verifies the accuracy of the readback.

  - The fact that the critical result was verbally conveyed to an appropriate LIP/caregiver, in accordance with this procedure, will be documented in the Critical Call-Out Form (TDPL/GE/FOM-008) by the reporting departments. Documentation should include:

    - the critical result

    - reported to, readback and acknowledged by (first and last name of LIP/caregiver and credentials)

    - date and time

    - e.g. Tarun reported to, read back and acknowledged by Dr. Sunita on 5/8/2014 @ 1:10 PM

  - In cases where the doctor/physician is not available, results must be communicated to the next authorized person, following the established chain of responsibility.

  - The following steps should be taken when the doctor/physician cannot or does not respond to notification of a critical test result:

    - If the doctor/physician cannot or does not respond within 10 minutes, call the doctor/physician a second time.

    - If the test requestor does not respond within 15 minutes of the second call, call the hospital staff and request an alternative number of the same doctor/physician or an alternative doctor contact.

  - Critical Alert messages with the results are messaged to the test requester by WhatsApp and SMS.

  - All communication or attempts to communicate must be documented.

Releasing Preliminary/Provisional Report

- Certain situations may require the release of a Preliminary Report:

  - Certain tests in some departments require the release of a Preliminary report, e.g. culture reports that require a preliminary 24-hour report.

- A Provisional report may be issued for tests when the report has not yet been approved by the Head of the Department, but the responsible licensed caregiver requires urgent release of the results.

  - Such a situation may arise, for example, when a report is needed outside the operating hours of our laboratory, for the purpose of immediate patient care.

  - In such cases a “Provisional” watermark is placed across the report to indicate that it is only a provisional report and is pending approval. Provisional reports will not bear the signature of the authorized signatory.

Task-wise Responsibilities

| Task | Responsible |
| --- | --- |
| Reporting the correct result to the relevant patient, doctor | Department Head/Designee |
| Ensuring all staff compliance with the procedure | Laboratory Head/Lab Manager |
| Ensuring correct results are reported to all concerned parties | Laboratory Manager/Senior Tech |
| For checking & noticing reports that needs corrections | Laboratory Manager/Senior Tech |
| Overall supervision | Quality Manager |

Quality Control

- The Quality Manager or designee will do random Quality Control checks to ensure that this SOP is implemented as written. If Quality control checks reveal non-compliance with this SOP, the SOP deviations and violations will be investigated, corrective and preventive action initiated, implemented, reviewed and closed out. All corrective and preventive action should be documented in the Non-Conformity & Corrective Action Form (TDPL/GE/FOM-021).

Related Documents

- N/A.

Reference Regulations & Guidelines

Specific Criteria for Accreditation of Medical Laboratories (NABL 112 A; Issue 01)

Guidance document: Medical Laboratories (NABL 112B; Issue 01)

ISO 15189:2022 Medical Laboratories – Requirements for Quality and Competence

Records Management

Test Result Report Form (TDPL/GE/FOM-045)

Authorised Specimen Signatures Form (TDPL/QA/FOM-017)

Test Requisition Form (TDPL/GE/FOM-040)

Test Report Issue Register (TDPL/GE/REG-004)

Critical Call-Out Form (TDPL/GE/FOM-008)

Non-Conformity & Corrective Action Form (TDPL/GE/FOM-021)