SOP – 011

PROCEDURE FOR

INDIRECT BILIRUBIN TEST

- TABLE OF CONTENTS

TABLE OF CONTENTS	2

APPROVAL AUTHORITY	3

RECORD OF AMENDMENTS	4

DISTRIBUTION LIST	5

LIST OF ABBREVIATIONS	6

1.	Objective	7

2.	Scope	7

3.	Responsibilities	7

4.	Definition	7

5.	Test Principle	7

6.	Specific Performance Characteristics	7

7.	Type of Sample	8

8.	Type of Container & Additives	8

9.	Patient Preparation	8

10.	Equipment & Reagents	9

11.	Safety	9

12.	Calibration Procedure	9

13.	Test Procedure	9

14.	Quality Control Procedure	9

15.	Interferences & Cross Reactions	9

16.	Result Calculation	9

17.	Biological Reference Interval	9

18.	Reportable Interval	10

19.	Procedure when Results are not in Range	10

20.	Critical Alert Values	10

21.	Laboratory Clinical Interpretation	10

22.	Potential Source of Variability	10

23.	References	11

- APPROVAL AUTHORITY

| Prepared by: | Head of Department | Dr. K. Anusha Name | Date: 14/11/2024 |
| --- | --- | --- | --- |
|  |  | Signature |  |
| Reviewed & Approved by: | Laboratory Head | Dr. Abdul Aleem Name | Date: 14/11/2024 |
|  |  | Signature |  |
| Issued by: | Quality Manager | Dr. Sai Varun Name | Date: 14/11/2024 |
|  |  | Signature |  |

- RECORD OF AMENDMENTS

| S. No. | Ver. No. | Ver. Date | Rev. No. | Rev. Date | Description of Changes |
| --- | --- | --- | --- | --- | --- |

| Document Copy No. | Name / Designation of the holder of the document copy | Type of Copy | Version No. | Revision No. |
| --- | --- | --- | --- | --- |
| 01/10 | Managing Director | Controlled Copy | 01 | 00 |
| 02/10 |  |  |  |  |
| 03/10 |  |  |  |  |
| 04/10 |  |  |  |  |
| 05/10 |  |  |  |  |
| 06/10 |  |  |  |  |
| 07/10 |  |  |  |  |
| 08/10 |  |  |  |  |
| 09/10 |  |  |  |  |
| 10/10 |  |  |  |  |

- DISTRIBUTION LIST

- LIST OF ABBREVIATIONS

| S. No. | Abbreviations | Expansion |
| --- | --- | --- |
| 1 | AMR | Analytical Measurement Range |
| 2 | BC | Biochemistry |
| 3 | °C | Degree Centigrade |
| 4 | CV | Coefficient of Variation |
| 5 | CRR | Clinical Reportable Range |
| 6 | EDTA | Ethylenediaminetetraacetic Acid |
| 7 | EQAS | External Quality Assurance Scheme |
| 8 | HOD | Head of Department |
| 9 | IFCC | International Federation for Clinical Chemistry |
| 10 | IQC | Internal Quality Control |
| 11 | LLOQ | Lower Limit of Quantitation |
| 12 | MSP | Management System Procedure |
| 13 | N/A | Not Applicable/Available |
| 14 | NIST | National Institute of Standards and Technology |
| 15 | pH | Potential of Hydrogen |
| 16 | QC | Quality Control |
| 17 | QM | Quality Manager |
| 18 | QNS | Quantity Not Sufficient |
| 19 | SD | Standard Deviation |
| 20 | SOP | Standard Operating Procedure |
| 21 | SST | Serum Separator Tube |
| 22 | TBIL | Total Bilirubin |
| 23 | TRF | Test Requisition Form |
| 24 | TDPL | TRUSTlab Diagnostics Private Limited |
| 25 | ULOQ | Upper Limit of Quantitation |

- Objective

- The purpose of this SOP is to describe the procedure for the quantitative estimation of Indirect Bilirubin in serum and plasma samples using Beckman Coulter automated biochemistry analyser. This SOP also provides information on Test principles, clinical significance etc.

- Scope

- This procedure applies to all staff of the Biochemistry Department tasked with the duty of performing these tests.

- Responsibilities

It is the responsibility of the technicians of the Biochemistry Department to comply with this SOP.

HOD shall ensure implementation.

The Quality Manager is responsible for monitoring, reviewing and ensuring compliance with this procedure.

All other TRUSTlab staff involved in this process (including contract personnel) are responsible for complying with this procedure.

- Definition

- Indirect bilirubin, also known as unconjugated bilirubin, is a form of bilirubin that is not yet processed by the liver. It is produced when hemoglobin from broken-down red blood cells is processed, and it is then transported to the liver. Unlike conjugated bilirubin, it is not yet water-soluble and needs to be further processed by the liver before it can be excreted.

- Test Principle

- Indirect Bilirubin is calculated from the output values of the Total Bilirubin and Direct Bilirubin Tests. Refer to SOPs for Total Bilirubin and Direct Bilirubin (TDPL/BC/SOP-12 & TDPL/BC/SOP-10 respectively).

- Specific Performance Characteristics

Limit of Quantitation:

      - LLOQ: N/A

      - ULOQ: N/A

Analytical measurement range (AMR): N/A

Clinical Reportable Range (CRR): N/A

- Type of Sample

A minimum of 2ml of Serum/Plasma

Sample Retention: Sample is retained for 24 hours at 2 – 8℃.

Sample Rejection Criteria:

      - Quantity not sufficient (QNS)

      - Sample received in wrong vacutainer

      - No TRF/Grossly incomplete TRF/Errors in labeling

      - Grossly haemolysed specimens

      - Highly lipemic specimens

      - Highly icteric specimens

      - Sample beyond stability period

      - Incomplete TRF details or labelling errors are clarified with Customer/Doctor/ Patient before sample acceptance/processing. If critical information is still not obtained, the sample is rejected.

- Type of Container & Additives

SST - Yellow top Vacutainer

Clot activator - Red top Vacutainer

Heparin tube- Green top Vacutainer

- Patient Preparation

The phlebotomists should be courteous and compassionate towards the patients and their attendants. Ask the patient to sit comfortably. Maintain calmness and silence during blood drawing to avoid mistakes.

Check the patient’s identification; ask the patient for his or her full name, address, identification number, and/or date of birth.

No other specific preparation required.

- Equipment & Reagents

Equipment

- Beckman Coulter DxC 700 AU Chemistry Analyzer

Reagents

- As required for the estimation of Total & Direct Bilirubin.

- Safety

For in vitro diagnostic use only. Do not pipette by mouth. Exercise the normal precautions required for handling laboratory reagents.

Exposed metal surfaces should be cleaned with 1% Sodium hypochlorite.

The reagents must be used only for the purpose intended by suitably qualified laboratory personnel, under appropriate laboratory conditions.

- Calibration Procedure

- N/A.

- Test Procedure

- Refer to SOPs for Total Bilirubin and Direct Bilirubin (TDPL/BC/SOP-12 & TDPL/BC/SOP-10 respectively).

- Quality Control Procedure

- N/A

- Interferences & Cross Reactions

- N/A.

- Result Calculation

- Formula:

- Indirect Bilirubin = Total Bilirubin – Direct Bilirubin

- Biological Reference Interval

- 0.0 to 1.1 mg/dl

- Reportable Interval

Analytical Measurement Range (AMR): N/A

Clinical Reportable Range (CRR): N/A

- Procedure when Results are not in Range

- N/A.

- Critical Alert Values

- N/A

- Laboratory Clinical Interpretation

Bilirubin assays are commonly used tests to assess liver function. Daily production of unconjugated bilirubin is mainly from senescent erythrocytes. The half-life of unconjugated bilirubin is <5 minutes.

Bilirubin is typically measured in two assays for “Total” and “Direct” ; subtracting direct from total gives “indirect bilirubin”. Direct bilirubin measures the majority of delta and conjugated bilirubin and a small percentage of unconjugated bilirubin.

These assays are used in assessing liver function. Evaluating a wide range of diseases affecting production, uptake, storage, metabolism, or excretion of bilirubin.

It is increased in hepatocellular damage, Biliary obstruction, Hemolytic diseases, Neonatal physiological jaundice, Gilbert disease, Hypothyroidism.

Increased conjugated (direct) bilirubin in Hereditary diseases, Hepatic cellular damage, Biliary duct obstruction.

Increased unconjugated (indirect) bilirubin in increased biliary production, hemolytic diseases, Ineffective erythropoiesis, Blood transfusions.

- Potential Source of Variability

      - Lysed serum specimens may give falsely elevated values.

      - If the reagents are turbid, DO NOT USE, as it indicates contamination of the reagent.

      - Strong lipemic and haemolytic sera should not be used.

      - Presence of fibrin in the sample may lead to erroneous results and hence centrifuge the sample only after complete clot formation.

- References

      - Interpretations on Diagnostics Tests, 7th Edition, Jacques Wallach.

      - Tietz. Textbook of Clinical Biochemistry and Molecular Diagnostics, 4th edition.

      - Product insert of Total Bilirubin, Beckman Coulter AU680 / AU 480 / DXC700.

      - Procedure for Managing Internal QC Program (TDPL/QA/MAN-001: MSP-013).

      - Procedure for Total Bilirubin Test (TDPL/BC/SOP-012)

      - Procedure for Direct Bilirubin Test (TDPL/BC/SOP-010)