SOP – 003

PROCEDURE FOR

URINE PROTEIN

- TABLE OF CONTENTS

TABLE OF CONTENTS	2

APPROVAL AUTHORITY	3

RECORD OF AMENDMENTS	4

DISTRIBUTION LIST	5

LIST OF ABBREVIATIONS	6

1.	Objective	7

2.	Scope	7

3.	Responsibilities	7

4.	Definition	7

5.	Test Principle	7

6.	Specific Performance Characteristics	7

7.	Type of Sample	8

8.	Type of Container & Additives	8

9.	Patient Preparation	8

10.	Equipment & Reagents	8

11.	Safety	8

12.	Calibration Procedure	9

13.	Test Procedure	9

14.	Quality Control Procedure	9

15.	Interferences & Cross Reactions	9

16.	Result Calculation/Interpretation	9

17.	Biological Reference Interval	10

18.	Reportable Interval	10

19.	Procedure when Results are not in Range	10

20.	Critical Alert Values	10

21.	Laboratory Clinical Interpretation	10

22.	Potential Source of Variability	10

23.	References	10

- APPROVAL AUTHORITY

| Prepared by: | Head of Department | Dr. Abdul Aleem Name | Date: 14/11/2024 |
| --- | --- | --- | --- |
|  |  | Signature |  |
| Reviewed & Approved by: | Laboratory Head | Dr. Abdul Aleem Name | Date: 14/11/2024 |
|  |  | Signature |  |
| Issued by: | Quality Manager | Dr. Sai Varun Name | Date: 14/11/2024 |
|  |  | Signature |  |

- RECORD OF AMENDMENTS

| S. No. | Ver. No. | Ver. Date | Rev. No. | Rev. Date | Description of Changes |
| --- | --- | --- | --- | --- | --- |

| Document Copy No. | Name / Designation of the holder of the document copy | Type of Copy | Version No. | Revision No. |
| --- | --- | --- | --- | --- |
| 01/10 | Managing Director | Controlled Copy | 01 | 00 |
| 02/10 |  |  |  |  |
| 03/10 |  |  |  |  |
| 04/10 |  |  |  |  |
| 05/10 |  |  |  |  |
| 06/10 |  |  |  |  |
| 07/10 |  |  |  |  |
| 08/10 |  |  |  |  |
| 09/10 |  |  |  |  |
| 10/10 |  |  |  |  |

- DISTRIBUTION LIST

- LIST OF ABBREVIATIONS

| S. No. | Abbreviations | Expansion |
| --- | --- | --- |
| 1 | °C | Degree Centigrade |
| 2 | CP | Clinical Pathology |
| 3 | CV | Coefficient of Variation |
| 4 | EQAS | External Quality Assurance Scheme |
| 5 | HBV | Hepatitis B Virus |
| 6 | HIV | Human Immunodeficiency Virus |
| 7 | HOD | Head of Department |
| 8 | IQC | Internal Quality Control |
| 9 | MSP | Management System Procedure |
| 10 | N/A | Not Applicable/Available |
| 11 | pH | Potential of Hydrogen |
| 12 | QC | Quality Control |
| 13 | QM | Quality Manager |
| 14 | QNS | Quantity Not Sufficient |
| 15 | SOP | Standard Operating Procedure |
| 16 | TRF | Test Requisition Form |
| 17 | TDPL | TRUSTlab Diagnostics Private Limited |

- Objective

- The purpose of this SOP is to define a procedure for the qualitative identification of protein in urine by using the Precipitation by Heat and Acetic acid method. This SOP also provides information about the procedural principles, clinical significance and other details.

- Scope

- This procedure applies to all staff of the Clinical Pathology Department tasked with the duty of performing these tests.

- Responsibilities

It is the responsibility of the technicians of the Clinical Pathology Department to comply with this SOP.

HOD shall ensure implementation.

The Quality Manager is responsible for monitoring, reviewing and ensuring compliance with this procedure.

All other TRUSTlab staff involved in this process (including contract personnel) are responsible for complying with this procedure.

- Definition

- A urine protein test measures the amount of protein in your urine, which can indicate kidney function status and overall health. Normally, very little protein is present in urine, and a large amount can suggest kidney problems. This test helps in the diagnosis of persistent proteinuria indicates renal disease, either renal failure due to glomerular or tubular damage and urinary tract infection.

- Test Principle

- This test is based on the principle that heat induces coagulation of protein causing its precipitation. However, the presence of phosphates and carbonates also causes cloudiness in alkaline urine. The latter is prevented by mildly acidifying the urine.

- Specific Performance Characteristics

- N/A.

- Type of Sample

Urine is the primary specimen for the test.

Collect a minimum 15 ml of a random sample of urine.

Process the samples within 1 hour of collection. If delay is anticipated instruct patients to store the sample at 2-8°C in the refrigerator for a maximum of up to 4 hours until transport to the laboratory.

Do not accept if the sample volume is inadequate or if samples are received without an ID number.

- Type of Container & Additives

- Sterile screw-capped container.

- Patient Preparation

Instruct patients to collect urine samples in a clean dry container (preferably one provided by TRUSTlab).

Instruct patients to void urine directly into the container and while collecting allow the first portion of urine to escape.

- Equipment & Reagents

Equipment

      - Distilled water

      - Test Tubes

      - Urine Dropper

Reagents

- 3% Sulfosalicylic acid

- Safety

Follow universal precautions like wearing suitable gloves and eye/face protection while handling all reagents, samples and operating the instrument.

Universal precautions imply that all body fluids are treated as if known to be infectious for Human Immunodeficiency Virus (HIV), Hepatitis B Virus (HBV) or other Blood borne Pathogens.

Dispose of all waste in accordance with applicable national and/or local regulations.

Some reagents contain sodium azide, which may react with copper or lead plumbing to form explosive metal azides. Use caution in disposing of these reagents. If disposing to drain, flush with large volumes of water to prevent azide buildup.

Waste material containing patient samples or biological products should be considered biohazardous when disposing or treating.

Clean up all spills immediately and thoroughly. Disinfect the area for any spills involving biohazardous materials. Dispose of all contaminated materials appropriately.

- Calibration Procedure

- N/A.

- Test Procedure

- Method: Manual

      - Pour 3 ml of centrifuged urine into a test tube.

      - Add 3-4 drops of 3% Sulfosalicylic acid.

      - Invert to mix.

      - Allow to stand for 10 minutes. Invert again.

      - Grade the turbidity for presence of protein

- Quality Control Procedure

Internal Quality Control

- As defined in the procedure Urine IQC (TDPL/CP/SOP-002).

External Quality Control

- EQAS: Metropolis EQAS

- Interferences & Cross Reactions

- N/A.

- Result Calculation/Interpretation

Negative:	No cloudiness

Trace:	Barely perceptible turbidity

1+:		Distinct turbidity - No granulation

2+:		Turbidity with granulation

3+:		Turbidity with granulation and flocculation

4+:		Clumps of precipitate

- Biological Reference Interval

- Negative.

- Reportable Interval

- N/A.

- Procedure when Results are not in Range

- N/A.

- Critical Alert Values

- N/A.

- Laboratory Clinical Interpretation

This test can detect 5-10 mg/dl of protein in urine specimens and cannot differentiate between albumin and Bence-Jones protein. A positive test for protein has to be correlated with the microscopic examination of urinary sediment for pus cells, bacteria and casts.

Normal urine (in healthy individuals) contains only a small amount (less than 150 mg/24 hours) of low molecular (<68000) weight protein which cannot be detected by this method.

- Potential Source of Variability

- N/A.

- References

      - Textbook of Medical Laboratory Technology - B. Godkar, Darshan P. Godkar

      - Procedure for Managing Internal QC Program (TDPL/QA/MAN-001: MSP-013)

      - NABL 112