SOP – 004

PROCEDURE FOR

URINE pH

- TABLE OF CONTENTS

TABLE OF CONTENTS	2

APPROVAL AUTHORITY	3

RECORD OF AMENDMENTS	4

DISTRIBUTION LIST	5

LIST OF ABBREVIATIONS	6

1.	Objective	7

2.	Scope	7

3.	Responsibilities	7

4.	Definition	7

5.	Test Principle	7

6.	Specific Performance Characteristics	7

7.	Type of Sample	8

8.	Type of Container & Additives	8

9.	Patient Preparation	8

10.	Equipment & Reagents	8

11.	Safety	8

12.	Calibration Procedure	9

13.	Test Procedure	9

14.	Quality Control Procedure	9

15.	Interferences & Cross Reactions	10

16.	Result Calculation/Interpretation	10

17.	Biological Reference Interval	10

18.	Reportable Interval	10

19.	Procedure when Results are not in Range	10

20.	Critical Alert Values	10

21.	Laboratory Clinical Interpretation	10

22.	Potential Source of Variability	10

23.	References	11

- APPROVAL AUTHORITY

| Prepared by: | Head of Department | Dr. Abdul Aleem Name | Date: 14/11/2024 |
| --- | --- | --- | --- |
|  |  | Signature |  |
| Reviewed & Approved by: | Laboratory Head | Dr. Abdul Aleem Name | Date: 14/11/2024 |
|  |  | Signature |  |
| Issued by: | Quality Manager | Dr. Sai Varun Name | Date: 14/11/2024 |
|  |  | Signature |  |

- RECORD OF AMENDMENTS

| S. No. | Ver. No. | Ver. Date | Rev. No. | Rev. Date | Description of Changes |
| --- | --- | --- | --- | --- | --- |

| Document Copy No. | Name / Designation of the holder of the document copy | Type of Copy | Version No. | Revision No. |
| --- | --- | --- | --- | --- |
| 01/10 | Managing Director | Controlled Copy | 01 | 00 |
| 02/10 |  |  |  |  |
| 03/10 |  |  |  |  |
| 04/10 |  |  |  |  |
| 05/10 |  |  |  |  |
| 06/10 |  |  |  |  |
| 07/10 |  |  |  |  |
| 08/10 |  |  |  |  |
| 09/10 |  |  |  |  |
| 10/10 |  |  |  |  |

- DISTRIBUTION LIST

- LIST OF ABBREVIATIONS

| S. No. | Abbreviations | Expansion |
| --- | --- | --- |
| 1 | °C | Degree Centigrade |
| 2 | CP | Clinical Pathology |
| 3 | CV | Coefficient of Variation |
| 4 | EQAS | External Quality Assurance Scheme |
| 5 | HBV | Hepatitis B Virus |
| 6 | HIV | Human Immunodeficiency Virus |
| 7 | HOD | Head of Department |
| 8 | IQC | Internal Quality Control |
| 9 | MSP | Management System Procedure |
| 10 | N/A | Not Applicable/Available |
| 11 | pH | Potential of Hydrogen |
| 12 | QC | Quality Control |
| 13 | QM | Quality Manager |
| 14 | QNS | Quantity Not Sufficient |
| 15 | SOP | Standard Operating Procedure |
| 16 | TRF | Test Requisition Form |
| 17 | TDPL | TRUSTlab Diagnostics Private Limited |

- Objective

- The purpose of this SOP is to define a procedure for the determination of pH in urine by the Litmus paper method. This SOP also provides information about the test principles, clinical significance and other details.

- Scope

- This procedure applies to all staff of the Clinical Pathology Department tasked with the duty of performing these tests.

- Responsibilities

It is the responsibility of the technicians of the Clinical Pathology Department to comply with this SOP.

HOD shall ensure implementation.

The Quality Manager is responsible for monitoring, reviewing and ensuring compliance with this procedure.

All other TRUSTlab staff involved in this process (including contract personnel) are responsible for complying with this procedure.

- Definition

- A urine pH test measures the level of acid in urine. Some types of kidney stones are more prone to develop in alkaline urine and others are more likely to form in acidic urine. Monitoring the urine pH may be helpful in preventing the formation of kidney stones.

- Test Principle

- The pH test consists of a combination of methyl-red (pH range 4.4-6.2) and bromothymol-blue (pH range 8.0-9.6) as indicators. The color gradations extend from orange via green to blue. Depending upon the pH of urine, the color of the reaction area changes from orange to yellow and green to blue.

- Specific Performance Characteristics

- N/A.

- Type of Sample

Urine is the primary specimen for the test.

Collect a minimum 15 ml of a random sample of urine.

Process the samples within 1 hour of collection. If delay is anticipated instruct patients to store the sample at 2-8°C in the refrigerator for a maximum of up to 4 hours until transport to the laboratory.

Use fresh and un-centrifuged samples for the test.

Do not accept if the sample volume is inadequate or if samples are received without an ID number.

- Type of Container & Additives

- Sterile screw-capped container.

- Patient Preparation

Instruct patients to collect urine samples in a clean dry container (preferably one provided by TRUSTlab).

Instruct patients to void urine directly into the container and while collecting allow the first portion of urine to escape.

- Equipment & Reagents

Equipment

- N/A

Reagents

      - Blue Litmus Paper

      - pH papers of range from 2-10.5

- Safety

Follow universal precautions like wearing suitable gloves and eye/face protection while handling all reagents, samples and operating the instrument.

Universal precautions imply that all body fluids are treated as if known to be infectious for Human Immunodeficiency Virus (HIV), Hepatitis B Virus (HBV) or other Blood borne Pathogens.

Dispose of all waste in accordance with applicable national and/or local regulations.

Some reagents contain sodium azide, which may react with copper or lead plumbing to form explosive metal azides. Use caution in disposing of these reagents. If disposing to drain, flush with large volumes of water to prevent azide buildup.

Waste material containing patient samples or biological products should be considered biohazardous when disposing or treating.

Clean up all spills immediately and thoroughly. Disinfect the area for any spills involving biohazardous materials. Dispose of all contaminated materials appropriately.

- Calibration Procedure

- N/A.

- Test Procedure

Precautions:

      - Do not touch the edges of the strip where the reagent patch is present.

      - Tap excess urine from the strip by stroking along the rim of the container after dipping the strip into the sample.

      - Do not expose the strip to the atmospheric air when not in use and keep the bottle closed.

      - Protect the strips from moisture and excessive heat but do not refrigerate.

      - Replace the top on the storage container immediately after removing a pH paper.

Procedure:

      - Place a drop of urine on the Litmus paper/ pH paper using a glass rod or Pasteur pipette.

      - Note the color change and match it with the standard pH color chart provided.

      - If the blue litmus turns red, then the urine is acidic in nature. If there is no change in color, then the urine is alkaline.

- Quality Control Procedure

Internal Quality Control

- As defined in the procedure Urine IQC (TDPL/CP/SOP-002).

External Quality Control

- EQAS: Metropolis EQAS

- Interferences & Cross Reactions

- N/A.

- Result Calculation/Interpretation

- If the blue litmus turns red, then the urine is acidic in nature. If there is no change in color, then the urine is alkaline.

- Biological Reference Interval

      - pH of normal urine is acidic.

      - The range is from 4.7-7.5.

      - The average is 6.0.

- Reportable Interval

- N/A.

- Procedure when Results are not in Range

- N/A.

- Critical Alert Values

- N/A.

- Laboratory Clinical Interpretation

Decreased pH is clinically more significant than increased pH and occurs with high protein intake and ketosis. Determination of pH is useful in evaluating renal disorders such as renal failure and Renal Tubular Acidosis (RTA).

- Potential Source of Variability

Carry out examination of the urine sample when fresh, preferably within 1 hour of collection as delay will cause alterations in the chemical composition of urine (leucocytes to be destroyed, casts to decompose and urea will be broken-down due to bacterial action and make the urine alkaline).

Contamination of glassware used in testing with Sodium Hypochlorite and Bleaching powder and detergents like sodium phosphate will oxidize and change the color of chromogen in pH paper. Hence ensure the glassware is free of these chemicals.

- References

      - Handbook of Medical Laboratory Technology 2nd Editon, Edited by Robert H. Caraman, M.D. Laboratory Training Committee, Christian Medical Association of India, Page: 225

      - Procedure for Managing Internal QC Program (TDPL/QA/MAN-001: MSP-013)

      - NABL 112