SOP – 005

PROCEDURE FOR

URINE BILE PIGMENTS

- TABLE OF CONTENTS

TABLE OF CONTENTS	2

APPROVAL AUTHORITY	3

RECORD OF AMENDMENTS	4

DISTRIBUTION LIST	5

LIST OF ABBREVIATIONS	6

1.	Objective	7

2.	Scope	7

3.	Responsibilities	7

4.	Definition	7

5.	Test Principle	7

6.	Specific Performance Characteristics	7

7.	Type of Sample	8

8.	Type of Container & Additives	8

9.	Patient Preparation	8

10.	Equipment & Reagents	8

11.	Safety	8

12.	Calibration Procedure	9

13.	Test Procedure	9

14.	Quality Control Procedure	9

15.	Interferences & Cross Reactions	10

16.	Result Calculation/Interpretation	10

17.	Biological Reference Interval	10

18.	Reportable Interval	10

19.	Procedure when Results are not in Range	10

20.	Critical Alert Values	10

21.	Laboratory Clinical Interpretation	10

22.	Potential Source of Variability	10

23.	References	11

- APPROVAL AUTHORITY

| Prepared by: | Head of Department | Dr. Abdul Aleem Name | Date: 14/11/2024 |
| --- | --- | --- | --- |
|  |  | Signature |  |
| Reviewed & Approved by: | Laboratory Head | Dr. Abdul Aleem Name | Date: 14/11/2024 |
|  |  | Signature |  |
| Issued by: | Quality Manager | Dr. Sai Varun Name | Date: 14/11/2024 |
|  |  | Signature |  |

- RECORD OF AMENDMENTS

| S. No. | Ver. No. | Ver. Date | Rev. No. | Rev. Date | Description of Changes |
| --- | --- | --- | --- | --- | --- |

| Document Copy No. | Name / Designation of the holder of the document copy | Type of Copy | Version No. | Revision No. |
| --- | --- | --- | --- | --- |
| 01/10 | Managing Director | Controlled Copy | 01 | 00 |
| 02/10 |  |  |  |  |
| 03/10 |  |  |  |  |
| 04/10 |  |  |  |  |
| 05/10 |  |  |  |  |
| 06/10 |  |  |  |  |
| 07/10 |  |  |  |  |
| 08/10 |  |  |  |  |
| 09/10 |  |  |  |  |
| 10/10 |  |  |  |  |

- DISTRIBUTION LIST

- LIST OF ABBREVIATIONS

| S. No. | Abbreviations | Expansion |
| --- | --- | --- |
| 1 | °C | Degree Centigrade |
| 2 | CP | Clinical Pathology |
| 3 | CV | Coefficient of Variation |
| 4 | EQAS | External Quality Assurance Scheme |
| 5 | HBV | Hepatitis B Virus |
| 6 | HIV | Human Immunodeficiency Virus |
| 7 | HOD | Head of Department |
| 8 | IQC | Internal Quality Control |
| 9 | MSP | Management System Procedure |
| 10 | N/A | Not Applicable/Available |
| 11 | pH | Potential of Hydrogen |
| 12 | QC | Quality Control |
| 13 | QM | Quality Manager |
| 14 | QNS | Quantity Not Sufficient |
| 15 | SOP | Standard Operating Procedure |
| 16 | TRF | Test Requisition Form |
| 17 | TDPL | TRUSTlab Diagnostics Private Limited |

- Objective

- The purpose of this SOP is to define a procedure for the qualitative detection of bile pigments (bilirubin and biliverdin) in urine, by Fouchet’s test. This SOP also provides information about the test principles, clinical significance and other details.

- Scope

- This procedure applies to all staff of the Clinical Pathology Department tasked with the duty of performing these tests.

- Responsibilities

It is the responsibility of the technicians of the Clinical Pathology Department to comply with this SOP.

HOD shall ensure implementation.

The Quality Manager is responsible for monitoring, reviewing and ensuring compliance with this procedure.

All other TRUSTlab staff involved in this process (including contract personnel) are responsible for complying with this procedure.

- Definition

- Urine bile pigments, primarily bilirubin, can indicate liver dysfunction or bile duct obstruction. The presence of bile pigments in urine is generally not normal and may suggest various medical conditions.

- Test Principle

- Barium chloride reacts with sulfate radicals present in urine to form a precipitate of barium sulfate. If bile pigments are present in the urine sample, they adhere to the precipitate and is detected by the oxidation of bilirubin (yellow – colored) to biliverdin (green - colored) on treatment with ferric Chloride (Fouchet’s reagent) in the presence of trichloroacetic acid.

- Specific Performance Characteristics

- N/A.

- Type of Sample

Urine is the primary specimen for the test.

Collect a minimum 15 ml of a random sample of urine.

Process the samples within 1 hour of collection. If delay is anticipated instruct patients to store the sample at 2-8°C in the refrigerator for a maximum of up to 4 hours until transport to the laboratory.

Use fresh and un-centrifuged samples for the test.

Do not accept if the sample volume is inadequate or if samples are received without an ID number.

- Type of Container & Additives

- Sterile screw-capped container.

- Patient Preparation

Instruct patients to collect urine samples in a clean dry container (preferably one provided by TRUSTlab).

Instruct patients to void urine directly into the container and while collecting allow the first portion of urine to escape.

- Equipment & Reagents

Equipment

- Test tubes

Reagents

- Fouchet’s reagent prepared by

      - Trichloro acetic acid (25 g)

      - 10% Ferric chloride (10 ml)

      - Distilled water (50 ml). Make up to 100 ml in a volumetric flask.

      - 10% Barium Chloride solution

- Safety

Follow universal precautions like wearing suitable gloves and eye/face protection while handling all reagents, samples and operating the instrument.

Universal precautions imply that all body fluids are treated as if known to be infectious for Human Immunodeficiency Virus (HIV), Hepatitis B Virus (HBV) or other Blood borne Pathogens.

Dispose of all waste in accordance with applicable national and/or local regulations.

Some reagents contain sodium azide, which may react with copper or lead plumbing to form explosive metal azides. Use caution in disposing of these reagents. If disposing to drain, flush with large volumes of water to prevent azide buildup.

Waste material containing patient samples or biological products should be considered biohazardous when disposing or treating.

Clean up all spills immediately and thoroughly. Disinfect the area for any spills involving biohazardous materials. Dispose of all contaminated materials appropriately.

- Calibration Procedure

- N/A.

- Test Procedure

      - Check the pH of urine with a litmus paper strip. If the urine is alkaline, add a few drops of 33% acetic acid to make it acidic pH should be less than 7.0.

      - Take 2.5 ml of clear uncentrifuged urine sample in a test tube.

      - Add 2.5 ml of 10% barium chloride solution. Mix well. The solution turns milky with white or yellow precipitate.

      - Filter the precipitate through a Whatman No 1 filter paper held in a small funnel.

      - Carefully unfold the filter paper and dry it by blotting with another filter paper.

      - Place the precipitate on a tile or wax paper.

      - Add 1-2 drops of Fouchet’s reagent on to the precipitate in the centre of the wax paper or tile.

- Quality Control Procedure

Internal Quality Control

- As defined in the procedure Urine IQC (TDPL/CP/SOP-002).

External Quality Control

- EQAS: Metropolis EQAS

- Interferences & Cross Reactions

- N/A.

- Result Calculation/Interpretation

- A green or blue colour indicates the presence of bilirubin.

- Biological Reference Interval

- Negative.

- Reportable Interval

- N/A.

- Procedure when Results are not in Range

- N/A.

- Critical Alert Values

- N/A.

- Laboratory Clinical Interpretation

The presence of bile pigments in urine is diagnostic of hepatic jaundice or obstructive jaundice. Presence of bilirubin in urine is detected before the clinical symptoms of jaundice (change of skin color) are recognized).

- Potential Source of Variability

If the urine is not tested fresh, the bilirubin present in the urine is oxidized to biliverdin which will not respond to the test. Hence the urine has to be tested fresh for bilirubin.

Also, when urine containing bilirubin is exposed to light it is oxidized to biliverdin and will be negative when tested for bilirubin.

Drugs like asprin and pigmented metabolites of certain drugs may give an atypical color reaction and mask the normal positive color due to the presence of bilirubin.

- References

      - Handbook of Medical Laboratory Technology 2nd Editon, Edited by Robert H. Caraman, M.D. Laboratory Training Committee, Christian Medical Association of India, Page: 240

      - Procedure for Managing Internal QC Program (TDPL/QA/MAN-001: MSP-013)

      - NABL 112