SOP – 009

PROCEDURE FOR

URINE REDUCING SUGAR

- TABLE OF CONTENTS

TABLE OF CONTENTS	2

APPROVAL AUTHORITY	3

RECORD OF AMENDMENTS	4

DISTRIBUTION LIST	5

LIST OF ABBREVIATIONS	6

1.	Objective	7

2.	Scope	7

3.	Responsibilities	7

4.	Definition	7

5.	Test Principle	7

6.	Specific Performance Characteristics	7

7.	Type of Sample	8

8.	Type of Container & Additives	8

9.	Patient Preparation	8

10.	Equipment & Reagents	8

11.	Safety	8

12.	Calibration Procedure	9

13.	Test Procedure	9

14.	Quality Control Procedure	9

15.	Interferences & Cross Reactions	9

16.	Result Calculation/Interpretation	9

17.	Biological Reference Interval	10

18.	Reportable Interval	10

19.	Procedure when Results are not in Range	10

20.	Critical Alert Values	10

21.	Laboratory Clinical Interpretation	10

22.	Potential Source of Variability	10

23.	References	10

| Prepared by: | Head of Department | Name | Date: |
| --- | --- | --- | --- |
|  |  | Signature |  |
| Reviewed & Approved by: | Lab Head | Name | Date: |
|  |  | Signature |  |
| Issued by: | Quality Manager | Name | Date: |
|  |  | Signature |  |

- APPROVAL AUTHORITY

- RECORD OF AMENDMENTS

| S. No. | Ver. No. | Ver. Date | Rev. No. | Rev. Date | Description of Changes |
| --- | --- | --- | --- | --- | --- |

| Document Copy No. | Name / Designation of the holder of the document copy | Type of Copy | Version No. | Revision No. |
| --- | --- | --- | --- | --- |
| 01/10 | Managing Director | Controlled Copy | 01 | 00 |
| 02/10 |  |  |  |  |
| 03/10 |  |  |  |  |
| 04/10 |  |  |  |  |
| 05/10 |  |  |  |  |
| 06/10 |  |  |  |  |
| 07/10 |  |  |  |  |
| 08/10 |  |  |  |  |
| 09/10 |  |  |  |  |
| 10/10 |  |  |  |  |

- DISTRIBUTION LIST

- LIST OF ABBREVIATIONS

| S. No. | Abbreviations | Expansion |
| --- | --- | --- |
| 1 | °C | Degree Centigrade |
| 2 | CP | Clinical Pathology |
| 3 | CV | Coefficient of Variation |
| 4 | EQAS | External Quality Assurance Scheme |
| 5 | HBV | Hepatitis B Virus |
| 6 | HIV | Human Immunodeficiency Virus |
| 7 | HOD | Head of Department |
| 8 | IQC | Internal Quality Control |
| 9 | MSP | Management System Procedure |
| 10 | N/A | Not Applicable/Available |
| 11 | pH | Potential of Hydrogen |
| 12 | QC | Quality Control |
| 13 | QM | Quality Manager |
| 14 | QNS | Quantity Not Sufficient |
| 15 | SOP | Standard Operating Procedure |
| 16 | TRF | Test Requisition Form |
| 17 | TDPL | TRUSTlab Diagnostics Private Limited |

- Objective

- The purpose of this SOP is to describe the qualitative identification of reducing sugar in urine by - Benedict’s test. This SOP also provides information about the test principles, clinical significance and other details.

- Scope

- This procedure applies to all staff of the Clinical Pathology Department tasked with the duty of performing these tests.

- Responsibilities

It is the responsibility of the technicians of the Clinical Pathology Department to comply with this SOP.

HOD shall ensure implementation.

The Quality Manager is responsible for monitoring, reviewing and ensuring compliance with this procedure.

All other TRUSTlab staff involved in this process (including contract personnel) are responsible for complying with this procedure.

- Definition

- Urine reducing sugar, also known as glycosuria, refers to the presence of sugars, such as glucose, fructose, or galactose, in the urine. While some sugars can be normally excreted in small amounts, excessive levels may indicate underlying health conditions.

- Test Principle

- Reducing sugars containing free aldehyde group/free keto group form 1,2 enediols in weak alkaline medium that reduce cupric ions to cuprous ions and form a precipitate of cuprous oxide, the color of which varies depending upon the concentration of the reducing sugar in the sample of urine.

- Specific Performance Characteristics

- N/A.

- Type of Sample

Urine is the primary specimen for the test.

Collect a minimum 15 ml of a random sample of urine.

No preservative should be added to the urine.

Process the samples within 1 hour of collection. If delay is anticipated instruct patients to store the sample at 2-8°C in the refrigerator for a maximum of up to 4 hours until transport to the laboratory.

Use fresh and un-centrifuged samples for the test.

Do not accept if the sample volume is inadequate or if samples are received without an ID number.

- Type of Container & Additives

- Sterile screw-capped container.

- Patient Preparation

Instruct patients to collect urine samples in a clean dry container (preferably one provided by TRUSTlab).

Instruct patients to void urine directly into the container and while collecting allow the first portion of urine to escape.

- Equipment & Reagents

Equipment

      - Bunsen Burner

      - Test tubes

      - Pasteur pipette

Reagents

- Benedict’s qualitative reagent

- Safety

Follow universal precautions like wearing suitable gloves and eye/face protection while handling all reagents, samples and operating the instrument.

Universal precautions imply that all body fluids are treated as if known to be infectious for Human Immunodeficiency Virus (HIV), Hepatitis B Virus (HBV) or other Blood borne Pathogens.

Dispose of all waste in accordance with applicable national and/or local regulations.

Some reagents contain sodium azide, which may react with copper or lead plumbing to form explosive metal azides. Use caution in disposing of these reagents. If disposing to drain, flush with large volumes of water to prevent azide buildup.

Waste material containing patient samples or biological products should be considered biohazardous when disposing or treating.

Clean up all spills immediately and thoroughly. Disinfect the area for any spills involving biohazardous materials. Dispose of all contaminated materials appropriately.

- Calibration Procedure

- N/A.

- Test Procedure

      - Take 5 ml of Benedict’s qualitative solution in a test tube and boil it.

      - To it, add 8 drops (0.5 ml) of urine sample and then boil directly over the flame of a burner for 2- 3 minutes.

      - Allow the test tube to cool in air.

      - Observe the color of the precipitate formed.

- Quality Control Procedure

Internal Quality Control

- As defined in the procedure Urine IQC (TDPL/CP/SOP-002).

External Quality Control

- EQAS: Metropolis EQAS

- Interferences & Cross Reactions

- N/A.

- Result Calculation/Interpretation

Negative: no change in colour.

Trace: pale green & slightly cloudy.

1+: definite cloudy green

2+: yellow to orange

3+: orange to red precipitate

4+: Brick red precipitate

- Biological Reference Interval

- Negative.

- Reportable Interval

- N/A.

- Procedure when Results are not in Range

- N/A.

- Critical Alert Values

- N/A.

- Laboratory Clinical Interpretation

- This test is useful in the diagnosis of Diabetes Mellitus, renal glycosuria and glycosuria due to other causes and in monitoring the response to treatment with Insulin or oral hypoglycemic agents.

- Potential Source of Variability

This test is not specific for glucose as all reducing sugars (fructose, lactose, galactose) answer this test.

Non-carbohydrate reducing substances like Ascorbic acid, Aspirin, Uric acid, Homogentisic acid and glucuronic acid also answers this test.

The sugar present in the urine of normal healthy individuals cannot be detected by this method.

- References.

      - Handbook of Medical Laboratory Technology 2nd Editon, Edited by Robert H. Caraman, M.D. Laboratory Training Committee, Christian Medical Association of India. Page - 233

      - Procedure for Managing Internal QC Program (TDPL/QA/MAN-001: MSP-013)

      - NABL 112