SOP – 010

PROCEDURE FOR

URINE SPECIFIC GRAVITY

- TABLE OF CONTENTS

TABLE OF CONTENTS	2

APPROVAL AUTHORITY	3

RECORD OF AMENDMENTS	4

DISTRIBUTION LIST	5

LIST OF ABBREVIATIONS	6

1.	Objective	7

2.	Scope	7

3.	Responsibilities	7

4.	Definition	7

5.	Test Principle	7

6.	Specific Performance Characteristics	7

7.	Type of Sample	8

8.	Type of Container & Additives	8

9.	Patient Preparation	8

10.	Equipment & Reagents	8

11.	Safety	9

12.	Calibration Procedure	9

13.	Test Procedure	9

14.	Quality Control Procedure	10

15.	Interferences & Cross Reactions	10

16.	Result Calculation/Interpretation	10

17.	Biological Reference Interval	10

18.	Reportable Interval	10

19.	Procedure when Results are not in Range	10

20.	Critical Alert Values	10

21.	Laboratory Clinical Interpretation	10

22.	Potential Source of Variability	11

23.	References	11

- APPROVAL AUTHORITY

| Prepared by: | Head of Department | Dr. Abdul Aleem Name | Date: 14/11/2024 |
| --- | --- | --- | --- |
|  |  | Signature |  |
| Reviewed & Approved by: | Laboratory Head | Dr. Abdul Aleem Name | Date: 14/11/2024 |
|  |  | Signature |  |
| Issued by: | Quality Manager | Dr. Sai Varun Name | Date: 14/11/2024 |
|  |  | Signature |  |

- RECORD OF AMENDMENTS

| S. No. | Ver. No. | Ver. Date | Rev. No. | Rev. Date | Description of Changes |
| --- | --- | --- | --- | --- | --- |

| Document Copy No. | Name / Designation of the holder of the document copy | Type of Copy | Version No. | Revision No. |
| --- | --- | --- | --- | --- |
| 01/10 | Managing Director | Controlled Copy | 01 | 00 |
| 02/10 |  |  |  |  |
| 03/10 |  |  |  |  |
| 04/10 |  |  |  |  |
| 05/10 |  |  |  |  |
| 06/10 |  |  |  |  |
| 07/10 |  |  |  |  |
| 08/10 |  |  |  |  |
| 09/10 |  |  |  |  |
| 10/10 |  |  |  |  |

- DISTRIBUTION LIST

- LIST OF ABBREVIATIONS

| S. No. | Abbreviations | Expansion |
| --- | --- | --- |
| 1 | °C | Degree Centigrade |
| 2 | CP | Clinical Pathology |
| 3 | CV | Coefficient of Variation |
| 4 | EQAS | External Quality Assurance Scheme |
| 5 | HBV | Hepatitis B Virus |
| 6 | HIV | Human Immunodeficiency Virus |
| 7 | HOD | Head of Department |
| 8 | IQC | Internal Quality Control |
| 9 | MSP | Management System Procedure |
| 10 | N/A | Not Applicable/Available |
| 11 | pH | Potential of Hydrogen |
| 12 | QC | Quality Control |
| 13 | QM | Quality Manager |
| 14 | QNS | Quantity Not Sufficient |
| 15 | SOP | Standard Operating Procedure |
| 16 | TRF | Test Requisition Form |
| 17 | TDPL | TRUSTlab Diagnostics Private Limited |

- Objective

- The purpose of this SOP is to describe the procedure to determine the specific gravity by urinometer. This SOP also provides information about the test principles, clinical significance and other details.

- Scope

- This procedure applies to all staff of the Clinical Pathology Department tasked with the duty of performing these tests.

- Responsibilities

It is the responsibility of the technicians of the Clinical Pathology Department to comply with this SOP.

HOD shall ensure implementation.

The Quality Manager is responsible for monitoring, reviewing and ensuring compliance with this procedure.

All other TRUSTlab staff involved in this process (including contract personnel) are responsible for complying with this procedure.

- Definition

- Urine specific gravity is a measure of the concentration of solutes in urine, indicating how well your kidneys are conserving water. It's a routine part of urinalysis and helps assess kidney function and hydration status.

- Test Principle

- The measurement of Specific Gravity of urine by urinometer is based on the principle of buoyancy and is dependent on the amount of solutes dissolved in the urine sample. As the concentration of solutes increases, the upthrust of the solution increases and the Specific Gravity correspondingly increases and vice versa. The increased upthrust is reflected by the scale of the urinometer as the latter is pushed up.

- Specific Performance Characteristics

- The Specific Gravity of normal urine from healthy individuals itself is subjected to wide variation depending upon the water intake. Excessive water intake      decreases the Specific Gravity of urine to well below the normal and decreased water intake and excessive perspiration will increase the Specific Gravity. of urine above the normal range of 1025. Correction for temperature above 15 °C / Correction abnormal solute concentration like glucose and protein and correction for dilution have to be applied for measuring the Actual Specific Gravity of urine.

- Type of Sample

Urine is the primary specimen for the test.

Collect a minimum 15 ml of a random sample of urine.

No preservative should be added to the urine.

Process the samples within 1 hour of collection. If delay is anticipated instruct patients to store the sample at 2-8°C in the refrigerator for a maximum of up to 4 hours until transport to the laboratory.

Use fresh and un-centrifuged samples for the test.

Do not accept if samples are received without an ID number.

- Type of Container & Additives

- Sterile screw-capped container.

- Patient Preparation

Instruct patients to collect urine samples in a clean dry container (preferably one provided by TRUSTlab).

Instruct patients to void urine directly into the container and while collecting allow the first portion of urine to escape.

- Equipment & Reagents

Equipment

      - Urinometer

      - 25 ml glass cylinder

Reagents

- N/A.

- Safety

Follow universal precautions like wearing suitable gloves and eye/face protection while handling all reagents, samples and operating the instrument.

Universal precautions imply that all body fluids are treated as if known to be infectious for Human Immunodeficiency Virus (HIV), Hepatitis B Virus (HBV) or other Blood borne Pathogens.

Dispose of all waste in accordance with applicable national and/or local regulations.

Some reagents contain sodium azide, which may react with copper or lead plumbing to form explosive metal azides. Use caution in disposing of these reagents. If disposing to drain, flush with large volumes of water to prevent azide buildup.

Waste material containing patient samples or biological products should be considered biohazardous when disposing or treating.

Clean up all spills immediately and thoroughly. Disinfect the area for any spills involving biohazardous materials. Dispose of all contaminated materials appropriately.

- Calibration Procedure

- N/A.

- Test Procedure

      - Keep the glass cylinder on a flat surface.

      - Mix the urine well and pour it into the cylinder. Leave about 1 inch at the top so that the urine does not overflow when the urinometer is let to float on the sample.

      - Remove any air bubbles from the top with a filter paper.

      - Carefully float the urinometer by holding the stem and gently insert it into the urine.

      - Gently swirl the Urinometer as it is inserted.

      - Ensure the urinometer floats and does not touch the sides of the cylinder while taking the reading.

      - Read the Specific Gravity as soon as the urinometer comes to rest.

      - Take the readings of the meniscus from the graduations given on the stem of the urinometer. View at eye level to minimize parallax error.

      - QNS Handling: If the urine specimen is not sufficient, dilute the urine with equal volume of distilled water and multiply the last two digits of the recorded Specific Gravity by the dilution factor (x 2).

      - Temperature correction for every 3°C above or below 25°C add or subtract 0.001 from the recorded Specific Gravity.

- Quality Control Procedure

Internal Quality Control

- As defined in the procedure Urine IQC (TDPL/CP/SOP-002).

External Quality Control

- EQAS: Metropolis EQAS

- Interferences & Cross Reactions

- N/A.

- Result Calculation/Interpretation

- N/A

- Biological Reference Interval

- Specific Gravity of normal urine 1005 -1025.

- Reportable Interval

- N/A.

- Procedure when Results are not in Range

- N/A.

- Critical Alert Values

- N/A.

- Laboratory Clinical Interpretation

- Decreased Specific Gravity of urine is observed in Diabetes Insipidus and over hydration. Decreased Specific Gravity of urine is observed in the presence of dehydration, proteinuria and glycosuria due to any cause as in Diabetes Mellitus and renal failure. A low and fixed Specific Gravity is observed in tubular disorders of the kidney where the concentration ability of the kidney is impaired.

- Potential Source of Variability

Carry out examination of the urine sample when fresh preferably within 1 hour of collection as delay will cause alterations in the chemical composition of urine (leucocytes to be destroyed, casts to decompose and urea will be broken-down due to bacterial action and make the urine alkaline).

Contamination of glassware used in testing with Sodium Hypochlorite and Bleaching powder and detergents like sodium phosphate will oxidize and change the color of chromogen in test strip. Hence ensure the glassware is free of these chemicals.

The Specific Gravity of normal urine from healthy individuals itself is subjected to wide variation depending upon the water intake. Excessive water intake decreases the Specific Gravity of urine to well below the normal and decreased water intake and excessive perspiration will increase the Specific Gravity. of urine above the normal range of 1025.

- References.

      - Textbook of Medical Laboratory Technology, 2nd Edition, Page: 886.

      - Textbook of Medical Laboratory Technology, Volume II, Page: 810-813

      - Procedure for Managing Internal QC Program (TDPL/QA/MAN-001: MSP-013)

      - NABL 112