SOP – 002

PROCEDURE FOR

PREPARATION OF POOLED CELLS

- TABLE OF CONTENTS

TABLE OF CONTENTS	2

APPROVAL AUTHORITY	3

RECORD OF AMENDMENTS	4

DISTRIBUTION LIST	5

LIST OF ABBREVIATIONS	6

1.	Objective	7

2.	Scope	7

3.	Responsibilities	7

4.	Definition	7

5.	Test Principle	7

6.	Specific Performance Characteristics	7

7.	Type of Sample	7

8.	Type of Container & Additives	8

9.	Patient Preparation	8

10.	Equipment & Reagents	8

11.	Safety	8

12.	Calibration Procedure	9

13.	Procedure	9

14.	Quality Control Procedure	9

15.	Interferences & Cross Reactions	9

16.	Result Calculation	9

17.	Biological Reference Interval	9

18.	Reportable Interval	9

19.	Procedure when Results are not in Range	9

20.	Critical Alert Values	10

21.	Laboratory Clinical Interpretation	10

22.	Potential Source of Variability	10

23.	References	10

- APPROVAL AUTHORITY

| Prepared by: | Head of Department | Dr. Abdul Aleem Name | Date: 14/11/2024 |
| --- | --- | --- | --- |
|  |  | Signature |  |
| Reviewed & Approved by: | Laboratory Head | Dr. Abdul Aleem Name | Date: 14/11/2024 |
|  |  | Signature |  |
| Issued by: | Quality Manager | Dr. Sai Varun Name | Date: 14/11/2024 |
|  |  | Signature |  |

- RECORD OF AMENDMENTS

| S. No. | Ver. No. | Ver. Date | Rev. No. | Rev. Date | Description of Changes |
| --- | --- | --- | --- | --- | --- |

| Document Copy No. | Name / Designation of the holder of the document copy | Type of Copy | Version No. | Revision No. |
| --- | --- | --- | --- | --- |
| 01/10 | Managing Director | Controlled Copy | 01 | 00 |
| 02/10 |  |  |  |  |
| 03/10 |  |  |  |  |
| 04/10 |  |  |  |  |
| 05/10 |  |  |  |  |
| 06/10 |  |  |  |  |
| 07/10 |  |  |  |  |
| 08/10 |  |  |  |  |
| 09/10 |  |  |  |  |
| 10/10 |  |  |  |  |

- DISTRIBUTION LIST

- LIST OF ABBREVIATIONS

| S. No. | Abbreviations | Expansion |
| --- | --- | --- |
| 1 | AMR | Analytical Measurement Range |
| 2 | °C | Degree Centigrade |
| 3 | CV | Coefficient of Variation |
| 4 | CRR | Clinical Reportable Range |
| 5 | EDTA | Ethylenediamine Tetra-acetic Acid |
| 6 | EQAS | External Quality Assurance Scheme |
| 7 | HOD | Head of Department |
| 8 | HBV | Hepatitis B Virus |
| 9 | HIV | Human Immunodeficiency Virus |
| 10 | HM | Haematology |
| 11 | IQC | Internal Quality Control |
| 12 | MSP | Management System Procedure |
| 13 | N/A | Not Applicable/Available |
| 14 | NS | Normal Saline |
| 15 | pH | Potential of Hydrogen |
| 16 | QC | Quality Control |
| 17 | QM | Quality Manager |
| 18 | QNS | Quantity Not Sufficient |
| 19 | rpm | Revolutions per minute |
| 20 | SD | Standard Deviation |
| 21 | SOP | Standard Operating Procedure |
| 22 | TRF | Test Requisition Form |
| 23 | TDPL | TRUSTlab Diagnostics Private Limited |

- Objective

- The purpose of this SOP is to describe the procedure for the preparation of pooled cells. This SOP also provides information about the test principle, clinical significance and other details.

- Scope

- This procedure applies to all staff of the Haematology Department tasked with the duty of performing these tests.

- Responsibilities

It is the responsibility of the technicians of the Haematology Department to comply with this SOP.

HOD shall ensure implementation.

The Quality Manager is responsible for monitoring, reviewing and ensuring compliance with this procedure.

All other TRUSTlab staff involved in this process (including contract personnel) are responsible for complying with this procedure.

- Definition

- Pooled cells refer to combining multiple individual cells, often for testing or analysis. In antibody screening, pooled cells, also known as pooled screening cells, are used to detect clinically relevant antibodies. Pooled cells are made by mixing equal volumes of different cells, and they are a simplified way of checking for multiple antibodies at once.

- Test Principle

- Pooling of 3 same blood group cells and saline washed by Centrifugation method.

- Specific Performance Characteristics

- N/A.

- Type of Sample

- Blood sample collected following universally accepted procedures of venipuncture.

- Type of Container & Additives

- K2 EDTA anticoagulant vacutainer.

- Patient Preparation

- N/A.

- Equipment & Reagents

Equipment

      - Test tubes 12x5mm

      - Pipettes

      - Centrifuge

      - Droppers

Reagents & others

      - Normal Saline

      - Already known groups of A1, B and O - minimum 3 Samples each

- Safety

Follow universal precautions, like wearing suitable gloves and eye/face protection, while handling all reagents, samples and operating the instrument.

Universal precautions imply that all body fluids are treated as if known to be infectious for Human Immunodeficiency Virus (HIV), Hepatitis B Virus (HBV) or other Blood borne Pathogens.

Dispose of all waste in accordance with applicable national and/or local regulations.

Some reagents contain sodium azide, which may react with copper or lead plumbing to form explosive metal azides. Use caution in disposing of these reagents. If disposing to drain, flush with large volumes of water to prevent azide buildup.

Waste material containing patient samples or biological products should be considered biohazardous when disposing or treating.

Clean up all spills immediately and thoroughly. Disinfect the area for any spills involving biohazardous materials. Dispose of all contaminated materials appropriately.

- Calibration Procedure

- N/A.

- Procedure

      - Take known groups of blood samples (A1, B, & O) minimum 3 samples each approximately 200 µl.

      - Centrifuge at 2500 rpm for 5 minutes.

      - Decant the plasma.

      - Add 3 ml of normal saline and mix well.

      - Centrifuge at 3000 rpm 3 minutes.

      - Decant the supernatant.

      - To the remaining cell button repeat steps 4, 5 and 6 thrice.

      - After the final wash, to prepare 2 - 5% suspension of red cells, add 2-2.5 ml of normal saline until the solution attains a tomato red colour.

- Quality Control Procedure

- N/A.

- Interferences & Cross Reactions

- N/A.

- Result Calculation

- N/A.

- Biological Reference Interval

- N/A.

- Reportable Interval

- N/A.

- Procedure when Results are not in Range

- N/A.

- Critical Alert Values

- N/A.

- Laboratory Clinical Interpretation

- N/A.

- Potential Source of Variability

- N/A.

- References

      - Practical Hematology, Dacie and Lewis, Tenth edition.