SOP – 009

PROCEDURE FOR

ABSOLUTE EOSINOPHIL COUNT

- TABLE OF CONTENTS

TABLE OF CONTENTS	2

APPROVAL AUTHORITY	3

RECORD OF AMENDMENTS	4

DISTRIBUTION LIST	5

LIST OF ABBREVIATIONS	6

1.	Objective	7

2.	Scope	7

3.	Responsibilities	7

4.	Definition	7

5.	Test Principle	7

6.	Specific Performance Characteristics	7

7.	Type of Sample	8

8.	Type of Container & Additives	8

9.	Patient Preparation	8

10.	Equipment & Reagents	8

11.	Safety	9

12.	Calibration Procedure	9

13.	Test Procedure	9

14.	Quality Control Procedure	10

15.	Interferences & Cross Reactions	10

16.	Result Calculation	10

17.	Biological Reference Interval	10

18.	Reportable Interval	11

19.	Procedure when Results are not in Range	11

20.	Critical Alert Values	11

21.	Laboratory Clinical Interpretation	11

22.	Potential Source of Variability	11

23.	References	12

- APPROVAL AUTHORITY

| Prepared by: | Head of Department | Dr. Abdul Aleem Name | Date: 14/11/2024 |
| --- | --- | --- | --- |
|  |  | Signature |  |
| Reviewed & Approved by: | Laboratory Head | Dr. Abdul Aleem Name | Date: 14/11/2024 |
|  |  | Signature |  |
| Issued by: | Quality Manager | Dr. Sai Varun Name | Date: 14/11/2024 |
|  |  | Signature |  |

- RECORD OF AMENDMENTS

| S. No. | Ver. No. | Ver. Date | Rev. No. | Rev. Date | Description of Changes |
| --- | --- | --- | --- | --- | --- |

| Document Copy No. | Name / Designation of the holder of the document copy | Type of Copy | Version No. | Revision No. |
| --- | --- | --- | --- | --- |
| 01/10 | Managing Director | Controlled Copy | 01 | 00 |
| 02/10 |  |  |  |  |
| 03/10 |  |  |  |  |
| 04/10 |  |  |  |  |
| 05/10 |  |  |  |  |
| 06/10 |  |  |  |  |
| 07/10 |  |  |  |  |
| 08/10 |  |  |  |  |
| 09/10 |  |  |  |  |
| 10/10 |  |  |  |  |

- DISTRIBUTION LIST

- LIST OF ABBREVIATIONS

| S. No. | Abbreviations | Expansion |
| --- | --- | --- |
| 1 | AEC | Absolute Eosinophil Count |
| 2 | °C | Degree Centigrade |
| 3 | EDTA | Ethylenediamine Tetra-acetic Acid |
| 4 | EQAS | External Quality Assurance Scheme |
| 5 | HOD | Head of Department |
| 6 | HM | Haematology |
| 7 | INR | International Normalized Ratio |
| 8 | IQC | Internal Quality Control |
| 9 | ISI | International Sensitivity Index of the reagent |
| 10 | MSP | Management System Procedure |
| 11 | N/A | Not Applicable/Available |
| 12 | QC | Quality Control |
| 13 | QM | Quality Manager |
| 14 | QNS | Quantity Not Sufficient |
| 15 | rpm | Revolutions per minute |
| 16 | SOP | Standard Operating Procedure |
| 17 | TRF | Test Requisition Form |
| 18 | TDPL | TRUSTlab Diagnostics Private Limited |

- Objective

- The purpose of this SOP is to describe the procedure for performing the Absolute Eosinophil Count test. This SOP also provides information about the test principle, clinical significance and other details.

- Scope

- This procedure applies to all staff of the Haematology Department tasked with the duty of performing these tests.

- Responsibilities

It is the responsibility of the technicians of the Haematology Department to comply with this SOP.

HOD shall ensure implementation.

The Quality Manager is responsible for monitoring, reviewing and ensuring compliance with this procedure.

All other TRUSTlab staff involved in this process (including contract personnel) are responsible for complying with this procedure.

- Definition

- The Absolute Eosinophil Count (AEC) test is a blood test that measures the number of eosinophils, a type of white blood cell. Eosinophils play a crucial role in the body's immune response, particularly in fighting off parasites, allergic reactions, and inflammatory conditions. The AEC test helps doctors assess an individual’s immune health and can be used to diagnose various conditions, including allergies, asthma, parasitic infections, and certain autoimmune diseases.

- Test Principle

The diluting fluid used stains the eosinophils distinctly and lyse the red cells. A preliminary background count is performed with the fluid alone loaded in the counting chamber and recorded.

Method: Staining and microscopic examination.

- Specific Performance Characteristics

- Clinical Reportable Range (CRR): 10-20,000 /µl

- Type of Sample

Whole Blood

Sample Retention: Sample is retained for 24 hours at 2 – 8℃.

Sample Rejection Criteria:

      - Quantity not sufficient (QNS)

      - Sample received in wrong vacutainer

      - No TRF; Grossly incomplete TRF; Errors in labeling

      - Grossly haemolysed specimens

      - Highly lipemic specimens

      - Highly icteric specimens

      - Sample received beyond stability period

      - Incomplete TRF details or labelling errors are clarified with Customer/Doctor/ Patient before sample is accepted for processing. If critical information is still not obtained, the sample is rejected.

- Type of Container & Additives

- K2 EDTA Vacutainer

- Patient Preparation

No special preparation.

- Equipment & Reagents

Equipment

      - Microscope.

      - Micropipette (20-200 microlitre Variable)

      - Improved Neubauer chamber

      - Test tube (10 x 75mm).

      - Coverslips

Reagents

- Commercially available Eosinophil diluting fluid.

- Safety

Handle all samples as potentially infectious.

Handle all reagents with care and avoid contact with eye, mouth and skin.

Do not perform mouth pipetting.

Discard used reagents and samples as per disposal procedure.

- Calibration Procedure

- N/A.

- Test Procedure

Take 1.8 ml of diluting fluid in a test tube and add 0.2 ml of anticoagulated blood. Mix it and keep at room Temperature (20-25 0 C) for 10 – 15 minutes before charging the counting chamber.

The improved Neubauer counting chamber is used.

Description of the improved Neubauer Counting Chamber

      - It usually has two ruled stages separated by a small gutter.

      - The surface of these two ridges is 1/10 mm above the surface of the stage so that when a cover slip is supported on them, there is a space of 1/10 mm between the bottom of the cover slip and the top of the   20-1 chamber stage.

      - There is a large, ruled area of approximately 9 sq.mm (3x3 mm) on each chamber stage. It is divided into 9 squares.

      - The corner squares are divided into 16 smaller squares each and are used for counting white blood cells.

      - The central square is divided into 25 squares, each of which in turn is divided into 16 smaller squares. Each smaller square measure 1/400 sq.mm in area.

      - Five of the medium squares are used in counting red blood cells.

Charging the Counting Chamber

      - The counting chamber should be perfectly clean and dry. The cover slip, also perfectly clean and dry, should be applied carefully (broken cover slips should not be used).

      - Mix the suspension in the test tube, well.

      - Then, with the help of a micro pipette and after discarding the first few drops, the chamber is loaded placing the tip on the platform adjacent to the edge of the coverslip. Hold the pipette slightly inclined and gently release the pressure and allow a small volume to flow down. This will be attracted under the cover slip by capillary action.

      - The filling should be done in one application of the tip of the pipette.

      - There should be no air bubbles under the cover slip.

      - The fluid should not overflow into the gutters.

      - Allow the cells to settle for 2 – 3 minutes so that they may all be seen in the same plane of focus.

      - The cells in all the 9 squares of the chamber are counted. It is convenient to use the low power objective and 10x eyepiece to focus on the ruling. Counting should be done under 40X objective.

- Quality Control Procedure

Internal Quality Control

    - Procedure:

- Before examination of the sample, the diluting fluid is run as a sample and the results are recorded. The acceptable background check is 0 cells.

External Quality Control

- EQAS participation –- AIIMS EQAS

- Interferences & Cross Reactions

- N/A.

- Result Calculation

- Use the following formula:

- Total number of cells per mm3=Number of cells counted × Dilution factorVolume of Fluid

- = Number of cells counted X 11.1

- Dilution = 10

- Volume of fluid = Area Counted X Depth

- = 9 sq.mm x 0.1 = 0.9

- Biological Reference Interval

Normal counts: 20 – 500 cells/mm3

Eosinophilia: > 500 cells/mm3

- Reportable Interval

- N/A.

- Procedure when Results are not in Range

- N/A.

- Critical Alert Values

- N/A.

- Laboratory Clinical Interpretation

      - A high number of eosinophils (eosinophilia) is often linked to allergic diseases and infections from parasites such as worms. A high eosinophil count may be due to:

          - Asthma

          - Autoimmune diseases

          - Eczema

          - Hay fever

          - Leukemia

      - A lower-than-normal eosinophil count may be due to:

          - Alcohol intoxication

          - Over production of certain steroids in the body (such as cortisol).

- Potential Source of Variability

Eosinophils disintegrate in the diluting fluid, hence counting should be done within 30 mins of the diluting sample with the diluting fluid.

Errors due to the nature of the sample: partial coagulation of the venous blood introduces errors by the changes in the cell distribution.

Operator’s errors: Errors caused by faulty technique may occur when blood and the diluting fluid are drawn into the pipette, when the chamber is loaded, and when the cells are counted.

Faulty application of the cover glass and overflowing of the suspension into the moat are examples of faulty technique.

Errors due to Equipment: Inaccuracies in the graduations of the pipettes and of the ruled areas and the depth of the counting chambers are frequent sources of error.

Inherent of field error: Even in a perfectly mixed sample, variation occurs in the number of suspended cells that are distributed in the given volume.

- References

      - Practical Hematology, J. Dacie and M. Lewis, Tenth edition, Page No 26-27, 33, 62 ,69, 104, 107, 614.

      - Procedure for Managing Internal QC Program (TDPL/QA/MAN-001: MSP-013).