SOP – 012

PROCEDURE FOR

MALARIA SMEAR EXAMINATION

- TABLE OF CONTENTS

TABLE OF CONTENTS	2

APPROVAL AUTHORITY	3

RECORD OF AMENDMENTS	4

DISTRIBUTION LIST	5

LIST OF ABBREVIATIONS	6

1.	Objective	7

2.	Scope	7

3.	Responsibilities	7

4.	Definition	7

5.	Test Principle	7

6.	Specific Performance Characteristics	7

7.	Type of Sample	8

8.	Type of Container & Additives	8

9.	Patient Preparation	8

10.	Equipment & Reagents	8

11.	Safety	9

12.	Calibration Procedure	9

13.	Test Procedure	9

14.	Quality Control Procedure	12

15.	Interferences & Cross Reactions	13

16.	Result Calculation	13

17.	Biological Reference Interval	13

18.	Reportable Interval	14

19.	Procedure when Results are not in Range	14

20.	Critical Alert Values	14

21.	Laboratory Clinical Interpretation	14

22.	Potential Source of Variability	14

23.	References	15

- APPROVAL AUTHORITY

| Prepared by: | Head of Department | Dr. Abdul Aleem Name | Date: 14/11/2024 |
| --- | --- | --- | --- |
|  |  | Signature |  |
| Reviewed & Approved by: | Laboratory Head | Dr. Abdul Aleem Name | Date: 14/11/2024 |
|  |  | Signature |  |
| Issued by: | Quality Manager | Dr. Sai Varun Name | Date: 14/11/2024 |
|  |  | Signature |  |

- RECORD OF AMENDMENTS

| S. No. | Ver. No. | Ver. Date | Rev. No. | Rev. Date | Description of Changes |
| --- | --- | --- | --- | --- | --- |

| Document Copy No. | Name / Designation of the holder of the document copy | Type of Copy | Version No. | Revision No. |
| --- | --- | --- | --- | --- |
| 01/10 | Managing Director | Controlled Copy | 01 | 00 |
| 02/10 |  |  |  |  |
| 03/10 |  |  |  |  |
| 04/10 |  |  |  |  |
| 05/10 |  |  |  |  |
| 06/10 |  |  |  |  |
| 07/10 |  |  |  |  |
| 08/10 |  |  |  |  |
| 09/10 |  |  |  |  |
| 10/10 |  |  |  |  |

- DISTRIBUTION LIST

- LIST OF ABBREVIATIONS

| S. No. | Abbreviations | Expansion |
| --- | --- | --- |
| 1 | °C | Degree Centigrade |
| 2 | EDTA | Ethylenediamine Tetra-acetic Acid |
| 3 | EQAS | External Quality Assurance Scheme |
| 4 | ESR | Erythrocyte Sedimentation Rate |
| 5 | HOD | Head of Department |
| 6 | HM | Haematology |
| 7 | INR | International Normalized Ratio |
| 8 | IQC | Internal Quality Control |
| 9 | ISI | International Sensitivity Index of the reagent |
| 10 | MSP | Management System Procedure |
| 11 | N/A | Not Applicable/Available |
| 12 | QC | Quality Control |
| 13 | QM | Quality Manager |
| 14 | QNS | Quantity Not Sufficient |
| 15 | rpm | Revolutions per minute |
| 16 | SOP | Standard Operating Procedure |
| 17 | TRF | Test Requisition Form |
| 18 | TDPL | TRUSTlab Diagnostics Private Limited |

- Objective

- The purpose of this SOP is to describe the procedure for the demonstration of Plasmodium species in thick and thin blood films. This SOP also provides information about the test principle, clinical significance and other details.

- Scope

- This procedure applies to all staff of the Haematology Department tasked with the duty of performing these tests.

- Responsibilities

It is the responsibility of the technicians of the Haematology Department to comply with this SOP.

HOD shall ensure implementation.

The Quality Manager is responsible for monitoring, reviewing and ensuring compliance with this procedure.

All other TRUSTlab staff involved in this process (including contract personnel) are responsible for complying with this procedure.

- Definition

- A malarial smear, also known as a blood smear for malaria, is a microscopic examination of a blood sample to detect malaria parasites. It's a common diagnostic tool for malaria and involves spreading a blood sample on a slide, staining it, and then examining it under a microscope to identify the parasite.

- Test Principle

The definitive diagnosis of Malaria is based on the demonstration of the parasites in the blood. Giemsa Stain is a special stain designed for malarial thick and thin smears used to diagnose the presence of blood parasites.

Method: Staining and microscopy

- Specific Performance Characteristics

Measurement Range: Parasitic Index: ≤ 0.1 %, 0.2 – 0.9%, 1.0 – 2.0%, 2.1 – 5.0%, > 5.0%.

Clinical Reportable Range: Negative or Positive

- Type of Sample

3 ml Whole Blood

Sample Retention: Sample is retained for 24 hours at 2 – 8℃.

Sample Rejection Criteria:

      - Quantity not sufficient (QNS)

      - Sample received in wrong vacutainer

      - No TRF; Grossly incomplete TRF; Errors in labeling

      - Grossly haemolysed specimens

      - Highly lipemic specimens

      - Highly icteric specimens

      - Sample received beyond stability period

      - Incomplete TRF details or labelling errors are clarified with Customer/Doctor/ Patient before sample is accepted for processing. If critical information is still not obtained, the sample is rejected.

- Type of Container & Additives

K2 EDTA Vacutainer

- Patient Preparation

No special preparation.

- Equipment & Reagents

Equipment

      - Coplin jars

      - Timer

      - Microscope

Reagents

      - Giemsa Stain, stock

      - Methyl alcohol, absolute

      - Buffered water (pH 6.8 – 7.0.)

- Safety

Handle all samples as potentially infectious.

Handle all reagents with care and avoid contact with eye, mouth and skin.

Do not perform mouth pipetting.

Discard used reagents and samples as per disposal procedure.

- Calibration Procedure

- N/A.

- Test Procedure

Procedure

      - Use separate slides for thick and thin smear.

      - Thin film (a): Bring a clean spreader slide, held at a 45° angle, toward the drop of blood on the specimen slide. Allow thin smears to air dry.

      - Thin film (b): Wait until the blood spreads along the entire width of the spreader slide.

      - Thin film (c): While holding the spreader slide at the same angle, push it forward rapidly and smoothly.

      - Thick film: Using the corner of a clean slide, spread the drop of blood in a circle the size of a dime (diameter 1-2 cm). Do not make the smear too thick or it will fall off the slide. (You should be able to read newsprint through it.)

      - Wait until the thin and thick films are completely dry before staining. Fix the thin film with methanol (100% or absolute) and let it dry completely before staining. The thick film should not be fixed.

Staining Procedure

    - Thin Blood Film

      - Fix air-dried film in absolute methanol by dipping the film briefly (two dips) in a Coplin jar containing absolute methanol.

      - Remove and let it dry.

      - Stain with diluted Giemsa stain (1:10, vol/vol) for 5-10 min. For a 1:10 dilution, add 1 part of stock Giemsa to 9 parts of buffered water in a Coplin jar.

      - Wash, by briefly dipping the slide in and out of a Coplin jar of buffered water (one or two dips). Note: Excessive washing will decolorize the film.

      - Let air dry in a vertical position.

    - Thick Blood Film

      - Allow film to air dry thoroughly. Do not dry films in an incubator or by heat, because this will fix the blood and interfere with the lysing of the RBCs. Note: If a rapid diagnosis of malaria is needed, thick films can be made slightly thinner than usual, allowed to dry for 1 h, and then stained.

      - DO NOT FIX.

      - Dehemoglobinisation: Place the smear in a vertical position in a glass container containing distilled water for 5-10 minutes. When the film becomes white, it is taken out and allowed to dry in an upright position.

      - Stain with diluted Giemsa stain (1:10, vol/vol) for 5-10 min. For a 1:10 dilution, add 1 part of stock Giemsa to 9 parts of buffered water in a Coplin jar.

      - Wash by placing film in buffered water for 3 to 5 min. Let air dry in a vertical position.

Reporting

- Three components of the malarial parasite are found before identification is made. The genus and species of the organism identified in positive films is reported. Refer to the table for Laboratory Differentiation of Malarial Parasites below of this section. Giemsa stain colors the components of blood and parasites as follows:

      - Erythrocytes: pale red

      - Nuclei of white blood cells: purple with pale purple cytoplasm

      - Eosinophilic granules: bright purple-red

      - Neutrophilic granules: deep pink-purple

      - Malarial parasites, when present:

        - Cytoplasm: blue

        - Nuclear material: red to purple-red

        - Pigment granules: red to brown to black, depending on species.

      - Other blood parasites such as Babesia sp., Trypanosomes, and Leishmaniae:

        - Nuclear and Cytoplasmic staining characteristics will be like those seen in malarial parasites.

      - Microfilariae:

        - Sheath: may not always stain with Giemsa

        - Nuclei: blue to purple

Slide Review Procedure:

- Minimum 300 fields are examined under oil immersion using the 100X oil immersion objective.

Laboratory Differentiation of Malarial Parasites

| Tissue Response | P.V. Plasmodium vivax | P.O. Plasmodium ovale | P.M. Plasmodium malariae | P.F. Plasmodium falciparum |
| --- | --- | --- | --- | --- |
| Enlarged RBC | + | + | - | - |
| Oval shape of RBC | - | + | - | - |
| Pale colour | + | + | - | - |
| Fringed border | - | + | - | + |
| Schuffner’s granules | + | + | - | - |
| Maurer’s granules | - | - | - | + |
| Ziemann dots | - | - | + | - |
| Crenation & dark stain on border of RBC | - | - | - | + |
| Parasite Response | P.V. Plasmodium vivax | P.O. Plasmodium ovale | P.M. Plasmodium malariae | P.F. Plasmodium falciparum |
| All forms in peripheral blood | + | + | + | - |
| Very ameboid rings | + | - | - | - |
| Double chromatin dots | (unusual) + | - | - | + |
| Multiple infection in RBC | + | - | - | ++ |
| Band forms | - | - | + | - |
| Crescents (gametocytes) | - | - | - | + |

- Quality Control Procedure

Internal Quality Control

      - Known positive unstained smears and negative smears are run to check the validation of stain before in use and same used for quality check randomly.

      - Staining Procedure: Quality Control

        - To ensure that proper staining results have been achieved, a positive smear (malaria) is included with each new batch of working Giemsa stain. Alternatively, the specimen being stained may be used as the organisms and/or the white blood cells are a built-in quality control. They are prepared from a patient’s blood and stored for future use in the following manner:

        - Choose a patient blood specimen, anticoagulated with EDTA, that has enough parasites so that at least one is found in every two to three fields.

        - Make as many thin smears as possible, preferably within one hour after the blood was drawn from the patient.

        - Allow the smears to dry quickly at room temperature.

        - Fix the smears in absolute (100%) methanol; allow them to dry.

        - Wrap in tissue and place them, touching front to back, in a box without separating grooves.

        - Label the outside of the box with the species, date and “Giemsa control slides”.

        - Store the slides.

        - Just before use, remove the smear from the box; label the slide “+ malaria” and the present date. The smear is now ready for staining since it was previously fixed.

External Quality Control

- ILC – Dr. Lal Path Labs.

- Interferences & Cross Reactions

- N/A.

- Result Calculation

- Parasitic Index= 100 × Number of parasites identified in X number of RBCsX number of RBCs

- Biological Reference Interval

- Normal blood: Malarial parasite not detected.

- Reportable Interval

- Presence of Malarial parasite.

- Procedure when Results are not in Range

- N/A.

- Critical Alert Values

- Presence of Malarial parasite - first time.

- Laboratory Clinical Interpretation

Malaria is caused by protozoan parasites of the genus Plasmodium. Four species of Plasmodium can produce the disease in its various forms:

      - Plasmodium falciparum

      - Plasmodium vivax

      - Plasmodium ovale

      - Plasmodium malaria

P. falciparum is the most widespread and dangerous of the four: untreated it can lead to fatal cerebral malaria.

Malaria is complex, but it is a curable and preventable disease.

- Potential Source of Variability

      - Variability between readers.

      - Variability due to technique.

      - Variability between slides.

      - Very low parasitaemia.

      - Sequestration of parasitized red cells in the micro vasculature and maturation in broods.

      - Patients who have been recently or partially treated with antimalarial drugs or who have taken prophylactic drugs.

      - Technical factors such as poorly prepared or stained blood films, poor microscopy, inexperienced technicians, etc.

- References

      - Practical Hematology, J. Dacie and M. Lewis, Tenth edition, 2006. Page No 101-103.

      - Procedure for Managing Internal QC Program (TDPL/QA/MAN-001: MSP-013).