SOP – 015

PROCEDURE FOR

SEPARATION OF PLASMA AND SERUM

- TABLE OF CONTENTS

TABLE OF CONTENTS	2

APPROVAL AUTHORITY	3

RECORD OF AMENDMENTS	4

DISTRIBUTION LIST	5

LIST OF ABBREVIATIONS	6

1.	Objective	7

2.	Scope	7

3.	Responsibilities	7

4.	Definition	7

5.	Procedural Principle	7

6.	Specific Performance Characteristics	7

7.	Type of Sample	8

8.	Type of Container & Additives	8

9.	Patient Preparation	8

10.	Equipment & Reagents	8

11.	Safety	8

12.	Calibration Procedure	9

13.	Procedure	9

14.	Quality Control Procedure	10

15.	Interferences & Cross Reactions	10

16.	Result Calculation	10

17.	Biological Reference Interval	10

18.	Reportable Interval	10

19.	Procedure when Results are not in Range	11

20.	Critical Alert Values	11

21.	Laboratory Clinical Interpretation	11

22.	Potential Source of Variability	11

23.	References	11

- APPROVAL AUTHORITY

| Prepared by: | Head of Department | Dr. Abdul Aleem Name | Date: 14/11/2024 |
| --- | --- | --- | --- |
|  |  | Signature |  |
| Reviewed & Approved by: | Laboratory Head | Dr. Abdul Aleem Name | Date: 14/11/2024 |
|  |  | Signature |  |
| Issued by: | Quality Manager | Dr. Sai Varun Name | Date: 14/11/2024 |
|  |  | Signature |  |

- RECORD OF AMENDMENTS

| S. No. | Ver. No. | Ver. Date | Rev. No. | Rev. Date | Description of Changes |
| --- | --- | --- | --- | --- | --- |

| Document Copy No. | Name / Designation of the holder of the document copy | Type of Copy | Version No. | Revision No. |
| --- | --- | --- | --- | --- |
| 01/10 | Managing Director | Controlled Copy | 01 | 00 |
| 02/10 |  |  |  |  |
| 03/10 |  |  |  |  |
| 04/10 |  |  |  |  |
| 05/10 |  |  |  |  |
| 06/10 |  |  |  |  |
| 07/10 |  |  |  |  |
| 08/10 |  |  |  |  |
| 09/10 |  |  |  |  |
| 10/10 |  |  |  |  |

- DISTRIBUTION LIST

- LIST OF ABBREVIATIONS

| S. No. | Abbreviations | Expansion |
| --- | --- | --- |
| 1 | °C | Degree Centigrade |
| 2 | EDTA | Ethylenediamine Tetra-acetic Acid |
| 3 | EQAS | External Quality Assurance Scheme |
| 4 | HOD | Head of Department |
| 5 | HBV | Hepatitis B Virus |
| 6 | HIV | Human Immunodeficiency Syndrome |
| 7 | HM | Haematology |
| 8 | IQC | Internal Quality Control |
| 9 | MSP | Management System Procedure |
| 10 | N/A | Not Applicable/Available |
| 11 | NS | Normal Saline |
| 12 | QC | Quality Control |
| 13 | QM | Quality Manager |
| 14 | QNS | Quantity Not Sufficient |
| 15 | Rpm | Revolutions per minute |
| 16 | SOP | Standard Operating Procedure |
| 17 | TDPL | TRUSTlab Diagnostics Private Limited |

- Objective

- The purpose of this SOP is to describe the procedure for separating plasma and serum from whole blood. This SOP also provides information about the procedural principle and other details.

- Scope

- This procedure applies to all staff of the Haematology Department tasked with the duty of performing these tests.

- Responsibilities

It is the responsibility of the technicians of the Haematology Department to comply with this SOP.

HOD shall ensure implementation.

The Quality Manager is responsible for monitoring, reviewing and ensuring compliance with this procedure.

All other TRUSTlab staff involved in this process (including contract personnel) are responsible for complying with this procedure.

- Definition

- Serum and plasma, both liquid components of blood, are separated using centrifugation after different collection methods. Serum is the liquid remaining after blood clots, while plasma is obtained from blood treated with an anticoagulant.

- Procedural Principle

The separation of serum and plasma from blood using a centrifuge relies on the principle of differential density and centrifugal force. Denser cellular components, like red blood cells and platelets, are forced to the bottom of the tube during high-speed rotation, while the less dense liquid (plasma or serum) remains at the top.

Method: Centrifugation

- Specific Performance Characteristics

- N/A

- Type of Sample

- Blood collected following universal procedure for venipuncture.

- Type of Container & Additives

K2 EDTA anticoagulant vacutainer

Plain vacutainer

- Patient Preparation

- N/A.

- Equipment & Reagents

Equipment

      - Calibrated Centrifuge

      - Test tubes

      - Transfer pipettes / Droppers

      - Test tube rack

- Safety

Follow universal precautions like wearing suitable gloves and eye/face protection while handling all reagents, samples and operating the instrument.

Universal precautions imply that all body fluids are treated as if known to be infectious for Human Immunodeficiency Virus (HIV), Hepatitis B Virus (HBV) or other Blood borne Pathogens.

Dispose of all waste in accordance with applicable national and/or local regulations.

Some reagents contain sodium azide, which may react with copper or lead plumbing to form explosive metal azides. Use caution in disposing of these reagents. If disposing to drain, flush with large volumes of water to prevent azide buildup.

Waste material containing patient samples or biological products should be considered biohazardous when disposing or treating.

Clean up all spills immediately and thoroughly. Disinfect the area for any spills involving biohazardous materials. Dispose of all contaminated materials appropriately.

- Calibration Procedure

- N/A.

- Procedure

The requisition & sample type should agree

Is the sample stable?

Collect new sample

Transfer required quantity into another tube

Known & confirmed blood groups – A1, B or O

Centrifuge sample

Separation of plasma/serum

Preparation of 2 – 5 % RBC suspension

Preparation of pooled cells of A1, B & O blood groups protocol

Is the sample stable?

Yes

No

Yes

No

Part 1. Review and Process Flow of Samples

Part 2: Separation of Serum/Plasma

      - The requisition and sample should match.

      - Check the volume of the sample (at least 1 ml).

      - Centrifuge the tube at 2000 rpm for 5 minutes.

      - Check the sample for abnormal appearance after centrifugation (e.g. hemolysis)

      - Separate the serum (red vacutainer) / plasma (EDTA vacutainer) into another tube.

      - The sediment red cells can be used for the preparation of 2–5% red cell suspension.

Part 3: Preparation of Washed Cells

      - Take 100 µl of the red cells sediment.

      - Add 3 ml of normal saline (NS) and mix well.

      - Centrifuge at 3000 rpm for 3 minutes.

      - Decant the supernatant.

      - To the remaining cell button repeat steps b, c and d, thrice.

      - After the final wash, to prepare 2-5% suspension of red cells, add 2-5 ml of NS until the appearance of tomato red colour of the solution.

- Quality Control Procedure

- N/A.

- Interferences & Cross Reactions

- N/A.

- Result Calculation

- N/A.

- Biological Reference Interval

- N/A.

- Reportable Interval

- N/A.

- Procedure when Results are not in Range

- N/A.

- Critical Alert Values

- N/A.

- Laboratory Clinical Interpretation

- N/A.

- Potential Source of Variability

- N/A.

- References

- Practical Hematology, Dacie and Lewis, Tenth edition.