SOP - 005

PROCEDURE FOR

AEROBIC CULTURE

- TABLE OF CONTENTS

TABLE OF CONTENTS	2

APPROVAL AUTHORITY	3

RECORD OF AMENDMENTS	4

DISTRIBUTION LIST	5

LIST OF ABBREVIATIONS	6

1.	Objective	7

2.	Scope	7

3.	Responsibilities	7

4.	Definition	7

5.	Test Principle	7

6.	Specific Performance Characteristics	7

7.	Type of Sample	7

8.	Patient Preparation	8

9.	Type of Container & Additives	8

10.	Equipment & Reagents	8

11.	Safety	8

12.	Calibration Procedure	9

13.	Test Procedure	9

14.	Quality Control Procedure	15

15.	Interfering Substances	15

16.	Result Calculation	16

17.	Biological Reference Interval	16

18.	Reportable Interval	17

19.	Procedure when Results are not in Range	17

20.	Critical Alert Values	17

21.	Laboratory Clinical Interpretation	17

22.	Potential Source of Variability	17

23.	References	17

- APPROVAL AUTHORITY

| Prepared by: | Head of Department | Dr. Sadiya Shahnaz Name | Date: 14/11/2024 |
| --- | --- | --- | --- |
|  |  | Signature |  |
| Reviewed & Approved by: | Laboratory Head | Dr. Abdul Aleem Name | Date: 14/11/2024 |
|  |  | Signature |  |
| Issued by: | Quality Manager | Dr. Sai Varun Name | Date: 14/11/2024 |
|  |  | Signature |  |

- RECORD OF AMENDMENTS

| S. No. | Ver. No. | Ver. Date | Rev. No. | Rev. Date | Description of Changes |
| --- | --- | --- | --- | --- | --- |

| Document Copy No. | Name / Designation of the holder of the document copy | Type of Copy | Version No. | Revision No. |
| --- | --- | --- | --- | --- |
| 01/10 | Managing Director | Controlled Copy | 01 | 00 |
| 02/10 |  |  |  |  |
| 03/10 |  |  |  |  |
| 04/10 |  |  |  |  |
| 05/10 |  |  |  |  |
| 06/10 |  |  |  |  |
| 07/10 |  |  |  |  |
| 08/10 |  |  |  |  |
| 09/10 |  |  |  |  |
| 10/10 |  |  |  |  |

- DISTRIBUTION LIST

- LIST OF ABBREVIATIONS

| S. No. | Abbreviations | Expansion |
| --- | --- | --- |
| 1 | ATCC | American Type Culture Collection |
| 2 | BAL | Bronchoalveolar Lavage |
| 3 | CFU | Colony Forming Unit |
| 4 | CSF | Cerebral Spinal Fluid |
| 5 | ET | Endotracheal |
| 6 | FTM | Fluid Thioglycollate Medium |
| 7 | MI | Microbiology |
| 8 | NABL | National Accreditation Board for Testing and Calibration Laboratories |
| 9 | SOP | Standard Operating Procedure |
| 10 | TCBS | Thiosulfate Citrate Bile Salts Sucrose |
| 11 | TDPL | TRUSTlab Diagnostics Private Limited |
| 12 | XLD | Xylose Lysine Deoxycholate |

- Objective

  - This SOP describes the procedure for processing of samples for possible pathogens in the urine, stool, sputum, pus, pus swabs, exudates, CSF, swabs (Eye, Rectal, Skin, Throat, Nasal, Ear, wound), sterile body fluids, vaginal swab, cervical swab, semen, tissues and Catheter Tips, drain fluid samples based on the cultural properties on different media and the microscopic observation of the pathogen.

- Scope

This procedure applies to all staff of the Microbiology Department tasked with the duty of performing these tests.

- Responsibilities

It is the responsibility of the technicians of the Microbiology Department to comply with this SOP.

HOD shall ensure implementation.

The Quality Manager is responsible for monitoring, reviewing and ensuring compliance with this procedure.

All other TRUSTlab staff involved in this process (including contract personnel) are responsible for complying with this procedure.

- Definition

An aerobic culture is a laboratory test that identifies aerobic organisms, which require oxygen to survive. It can be used to identify infections in the blood, urine, throat, tissue, and wounds.

- Test Principle

Appropriate growth requirements are provided through culture media for sample inoculation and each bacterial cell in the sample multiplies after certain time at appropriate temperature to form an individual colony forming unit.

- Specific Performance Characteristics

- N/A.

- Type of Sample

- Urine, stool, sputum, pus, pus swabs, exudates, CSF, swabs (Eye, Rectal, Skin, Throat, Nasal, Ear, wound), sterile body fluids, vaginal swab, cervical swab, semen, tissues and Catheter Tips, drain fluid.

- Patient Preparation

- Refer to the primary sample collection manual.

- Type of Container & Additives

- Sterile & leak-proof capped containers, sterile swabs, syringes.

- Equipment & Reagents

Calibrated Inoculation loop

Microscope

Glass Slides

Gram stain kit

Bunsen burner

Culture Media

Marker

Incubator

Alcohol spirit

Cotton/Tissue paper

Biosafety Cabinet

- Safety

The use of disposable gloves and proper biohazardous clothing is STRONGLY RECOMMENDED while running the test.

In case there is a cut or wound on the hand, DO NOT PERFORM THE TEST.

Do not smoke, drink or eat in areas where specimens or kit reagents are being handled.

Tests are for in vitro diagnostic use only and should be run by competent personnel only.

Do not pipette by mouth.

All materials used in the assay and samples should be decontaminated in 1% sodium hypochlorite solution for 30-60 min. They should be disposed of in accordance with established safety procedures.

Wash hands thoroughly with soap or any suitable detergent, after the use of the kit. Consult a physician immediately in case of accident or contact with eyes, in the event that contaminated material is ingested or comes in contact with skin punctures or wounds.

Spills should be decontaminated promptly with Sodium Hypochlorite or any other suitable disinfectant.

Suitable infection prevention precautions to be followed.

- Calibration Procedure

Biosafety Cabinet and centrifuge calibration is done on an annual basis by an NABL accredited calibration agency. Necessary maintenance measures are undertaken for all the equipment.

Innoculation Loop (1 µl) Calibration Procedure

- Take 1000 µl of distilled water mixed with a drop of methylene blue in a test tube.  Take a small piece of filter paper and weigh it with a calibrated weighing balance and make a note of it on the loop calibration worksheet. Add 20 loopfuls with 1µl of inoculation loop of methylene blue mixed distilled water to the filter paper and measure the weight. The difference in weight of the filter paper should be 20mg + 2. This process is done 5 times to check for the appropriate transfer of liquid and an average weight transfer per loop should be 1mg + 0.1. Any deviation in this measurement indicates that the loop is not transferring the required amount of inoculum and is replaced by a new loop.

- Test Procedure

Precautions

      - Liquid media may be inoculated first when processing fluid specimens. This reduces the chances of carry-over from contaminated solid media. However, liquid media should be inoculated after the solid media when swabs and faeces are examined, to avoid diluting the organisms contained on or in the sample and to avoid any non-viable organisms present in a liquid medium being transferred to other liquid media, solid media or to slides.

      - Smears for staining are usually made after all culture media have been inoculated to avoid carry-over of contaminants that may be on the surface of the slide. However, there may be occasions where the smear is required prior to culture and then a sterile slide should be used.

      - For the isolation of individual colonies, the inoculum should be spread, usually by using a sterile loop, taking care to avoid the edges of the plate where contaminants are more likely to be located.

      - Antimicrobial discs for identification (eg Optochin, Bacitracin) may be added as appropriate. Discs should be placed near the edge of the plate, between the areas covered by the first and second spread, to avoid total inhibition of very susceptible organisms.

      - As a minimum requirement, all culture plates and containers must be labeled to identify the patient’s name or laboratory number or barcode. Additional labeling, including date of culture or sub-culture will be necessary for selected specimens, such as those requiring prolonged incubation or sub-culture from enrichment broth.

      - To work safely and minimize risks of cross contamination suitable racks should also be used when inoculating, incubating or storing liquid cultures or culture plates.

Aseptic Techniques

      - When handling specimens or cultures, aseptic technique is important to avoid their contamination and to protect the worker from infection from the sample.

      - The following points should be observed when culturing specimens or performing subcultures:

        - With the exception of urine specimens, caps and lids from containers should not be placed on the workbench but retained in the hand while the sample is being processed, taking care not to contaminate the hand or cap. Caps and lids should be replaced as soon as possible.

        - If the work is being carried out on the open bench, a Bunsen burner/loop sterilizer should be in close proximity to sterilize loops or wires.

        - Keep samples away from the face when opening culture containers.

        - Aerosol production should be minimized by:

          - Opening caps slowly as the contents of containers are sometimes under pressure.

          - Avoiding vigorous swirling or shaking of the sample prior to opening.

          - Cooling loops that have been heated before use.

          - Avoiding expelling the last drop from a pipette.

          - If forceps or scissors are used when handling specimens, they should be heated in a Bunsen flame/loop sterilizer and allowed to cool before use.

          - Loops and wires should be heated in the Bunsen flame/loop sterilizer with the loop end downwards until red-hot, then removed and allowed to cool. Care should be taken that the loop does not contain fluid or large particles of matter that may ‘splatter’ when placed in the flame, and that the entire loop/wire up to the loop holder is heated to red-hot state.

        - Urine samples are inoculated with Calibrated loops holding 0.001 microlitres of sample Suprapubic aspirates/urine obtained during Cystoscopy is inoculated with Calibrated loops holding 0.01 microlitres of sample to identify a lower count of bacteria.

| Specimen Type | Media | Loop Size | Culture Method |
| --- | --- | --- | --- |
| Urine | MICHROM agar plate | 1µL | Semiquantitative Streak plate |
| Sputum, BAL | Blood agar, Chocolate agar MacConkey agar | 10µL | Quadrant streak plate (purulent portion of the sample is used for inoculation) |
| Pus, Exudates, Tissues | Blood agar, MacConkey agar Fluid Thioglycollate Medium (FTM) | 10µL | Quadrant streak plate (purulent portion of the sample is used for inoculation; tissue to be rolled over the media) |
| CSF, sterile body fluids | Blood agar, Chocolate agar, MacConkey agar plates Fluid Thioglycollate Medium | 10µL | Quadrant streak plate |
| Swabs, Catheter Tips / Stents, ET secretions, Tracheal Secretions, Drain Fluids, Semen | Blood agar, MacConkey agar | 10µL | Quadrant streak plate (tips or stents are rolled over the media plate 3-4 times) |
| Stool | Selenite F Broth & Alkaline Peptone Water if Stool is Liquid and Vibrio Cholerae is suspected. Mac Conkey agar, XLD and TCBS agar | 10µL | Quadrant streak plate (purulent or portion of the sample is used for inoculation) |

Semi-Quantitative Streak Plate

Quadrant Streak Plate

Special Instructions for Sample Processing

      - Urine

        - Note the colour and turbid nature of the sample and look for any macroscopic particles.

        - Mix the urine sample well and gently to avoid foaming.

        - Dip the end of a sterile (flamed and cooled) calibrated loop holding 0.001µL of sample or calibrated loop holding 0.01µL for suprapubic aspirates into the urine to just below the surface and remove a loopful of urine taking care to ensure that no part of the urine is carried over the stainless-steel shank.

        - Inoculate a loopful of the uncentrifuged sample on to MICHROM agar by semi-quantitative streak technique.

      - Pus, Exudates, Bronchoalveolar lavage, Endotracheal secretions, Tracheal secretions

        - Note the colour, consistency, odour and any other peculiar feature such as the presence of granules.

        - Inoculate the most purulent portion of sample directly first into blood agar or chocolate agar and MacConkey agar plates, then make smears on slide for Gram Stain and finally it is inoculated into FTM to recover if very few organisms are present.

        - Incubate the plates and glucose broth aerobically at 37°C for 18-24 hours.

      - Stool

        - Note the color, consistency, the presence of mucus, frank pus and blood in the faecal specimen.

        - If the sample appears Liquid and like rice water flakes, Vibrio cholerae is suspected and immediate Hanging drop preparation is done to check dartingly motile organisms morphologically resembling Vibrio cholerae and if positive, preliminary report will be released.

        - Emulsify a small portion (1 g, size of matchstick head) of stool sample which has blood, pus or mucus in Selenite F broth and inoculate immediately on blood agar, Mac Conkey agar, XLD agar. Incubate the stool sample for 4-6 hours and reinoculated on the blood agar, Mac Conkey agar, XLD agar and alkaline peptone water if Vibrio cholerae is suspected (if sample is Liquid) and inoculated on to blood agar, MacConkey, XLD agar and incubated overnight at 37°C.

      - Cerebrospinal fluid (CSF), Sterile body fluids

        - Note the color, turbidity and a clot or deposit formation.

        - Transfer the sample into a sterile tube and cap it tightly and centrifuge the sample at 1500rpm for about 10 minutes.

        - Decant the supernatant into original container and deposit is used for inoculation and staining.

        - Inoculate the deposit onto blood agar / chocolate agar, MacConkey agar plates and into FTM.

        - Smears are made from sediment /direct sample if turbid and Gram stain is done.

        - If stained smears show presence of Pus cells and any organisms, same is considered as Critical Alert and informed immediately to the clinician and report is released as interim report.

      - Tissue in Normal Saline

        - Tissue removed from a part of a biopsy procedure or during surgery in patients should be sent in normal saline. External contamination should be avoided.

        - Culture methodology and choice of media for culture in tissue depends on the sample nature and the site of the lesion. In some cases, only a search for a known pathogen is conducted such as vegetations on a heart valve. On the other hand, a multidirectional approach is warranted when dealing with lesions of unknown etiology.

        - Make a note of the size of the specimen, the gross morphology and the exact site in the body from where the specimen is obtained.

        - Look for any purulent or pus-filled areas or any necrotic areas. These should be sampled separately from the rest of the specimen.

        - The tissue should be cut with a sterile scalpel into small pieces and each of the pieces should be used for culture.

        - Take a small piece of the tissue, cut up from a representative area of the tissue, and inoculate it onto blood agar or chocolate agar, Mac Conkey agar and FTM.

        - One of the pieces of the tissue should be used for making two smears. One smear should be used for a Gram stain. The other smear may be used for other stainings.

      - Swabs

        - All types of swabs should be inoculated onto a blood agar or chocolate agar and MacConkey agar plates and smears made for appropriate stains. If the swab is dry - moisten swab with sterile peptone water or sterile saline. After inoculation, the swab should be used for making smears for staining.

      - Sputum

        - Inspect the sputum sample to note the colour, presence of any grossly purulent areas, the presence of blood or mucus. Samples with only saliva should be do not give good results.

        - Sample is inoculated onto blood agar, chocolate agar and MacConkey agar.

        - Gram stain is checked to grade of sputum for processing.

      - Catheter Tips & Stents

        - Dip a forceps in 95% alcohol and flame to sterilize. Allow them to cool.

        - Transfer the catheter tip or stent from the sterile tube to the blood agar and MacConkey’s agar by a sterile forceps and inoculate rolling over the surface 3-4 times (Maki’s roll technique).

        - Using slight pressure, roll the catheter tips or stent backwards and forwards across the blood agar plate at least 4 times. Care should be taken to see that there is good contact maintained between the surface of the catheter tip or stunt and the blood agar plate.

        - Transfer the tip to FTM tube and incubate at 37˚C.

        - After the procedure, flame the forceps and dry the same.

      - Catheter Tips & Stents – Semi Quantitative

        - Aseptically transfer the catheter segment from the container to a sterile test tube.

        - Holding the tube vertically, flush the catheter lumen with a known quantity of peptone water (usually about 4mL of peptone water is taken).

        - The liquid is flushed down the lumen of the catheter with a syringe and needle.

        - Cap the tube or place the cotton plug tightly and vortex to remove any undislodged bacteria.

        - Make a 1:10 dilution, 1:100 dilution of the diluted material and inoculate direct and diluted material from the tube onto blood agar and Mac Conkey agar plate.

- Quality Control Procedure

QC of culture media is checked with American Type Culture Collection – Microbial strains

Sample collection containers are checked for their sterility and sample stability as per lot change.

- Interfering Substances

- Sample collection process and transportation should be done appropriate to the sample to avoid sample contamination with the commensal flora and environmental contaminants. Adequate aseptic precautions to be taken to avoid contamination during culturing process.

- Result Calculation

Culture media are incubated for 18-36 hours for identification of bacterial growth. Suspected pathogen colonies are identified and further processing of the colonies for their presumptive identification and susceptibility testing.

Colony count calculation for Urine culture

| Loop Volume | Formulae | No. of Colonies 1000/mL | No. of Colonies 10000/mL | No. of Colonies 100000/mL | No. of Colonies >100000/mL |
| --- | --- | --- | --- | --- | --- |
| 1µL | No. of Colonies X 1000 | 1 | 10 | 100 | >100 |
| 10µL | No. of Colonies X 100 | 10 | 100 | 1000 | >1000 |

Growth Quantification for other samples (other than urine and catheter tips)

| Growth | Pattern of growth |
| --- | --- |
| Scanty | Limited to well |
| Moderate | Extended up to secondary streak line |
| Heavy | Confluent growth till tail end |

- Significant Growth Quantification for catheter tips:

      - Rolled-out tip method: > 15 CFU

      - Broth dilution through lumen: > 100 CFU =  No.  of Colonies ×104

- Biological Reference Interval

- No growth is observed for all samples from sterile body sites unless there is an infection. Commensal growth may be observed in sputum, BAL, ET/ tracheal secretions, semen, stool, urine, specimens from – oral cavity, throat swab, eye, ear, vagina, endocervix, stool, open wounds.

- Reportable Interval

- 72 Hours

- Procedure when Results are not in Range

- When the results are found inappropriate to the sample type, the sample is re-stained or checked with a repeat sample as per individual sample and growth discretion.

- Critical Alert Values

- All significant growth observed in samples from sterile body sites is informed as critical alert.

- Laboratory Clinical Interpretation

- Specific clinical interpretation is given as per sample type and pathogen isolated.

- Potential Source of Variability

The following factors play a crucial role in the result variations and appropriate care should be taken to eliminate them:

      - prior antibiotic usage by patient

      - sample type

      - sample collection

      - sample transport temperature

      - time to sample collection and inoculation

      - portion of sample used for inoculation

      - culture media

      - loop size and sterility

      - temperature and time of incubation.

- References

Practice of Medical Microbiology – Cruickshank