SOP - 001

PROCEDURE FOR HIV TRI-DOT TEST

- TABLE OF CONTENTS

TABLE OF CONTENTS	2

APPROVAL AUTHORITY	4

RECORD OF AMENDMENTS	5

DISTRIBUTION LIST	6

LIST OF ABBREVIATIONS	7

1.	Objective	8

2.	Scope	8

3.	Responsibilities	8

4.	Definition	8

5.	Test Principle	8

6.	Specific Performance Characteristics	8

7.	Type of Sample	9

8.	Patient Preparation	9

9.	Type of Container & Additives	9

10.	Equipment & Reagents	9

11.	Safety	9

12.	Calibration Procedure	10

13.	Test Procedure	10

14.	Quality Control Procedure	11

15.	Interfering Substances	11

16.	Result Calculation	12

17.	Biological Reference Interval	12

18.	Reportable Interval	12

19.	Procedure when Results are not in Range	12

20.	Critical Alert Values	12

21.	Laboratory Clinical Interpretation	13

22.	Potential Source of Variability	13

23.	References	14

- APPROVAL AUTHORITY

| Prepared by: | Head of Department | Dr. Sadiya Shahnaz Name | Date: 14/11/2024 |
| --- | --- | --- | --- |
|  |  | Signature |  |
| Reviewed & Approved by: | Laboratory Head | Dr. Abdul Aleem Name | Date: 14/11/2024 |
|  |  | Signature |  |
| Issued by: | Quality Manager | Dr. Sai Varun Name | Date: 14/11/2024 |
|  |  | Signature |  |

- RECORD OF AMENDMENTS

| S. No. | Ver. No. | Ver. Date | Rev. No. | Rev. Date | Description of Changes |
| --- | --- | --- | --- | --- | --- |

| Document Copy No. | Name / Designation of the holder of the document copy | Type of Copy | Version No. | Revision No. |
| --- | --- | --- | --- | --- |
| 01/10 | Managing Director | Controlled Copy | 01 | 00 |
| 02/10 |  |  |  |  |
| 03/10 |  |  |  |  |
| 04/10 |  |  |  |  |
| 05/10 |  |  |  |  |
| 06/10 |  |  |  |  |
| 07/10 |  |  |  |  |
| 08/10 |  |  |  |  |
| 09/10 |  |  |  |  |
| 10/10 |  |  |  |  |

- DISTRIBUTION LIST

- LIST OF ABBREVIATIONS

| S. No. | Abbreviations | Expansion |
| --- | --- | --- |
| 1 | CSD | Customer Service Department |
| 2 | EDTA | Ethylenediaminetetraacetic Acid |
| 3 | EQAS | External Quality Assurance Scheme |
| 4 | HIV | Human Immunodeficiency Syndrome |
| 5 | HLA | Human Leukocyte Antigen |
| 6 | NACO | National AIDS Control Organization |
| 7 | rpm | Rotations per minute |
| 8 | TDPL | TRUSTlab Diagnostics Private Limited |
| 9 | WHO | World Health Organisation |

- Objective

  - This procedure describes the step-by-step tasks involved in the examination of the HIV test using a rapid method.

- Scope

This procedure applies to all staff of the Serology Department tasked with the duty of performing these tests.

- Responsibilities

It is the responsibility of the technicians of the Serology Department to comply with this SOP.

HOD shall ensure implementation.

The Quality Manager is responsible for monitoring, reviewing and ensuring compliance with this procedure.

All other TRUSTlab staff involved in this process (including contract personnel) are responsible for complying with this procedure.

- Definition

The Human Immunodeficiency Virus (HIV) Antibody Rapid Test is a visual, rapid, sensitive and accurate immunoassay for the differential detection of HIV-1 & HIV-2 antibodies (IgG) in Human Serum or Plasma using HIV-1 & HIV-2 Antigens immobilized on an immunofiltration membrane.

- Test Principle

HIV antigens are immobilized on a porous immunofiltration membrane. Sample and reagents pass through the membrane and are absorbed into the underlying absorbent.

As the patient's sample passes through the membrane, HIV antibodies, if present, bind to the immobilized antigens. Conjugate binds to the Fc portion of the HIV antibodies to give distinct pinkish purple DOT(s) against a white background.

- Specific Performance Characteristics

| Sample panel | Indian panel | WHO panel |
| --- | --- | --- |
| Sensitivity | 100% | 100% |
| Specificity | 100% | 100% |

- Type of Sample

- Serum or Plasma must be used for testing. Should a delay in testing occur, store the sample at 2-8°C. Samples can be stored for upto a week. Do not use hemolysed, icteric, lipemic serum.

- Patient Preparation

- No special preparation of the patient is required prior to specimen collection by approved techniques. Consent form as approval for testing is necessary before testing. Testing will be done as per NACO guidelines.

- Type of Container & Additives

- Collect 2 ml of venous blood in a plain red topped/ yellow gel/ EDTA vacutainer.

- Equipment & Reagents

HIV TRI-DOT Test Device

Buffer Solution

Protein-A Conjugate

Sample Dropper

Centrifuge to separate serum or plasma

- Safety

The use of disposable gloves and proper biohazardous clothing is strongly recommended while running the test.

In case there is a cut or wound in hand, do not perform the test.

Do not smoke, drink or eat in areas where specimens or kit reagents are being handled.

Tests are for in vitro diagnostic use only and should be run by competent personnel only.

Do not pipette by mouth.

All materials used in the assay and samples should be decontaminated in 1% sodium hypochlorite solution for 30-60 min. They should be disposed of in accordance with established safety procedures.

Wash hands thoroughly with soap or any suitable detergent, after the use of the kit. Consult a physician immediately in case of an accident or contact with eyes, in the event that contaminated material is ingested or comes in contact with skin punctures or wounds.

Spills should be decontaminated promptly with Sodium Hypochlorite or any other suitable disinfectant.

- Calibration Procedure

- Centrifuge calibration is done on an annual basis by a NABL accredited calibration agency.

- Test Procedure

Take care of the following points before starting the test:

      - Bring all the reagents and specimens to room temperature (25°C-30°C) before beginning the test. The immunological sequence of reactions which take place during different procedural steps shows best performance at room temperature.

      - Place the required number of HIV TRI-DOT test devices at the workstation.

      - Tear off the pouch and take out the device for performing the test. Write the sample number to be tested on the device.

      - While adding sample/reagents to the device, be sure to ALLOW EACH SOLUTION TO SOAK IN BEFORE ADDING THE NEXT SOLUTION.

      - However, drops of each solution should be added in continuous stream to wet the entire area of membrane.

      - If the solution does not soak in within 40-60 seconds: observe the sample for any suspended particulate matter. If it is present, centrifuge the sample at 3500 r.p.m. for 15 min. and use a fresh device to re-run the test. All solutions and the sample should be added to the centre of the membrane.

      - For consistent results ensure free falling of drops on the membrane.

      - Do not use kit components beyond the expiration date.

      - The liquid conjugate should not be subjected to frequent temperature fluctuations.

      - The procedural sequence of reagent addition should be strictly adhered to avoid any discrepant results.

Add 3 drops of Buffer Solution to the centre of the device.

Hold the dropper vertically and add 1 drop of patient's sample (serum or plasma) using the sample dropper provided (use a separate sample dropper for each specimen to be tested).

Add 5 drops of Buffer Solution.

Add 2 drops of Liquid Conjugate directly from the conjugate vial.

Add 5 drops of Buffer Solution and READ THE RESULTS IMMEDIATELY.

- Quality Control Procedure

Each testing device has an inbuilt Control Dot to confirm the validity of the device. The appearance of the control dot after completing the procedural test indicates a valid test.

All kits and lots will be tested as per reagent acceptability criteria.

EQAS participation – IAMM VIROLOGY EQAS, MHL EQAS

- Interfering Substances

FROZEN SAMPLES:

- The HIV TRI-DOT Test is best when used with fresh samples that have not been frozen and thawed. However, most frozen samples will perform well if the following suggested procedure is followed:

      - Allow the sample to thaw in a vertical position in the rack. Do not shake the sample. This allows particles to settle to the bottom. If a centrifuge is available, the sample can be centrifuged at 3500 rpm for 15 min.

      - Insert the dropper just below the top surface of the sample and withdraw one drop of sample. If the above procedure still yields a high background, dilute 1 drop of sample with 2 drops of normal saline. Use 1 drop of this diluted sample in the test.

THICK OR VISCOUS SAMPLES:

- Whenever possible, clear specimens should be used. However viscous, thick or turbid samples which may sometimes take more than 40-60 seconds to flow through the membrane should be centrifuged at 3500 rpm for 15 min. and retested on a fresh device to avoid inconsistent results.

- Result Calculation

Test interpretation to be done only if the Control Dot is present at the time of reading.

NON-REACTIVE

      - If only One DOT (only the Control Dot) appears the specimen is non-reactive for antibodies either to HIV-1 or HIV-2. Interpret sample as non-reactive.

REACTIVE

      - If two DOTS, one for the control and the other for HIV-1 appear, the specimen is reactive for antibodies to HIV-1.

      - If two DOTS, one for the control and the other for HIV-2 appear, the specimen is reactive for antibodies to HIV-2.

      - If all the three DOTS, one each for control, HIV-1 & HIV-2 appear, the specimen is reactive for antibodies to HIV-1 & HIV-2.

INVALID

      - The test is considered invalid if the control dot is not formed. The test should be repeated with a new device.

- Biological Reference Interval

- Non-Reactive

- Reportable Interval

- 4 Hours

- Procedure when Results are not in Range

HIV Rapid test is done with HIV TRIDOT (J’Mitra) kit for all the tests billed under HIV Rapid test.

If the sample is Reactive by HIV TRIDOT method, then as per NACO guidelines:

      - a second rapid test with HIV 1/2 combo rapid Lateral Flow test method,

      - and a third method by ELISA are used to confirm the results.

- Critical Alert Values

- N/A

- However, all reactive HIV tests should be tested by three methods as per NACO guidelines and released after proper counselling. Pre-test counselling is assumed to be done by the doctor requesting the test. Post-Test Counselling is mandatorily done for all reactive patients, after taking treating doctor’s guidance by the lab doctors through CSD team or by the treating physician himself.

- Laboratory Clinical Interpretation

- The availability of rapid HIV testing in the hospital for emergency pre-operative screening provides a useful tool for clinicians that can inform treatment and disposition decision making for patients, as well as serves to confirm the reactive results by other methods. Although no formal studies have been done yet to evaluate the impact of rapid testing for practice, significant anecdotal experience is being gathered from literature that has been involved in screening, risk-targeted testing, or testing based on clinical suspicion.

- Potential Source of Variability

The kit works best when used with fresh samples. Samples which have been frozen and thawed several times contain particulates which can block the membrane, hence resulting in improper flow of reagents and high background colour which may make the interpretation of results difficult.

Optimum test performance depends on strict adherence to the test procedure as described in this manual. Any deviation from test procedure may lead to erratic results.

HIV-1 and HIV-2 viruses share many morphological and biological characteristics. It is likely that due to this, their antibodies have a cross reactivity of 30-70%. Appearance of dots for HIV-1 and HIV-2 antibodies on the test device does not necessarily imply co-infection from HIV-1 & HIV-2.

Some samples show cross reactivity for HIV antibodies. Following factors are found to cause false positive HIV antibody test results: Naturally occurring antibodies, Passive immunization, Leprosy, Renal Disorders, Tuberculosis, Myco-bacterium avium, Herpes simplex, Hypergammaglobulinemia, Malignant neoplasms, Rheumatoid arthritis, Tetanus vaccination, Autoimmune diseases, Blood Transfusion, Multiple myeloma, Haemophilia, Heat treated specimens, Lipemic serum, Anti-nuclear antibodies, T-cell leukocyte antigen antibodies, Epstein Barr virus, HLA antibodies and other retroviruses.

This is only a screening test. All samples detected reactive must be confirmed by using HIV Western Blot. Therefore, for a definitive diagnosis, the patient’s clinical history, symptomatology as well as serological data, should be considered. The results should be reported only after complying with the above procedure.

- References

HIV TRIDOT – J’Mitra kit insert

NACO Guidelines

- HIV Testing Protocol