# LABORATORY SAFETY MANUAL

As per Standard ISO 15189:2022 International Standard for Medical Laboratories

- TABLE OF CONTENTS

TABLE OF CONTENTS	2

APPROVAL AUTHORITY	4

RECORD OF AMENDMENTS	5

DISTRIBUTION LIST	6

LIST OF ABBREVIATIONS	7

1.	Introduction	8

2.	Objective	8

3.	Scope	8

4.	Responsibilities	8

5.	Authorization for Implementation	9

6.	Responsibilities of the HODs	9

7.	Reporting of Injuries	10

8.	Safety Violation Procedures	10

9.	Safety Audits	11

10.	Review of Safety Manual	11

11.	General And Personal Safety	11

12.	Pre-Exposure Prophylaxis	19

13.	Post Exposure Prophylaxis	20

14.	Fire Safety	24

15.	Chemical Safety:	27

16.	Chemical Hygiene Plan	34

17.	Centrifuge Safety	40

18.	Electrical Safety	42

19.	Emergency Eye-Wash Procedure	43

20.	Microbiological Hazards	44

21.	Fumigation	47

22.	Freezer And Refrigerator Safety	48

23.	Mecury Reduction and Spill Planning and Prevention	49

24.	Radioactive Material Handling	49

25.	Handling Of Hazardous Materials	53

26.	Ergonomics	56

27.	Management Of Incidents And Accidents	61

28.	General Housekeeping Procedure	64

29.	Microbiological Waste	68

30.	Bio-Medical Waste Handling And Disposal	69

31.	Bomb Threat	73

32.	Civil Disorder	75

33.	Contingency Plan	76

34.	Laboratory Safety Training and Information	77

35.	Reference	78

- APPROVAL AUTHORITY

| Prepared by: | Quality Manager | Dr. Sai Varun Name | Date: 14/11/2024 |
| --- | --- | --- | --- |
|  |  | Signature |  |
| Reviewed & Approved by: | Laboratory Head | Dr. Abdul Aleem Name | Date: 14/11/2024 |
|  |  | Signature |  |
| Issued by: | Quality Manager | Dr. Sai Varun Name | Date: 14/11/2024 |
|  |  | Signature |  |

- RECORD OF AMENDMENTS

| S. No. | Ver. No. | Ver. Date | Rev. No. | Rev. Date | Description of Changes |
| --- | --- | --- | --- | --- | --- |

| Document Copy No. | Name / Designation of the holder of the document copy | Type of Copy | Version No. | Revision No. |
| --- | --- | --- | --- | --- |
| 01/10 | Managing Director | Controlled Copy | 01 | 00 |
| 02/10 |  |  |  |  |
| 03/10 |  |  |  |  |
| 04/10 |  |  |  |  |
| 05/10 |  |  |  |  |
| 06/10 |  |  |  |  |
| 07/10 |  |  |  |  |
| 08/10 |  |  |  |  |
| 09/10 |  |  |  |  |
| 10/10 |  |  |  |  |

- DISTRIBUTION LIST

- LIST OF ABBREVIATIONS

| S. No. | Abbreviations | Expansion |
| --- | --- | --- |
| 1 | TDPL | TRUSTlab Diagnostics Private Limited |
| 2 | HR | Human Resources |
| 3 | P.M | Preventive Maintenance |
| 4 | QC | Quality Control |
| 5 | QMS | Quality Management System |
| 6 | LQM | Lab Quality Manual |
| 7 | MD | Managing Director |
| 8 | IA | Internal Audit |
| 9 | QM | Quality Manager |
| 10 | SOP | Standard Operating Procedure |
| 11 | SQC | Statistical Quality Control |
| 12 | REV | Revision |
| 13 | SS | Sample Schedule |
| 14 | QSP | Quality System Procedure |
| 15 | MRM | Management Review Meeting |
| 16 | CA | Corrective Action |
| 17 | PA | Preventive Action |
| 18 | LIS | Lab Information System |
| 19 | AMC | Annual Maintenance Contract |
| 20 | CMC | Comprehensive Maintenance Contract |
| 21 | TAT | Turn Around Time |
| 22 | CPCB | Central Pollution Control Board |
| 23 | CBWTF | Common Bio-medical Waste Treatment and Disposal Facility |
| 24 | BMW | Bio-Medical Waste |

- Introduction

  - We firmly believe that testing of clinical specimens and safety go hand in hand and that a safe working environment leads to improved productivity. Efforts must be placed on safety by every employee at this testing facility.

  - This safety manual is prepared for the protection of every employee. All employees are instructed to get familiarized with the safety rules and practices and abide by them.

  - No rule book can cover every situation. When in doubt about a situation, condition, or procedure, ask your Head of the Department (HOD)/Lab Director. Under no circumstance are you to risk injury in carrying out your work.

  - It will not always be easy to comply with these rules; nothing worthwhile is ever easy. With your help, we can make this company a safer place to work

- Objective

- The Safety Manual is designed to comply with the Standards of the Occupational Safety and Health Administration, and to endeavour to maintain a safe and injury/illness free workplace. A copy of the Safety Manual is available ONLINE in the QMS Modules for use and reference. Compliance with the following Safety Manual and all items contained therein is mandatory for all employees of the company.

- Scope

- This Safety Manual applies to all employees of TRUSTlab Diagnostics, regardless of position within the company. Every employee is expected to comply with the Safety rules and practices.

- Responsibilities

- The Laboratory Safety Committee of TRUSTlab Labs, Central Laboratory comprises the following members:

Mr. Venkata Suman Cherukuri – Chairman & Managing Director

Dr. Abdul Aleem – Laboratory Head

Dr. Sadiya Shahnaz – Consultant Microbiologist

Dr. Sai Varun – Quality Manager

- Authorization for Implementation

- The authorization and responsibility for implementation has been given primarily to designated safety officer. The Safety Policy supports four fundamental means of maximum employee involvement:

Management commitment to safety.

Monthly safety meetings at all departments.

Effective job safety training for all categories of employees.

Safety training is provided to all employees.

The Safety Officers and HODs will meet at least once a month to evaluate all areas of safety and make recommendations to the management.

Employees must inform their HOD immediately of any situation beyond their ability or authority to correct.

HODs shall insist that employees observe and obey every safety rule, regulation, and order as necessary for the safe conduct of the work, and shall take such action as is necessary to obtain compliance.

Only qualified and/or licensed employees may operate any equipment.

Anyone known to be under the influence of alcohol and/or drugs shall not be allowed to work in the laboratory while in that condition.

No one shall knowingly be permitted or required to work while his or her ability or alertness is so impaired by fatigue, illness, or other causes that might necessarily expose the individual or others to injury.

Work shall be planned and supervised to prevent injuries in all work processes, particularly when working with equipment and handling chemicals.

Employees shall immediately report any personal injury or damage to property to their supervisor, no matter how trivial, regardless of the amount of damage and irrespective of cause or fault.

- Responsibilities of the HODs

To set examples of safe practices by their own conduct.

To investigate and correct, or have corrected promptly, unsafe conditions which have come to their attention.

To know, observe and enforce all the general safety rules and such special instructions that are set up for their department.

To thoroughly acquaint each employee with safety instructions and practices.

To take part in work place safety and health program activities and contribute to their success.

To investigate and report all personal injuries and illnesses sustained on the job by the personnel within their area of responsibility.

To investigate and report all property damage accidents.

To welcome and utilize, as far as practical, the safety suggestions which may be made by the employees.

To thoroughly instruct new personnel on safety and carefully observe them at their work.

To see that the employees have	and use personal protective equipment as determined by the safety officer.

To see that all tools and equipment are readily available and remain in safe and proper working conditions.

- Reporting of Injuries

All employees will be held accountable for filling out a “Injury incident record” immediately after an injury occurs, even if medical treatment is not required. (Notice must be made at or near the time of the injury and on the same day of the injury). Employees must report the injury to their HODs. A casual mentioning of the injury will not be sufficient. Employees must let their supervisor know:

      - How they think they hurt themselves?

      - What they were doing at the time of incident?

      - Who they were working with at the time of incident?

      - When and where did it happen?

    - Other pertinent information that will aid in the investigation of the incident. Failure to report an injury immediately (meaning at or near the time of the injury and on the same day of the injury) is a violation of the Safety Policy, and they may result in immediate termination, in accordance with company policy.

- Safety Violation Procedures

Safety violation notice(s) shall be issued to any employee violating the safety rules or regulations by the respective Head of the Department, with information to HR department. The four (4) step corrective action procedure adopted would be as follows:

      - First Occurrence: Verbal Warning, Documented In File

      - Second Occurrence: Written Warning, Documented In File

      - Third Occurrence: Three-Day Suspension, Documented In

      - File Fourth Occurrence: Termination.

    - These four steps will have to be followed at all times. Safety Violations Occurrences will be active for a twelve (12) month period. After twelve (12) months they will be removed from the employee file. Four occurrences in a twelve (12) month period will  be grounds for termination.

- Safety Audits

- Safety audits are conducted once in annually and the findings are discussed during the Safety Committee Meetings.

- Review of Safety Manual

Formal review of the safe work practices will be done on annual basis by Director (Lab Ops.) to reduce hazards. Any changes in the safe practices will be incorporated in the safety manual.

- General And Personal Safety

- Purpose: The purpose of the procedure is to explain the Personal safety for the Employees in the Laboratory.

- Scope: The Scope covers all the Personal safety.

- Responsibility: All Staff of the laboratory.

- Procedure: The need for bio safety procedures is to limit occupational exposure to blood and other potentially infectious materials since any exposure could result in transmission of blood borne pathogens that could lead to disease or death. These pathogens include hepatitis B virus (HBV) and human immuno deficiency virus (HIV), Syphilis, Malaria and many others.

General Safety Procedures

    - Access to the laboratory is restricted to authorized personnel only.

    - Personnel belongings are kept in the lockers provided near the security office.

    - Laboratory staff is instructed to always wear appropriate clean and neat clothing (e.g shirts, shoes) and personal protective equipment (e.g lab coats, gloves) in the laboratory and remove personal protective equipment before leaving the laboratory.

    - Hair should be secured back and off the shoulders in such a way as to prevent shedding into the work area. Hair must be kept away from moving machinery or instruments.

    - The laboratory staff is aware of the location of safety equipment including Fire extinguisher, circuit breakers, eye wash facility, first aid kit, spill handling kit and emergency procedures including operation of the above equipment in the laboratory.

    - Unless contraindicated by the test, plastic pipettes and tubes should be substituted for glass wherever possible.

    - Care must be taken to avoid the formation of aerosols or splashing of materials.

    - All contaminated waste or reusable materials must be appropriately decontaminated before disposal or reuse.

    - Chairs or stools with wheel or other unstable items must not be used as stepladders. Call the laboratory maintenance department for assistance if a stepladder or stepstool is not immediately available in the lab.

    - Exit of fire doors must not be blocked, propped open, bolted or obstructed in any way.

    - No ceiling may be opened by staff without prior written permission from the facility maintenance department.

    - All incidents or accidents must be reported immediately and appropriate action to be taken to prevent further occurrences.

    - All staff working in laboratories must be adequately trained, both in the duties they perform and in all safety aspects of laboratory work.

Eye And Face Wash / Whole Body Shower

The best treatment for chemical / Biohazard splashes in the eye and face is immediate flushing with copious amount of water for at least 15 minutes.

Standalone eye washes are in the lab areas for each access.

Plumbed Eye and face washes are equipped with a stay open ball valve.

All plumbed eye and face washes should be flushed by laboratory occupants on a weekly basis by allowing the water to flow for 3 minutes, to remove stagnant water from pipes.

Personnel Safety

Do not work alone in the laboratory.

Observe universal safety precautions.

Observe strict personal hygiene.

All specimens may be potentially positive for infectious agents including HIV and the hepatitis B and C viruses and other pathogens.

Wash your hands with soap at the start of the laboratory session before performing any procedures and before leaving the laboratory at the end of the session.

It is mandatory to wear gloves and lab coat when handling all human blood specimens.

Eating, drinking and smoking in the laboratory are strictly prohibited.

Become familiar with the location and the use of standard safety features in the laboratory.

The laboratory is equipped with fire extinguishers, eye washes, safety showers, bio- safety cabinets, first-aid and spill kits.

Special care for eye protection is required. Safety glasses must be used when certain procedures are being carried out.

The use of contact lenses is not recommended, since they reduce the rate of self- cleansing of the eye.

At the beginning and termination of each laboratory session, bench tops are to be wiped with 1% sodium hypochlorite solution.

Aprons/Clothing

Laboratory coat provided by the laboratory are to be worn during the working hours in the testing areas.

Due to the possible absorption and accumulation of chemicals in the material, lab coats should not be worn in the lunchroom or elsewhere outside the laboratory.

The lab coats must be washed once a week with detergents and the stores in-charge is responsible for the same. If spoiled with blood or body fluids, it must be soaked in bleach (1:10) for 4 hours and then washed with detergent.

Washing of clothing is provided within the laboratory.

Alternate arrangements are made by the laboratory in case of contamination during working.

Aprons are provided for all housekeeping staff and are used while carrying out their activities.

Footwear

    - Bare foot/Half cover/Slippers are not encouraged because of the danger of spillage of corrosive of irritating chemicals. Full covered shoes are allowed in laboratory in working hours.

Gloves

Latex disposable gloves (powder free) are used by the technical staff/housekeeping staff when handling samples, reagents, washing and disposing waste.

Before each use, gloves are inspected for discolorations, punctures, and tear.

Gloves may be checked by inflating with air.

Employee shall remove gloves before handling telephones, microscopes, door handles etc.,

Double gloves must be worn while handling specimens of known HIV/HBV/HCV patients.

In case any employee of the department develops allergy or skin irritation or infection due to use of latex gloves – the same has to be informed to the HOD for taking necessary corrective actions. Such incidents are recorded in the Incident and Accident Record.

Protection against Latex Allergy:

- Take the following steps to protect yourself from latex exposure and allergy in the workplace:

      - Use non-latex gloves for activities that are not likely to involve contact with infectious materials (food preparation, routine housekeeping, general maintenance, etc.).

      - Appropriate barrier protection is necessary when handling infectious materials. If you choose latex gloves, use powder-free gloves with reduced protein content.

      - Such gloves reduce exposures to latex protein and thus reduce the risk of latex allergy.

      - So-called hypoallergenic latex gloves do not reduce the risk of latex allergy. However, they may reduce reactions to chemical additives in the latex (allergic contact dermatitis).

      - Use appropriate work practices to reduce the chance of reactions to latex.

      - When wearing latex gloves, do not use oil-based hand creams or lotions (which can cause glove deterioration).

      - After removing latex gloves, wash hands with a mild soap and dry thoroughly.

      - Practice good housekeeping: frequently clean areas and equipment contaminated with latex-containing dust.

      - Take advantage of all latex allergy education and training provided by your employer and become familiar with procedures for preventing latex allergy.

      - Learn to recognize the symptoms of latex allergy: skin rash; hives; flushing; itching; nasal, eye, or sinus symptoms; asthma; and (rarely) shock.

Face Mask

Face masks are used when and where necessary by the Doctors/technicians/others.

All personnel working in the Microbiology and Molecular biology departments are instructed to use face mask having filter respirator of N-95 grade.

Bio-Safety Requirements

All staff working at the bio-safety cabinets should wear gloves, face masks, goggles and head gear.

Do not bring potentially contaminated materials out of the cabinet until they have been surface decontaminated.

Work at least 4 inches away from the inside edge of the front vent.

UV light should be in “OFF” position while working at the Bio-safety cabinets.

Eating, Drinking & Cosmetics

No Eateries, beverages are allowed to be stored or consumed inside the laboratory.

Applying heavy cosmetics/Lip gloss/Bangles is not allowed.

Pipetting

Mouth Pipetting is not allowed.

The pipetting is done only using micro pipettes.

Broken Glass

Broken glass and glass which could potentially break and become a cut or puncture hazard must be carefully packaged before being disposed.

The broken glassware is collected in white jar provided at specified locations in the laboratory. The containers are clearly labeled as “SHARP DISPOSAL CONTAINER”.

The broken glass should never be picked up with bare hands. It must be picked up using brush and dustpan, tongs, or forceps.

Hazard Warning

“Bio-Hazard” symbol is pasted on the laboratory door exterior and other places like equipment, BMW Dustbins etc.

Disinfectant

A 70% (v/v) aqueous solution of ethanol (STERILIUM) or Dettol/available disinfectant is used on the skin as a hand washing method. The contact time on skin should be 30 Sec.

First Aid Kit

“First Aid Kit” is placed at the entrance of the laboratory floor and shall be used by the technicians in case of any cuts etc., in consultation with the HOD.

Universal Safety Precautions

Purpose: The purpose of the procedure is to explain the Universal safety for the Employees in the laboratory.

Scope: The Scope covers all the universal safety precautions.

Responsibility: All Staff of the laboratory.

Universal Safety

Wash hands before and after handling patients or specimen collection.

Handle the blood and body fluids of all patients as potentially infectious.

Wear gloves for potential contact with blood and body fluids.

Place used needles after	destroying immediately in nearby impermeable container (sharps disposal container).

Do not recap or manipulate needle in any way.

Wear protective eye wear and mask if splatter with blood or body fluids is possible.

Wear gowns when splash with blood or body fluids is anticipated.

Handle all	linen soiled with blood and/or body	secretions as potentially infectious.

Process all laboratory specimens as potentially infectious.

Wear mask for open tuberculosis cases respiratory and air-borne infections.

Mouth pipetting is prohibited.

- Universal precautions are to protect PATIENTS and STAFF from the spread of blood borne viruses (HIV/Hepatitis) or other harmful microorganisms that may be present in blood or body fluids.

Universal precautions apply to blood, human secretions, body fluids containing visible blood, semen, vaginal fluid, cerebrospinal, synovial, pleural, peritoneal, pericardial, and amniotic fluid needles, scalpels and other sharp instruments.

Consider all blood and certain body fluids of all patients as POTENTIALLY INFECTIOUS for HIV, HBV and other blood borne pathogens.

Infectious material must be handled in	a manner that minimizes splashing, spraying, splattering, and generation of droplets.

Wear Personal Protective Equipment

Gloves are to be worn when hand exposure to blood and/or OPIM (other potentially infectious material) is anticipated, such procedures include phlebotomy, specimen collection, open wound contact and when handling or touching contaminated items or surfaces.

Band-Aid Cover any existing cuts or lesions with a waterproof dressing/plaster before wearing gloves.

Gowns and disposable plastic aprons are required during procedures when splashing with blood and/or OPIM is anticipated.

- Masks,  face shields, ventilation devices and protective eye wear are required during procedures when splashing, spraying, splatter or droplets of blood and OPIM to the eyes, nose or mouth is anticipated.

- TB respirator masks are required for protection against tuberculosis. Regular masks are required for protection against other airborne transmitted diseases such as chicken-pox.

pox.

- Use good hand hygiene. It is required before and after contact with patients and specimens, wearing gloves or other PPE, contact with mucous membranes. Hand hygiene may be accomplished with either waterless disinfectant or soap and water washing.

- Always dispose of sharps at the point of use in puncture proof containers. Do not dispose of sharps with other clinical waste.

- Do not recap or manipulate. Needles should not be recapped or manipulated in any way. Don't break, or bend needles. Don't reach your hand into a container that might contain sharps.

- Disinfect blood/body fluid SPILLS correctly. Dispose of waste and excreta carefully.

- Disinfect linen heavily contaminated with blood/body fluids by soaking in 5% Sodium hypochlorite solution before sending to laundry.

- Flush eyes, nose, or mouth with water as soon as possible after contact with blood or potentially infectious materials.

- Don't eat, drink, smoke, apply cosmetics, or handle contact lenses in areas that could contain infectious materials.

- Hepatitis B vaccination is strongly recommended for all employees who have the potential for occupational exposure to blood and OPIM. This is administered in a series of three injections followed by Anti HBsAg Antibody titre to confirm the response to vaccination.

Visitors Safety

Purpose:  The purpose of the procedure is to explain the restricted visitors to Employees in the laboratory.

Responsibility: All Staff of the laboratory.

Permitted Visitors:

      - Directors and statutory auditors of the organization.

      - Facility Head to address any engineering and maintenance related issues.

      - Floor manager for housekeeping activities.

      - Service engineers who come for trouble shooting, equipment related, examination related problem, preventive maintenance.

      - Quality consultant for audit and training purpose.

      - Auditiors form accreditation bodies during the assessment.

      - Housekeeping staff.

Procedure:

      - All visitors should be briefed about the safety aspects and precautions to be taken while entering the laboratory areas.

      - Provide aprons and PPEs during their visit to the laboratory areas.

      - Avoid visiting the bio-hazard areas.

      - HODs or persons designated shall accompany the visitors at all times.

- Pre-Exposure Prophylaxis

Purpose: The purpose of the procedure is to explain the Pre exposure Prophylaxis for staff for the Employees in the laboratory.

Responsibility: All Staff of the laboratory.

Procedure: TRUSTlab Labs recruits the staff with the documented evidence of complete vaccination against Hepatitis B virus or whose Anti HBsAg Antibody titre is in the protective region of >10 IU. The copy of documented evidence is filled in the Human Resource Department (HRD).

Applies to all employees of the laboratory.

All the staff who are not vaccinated against Hepatitis B vaccination - will be vaccinated with complete 3 doses (0, 1 and 6 months) of vaccine by the management after initial screening.

In case if the staff is already vaccinated – they are screened for Anti HBS Antibody Titre and if the titre is <10 IU the staff are given a booster course of 3 doses.

If the titre value is >10 IU the staff will be screened for Anti HBs Antibody after 3 years.

TB Exposure Plan

Procedure:

- The plan incorporates exposure determination at defined intervals viz., once a year for all the employees who may have occupational exposure to tuberculosis. All employee’s working in microbiology and molecular biology departments are instructed to wear face mask having filter respirator of N-95 grade.

- Post Exposure Prophylaxis

Purpose:

This policy describes the	procedure for	post exposure prophylaxis followed	in TRUSTlab Labs.

Scope: This is applicable to TRUSTlab Employees.

Responsibility: Safety Officer

Procedure:

    - DEFINITION OF EXPOSURE

- “Exposure” to blood, tissue, or other body fluids like semen, vaginal secretions, cerebrospinal, pleural, peritoneal, pericardial, synovial, and amniotic fluids have a potential risk of transmission of blood borne pathogens to healthcare workers and therefore post- exposure prophylaxis should be considered If there is:

      - A percutaneous injury (for example, a needle stick or cut with a sharp object).

      - Contact with mucous membrane or non-intact skin (for example, skin chapped or abraded or dermatitis).

      - Prolonged contact with intact skin or contact that involves extensive areas of skin.

    - Factors Influencing Risk of Transmission:

      - Depth of the injury (in case of a sharp object).

      - Device was visibly contaminated with blood.

      - Needle that was directly inside an artery or vein.

      - Type of needle: a hollow bore or a solid needle.

      - The thickness of the needle.

      - Viral load of the source.

      - Quantity of blood or body fluid exposed.

      - Duration of exposure.

    - General measures to be taken after an exposure:

- The following general guidelines are recommended as soon as potential exposure to blood or contaminated body fluids occurs:

      - Treatment to the exposed site

      - Allow site to bleed.

      - Cuts to be washed with plenty of soap and running water

      - Splashes into nose, mouth, skin to be flushed with water

      - Mucous membrane like eyes/mouth to be irrigated with clean water or saline for 5 minutes

      - Pricked finger should not be put into mouth (reflexly)

      - Do not squeeze blood from wound, this causes trauma and inflammation –increases the risk of transmission.

      - Do not use bleach, alcohol, betadine or iodine, which may be caustic and cause trauma.

      - Report immediately to the supervisor.

    - Documentation of the Data:

      - Maintain proper records of such incidents.

      - Trained medical personnel should document the data in the incident / Accident Register

      - Name and data of the source.

      - Time and date of exposure.

      - Nature of exposure (that is, non-intact skin, mucosal, or percutaneous exposure).

      - Body site exposed and contacted time.

      - Infective status of the source if documented.

      - For percutaneous injuries, a description of the injury (depth of wound, solid versus hollow needle, sharp, etc).

      - Circumstances under which the exposure incident occurred.

      - Previous testing and immune status of the exposed healthcare worker.

    - Testing of Source:

      - Testing of the source for HIV, HBsAg and HCV should be done as early as possible after counselling (rapid testing if available) if infective status is not known already.

    - Post Exposure Prophylaxis (PEP) For HIV:

- PEP is recommended for the following conditions:

      - When the exposure source is HIV Reactive

      - When source patient is at high risk of HIV

      - When the status of source patient is unknown

      - Exposure of percutaneous, mucous membrane, non- intact skin by infectious source material

      - National guidelines available

      - PEP is not indicated in these circumstances:

      - Low risk exposures e.g. exposure on intact skin

      - Source patient is HIV negative (no high risk behaviour)

      - Indications determined by case to case

      - COUNSELING:

        - The Health care worker should be given pretest counseling

        - Reference blood sample to be taken immediately after exposure and tested for HIV antibodies.

        - Blood testing done on 1-1/2 months, 3 months and 6 months

        - Individuals with high- risk exposures may be advised for the next 6 months to Avoid sharing of body fluids.

        - Stop breast feeding.

        - Avoid pregnancy.

        - Avoid organ/blood donation.

      - POST EXPOSURE PROPHYLAXIS

        - Detailed advice from treating Clinician should be followed.

        - PEP to be stopped before 8 weeks in case patient is found HIV negative.

      - POST EXPOSURE PROPHYLAXIS FOR HEPATITIS B:

        - Indications: percutaneous or permucosal contact protocols below intended for healthcare worker exposure.

| Vaccination And Antibody Response status of Exposed Persons | Source is HBsAg Positive | Source is HBsAg Negative |  | Source is not tested or Status Unknown |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | High Risk | Low Risk |
| Unvaccinated | HBIG (0.06ml/Kg) – 1 Dose & Initiate Vaccine | Initiate Hepatitis Vaccine Series |  | Begin Hepatitis Vaccine Series | Begin Hepatitis Vaccine Series |
| Previously Vaccinated |  |  |  |  |  |
| Known Responder (Anti HBs> 10mIU/mL) | No Treatment | No Treatment |  | No Treatment | No Treatment |
| Known Non Responder |  |  |  |  |  |
| Not Revaccinated | HBIG (0.06ml/Kg) – 1 Dose & Initiate Revaccination Series |  | Begin Revaccinatio Series | HBIG (1 Dose) And Begin Revaccination Seri |  |
| After Revaccination | HBIG (2 Doses) |  | No Treatment | HBIG (2 Doses) | No Treatment |
| Antibody Response Unknown | Test For Anti-HBs If Adequate, Treatment If Inadequate, HBIG And Vaccine Booster |  | No Treatment | Test For Anti-Hbs If Adequate, No Treatment If Inadequate, Give Vaccine Booster A Check Anti-HBs in 1–2 Months |  |

      - POST EXPOSURE PROPHYLAXIS TO HEPATITIS C:

        - At present, there is no proved effective post-exposure prophylaxis for healthcare workers exposed to HCV.

        - The following are recommendations for follow up of occupational HCV exposures: For the source perform testing for Anti-HCV

        - For the person exposed to a HCV positive source perform baseline testing for Anti-HCV and alanine aminotransferase activity and follow up testing (for example, at 4–6 months) for anti-HCV and alanine aminotransferase activity (if earlier diagnosis of HCV infection is desired, testing for HCV RNA may be performed at 4–6 weeks).

- Fire Safety

Purpose: Purpose of this procedure is to explain the various activities performed by the management to uphold the occupational health and safety requirements.

Scope: This procedure covers the requirements such as protective measures, firefighting equipment’s.

Responsibility: Safety Officer (Designated) is the responsible person for making decisions regarding Occupational health and safety.

In case of Laboratory emergency fire code – “CODE RED” should be paged.

Output:

      - Controlled working environment.

      - Individual responsibility for designations.

      - Improved safety conditions.

Monitoring and Measurement:

      - Number of accidents happened.

      - Major causes for accidents.

      - Awareness and effectiveness of health and safety measures.

      - Effectiveness of the safety equipments.

      - Employee awareness about Health and safety.

Fire Detection/Alarm:

      - Automatic fire detection and alarm system is installed in the facility.

      - Fire and smoke detectors are located in each room.

      - In case of fire – all access control doors open automatically.

      - Fire exits doors are provided at each floor.

      - In case of fire all personnel must escape and reach the common assembly point in the front side of the reception block on the 40 ft wide road.

Fire Alarm Station:

In case of major fire – the nearest Fire Station shall be contacted – the address along with the contact information should be displayed at the reception desk at the TRUSTlab Central Laboratory and its branches.

Procedure:

      - Dial 9999 on the telephone.

      - Say code red – Department - Ground Floor/……1st floor/…….2nd floor thrice

      - In case of minor fire handle/manage with fire extinguisher available in the departmental floor.

Fire Extinguisher:

    - Fire extinguishers have been strategically placed in the corridors just outside the departments depending on the hazard. The list of fire extinguishers located at each of the TRUSTlab facilities needs to be listed and displayed at the Reception desk.

    - All the laboratory staff are trained in fire safety (fire drills) and use of extinguishers once a year. The training records are maintained by the HR department.

    - In case of fire remember “RACE”

R – Rescue any person caught in fire.

A – Alarm should be raised (Code Red – Department/Room).

C – Contain the fire and evacuate.

E – Extinguisher must be used.

    - To use an Extinguisher, remember “PASS”:

P- Check pressure (needle pointing on green) pull the pin.

A- Aim the nozzle to the base of the fire.

S- Squeeze the handle to release the contents.

S- Sweep the cylinder sideways to cover the base of fire completely.

Electric Power:

    - Power sockets, cables, lamp are verified for good working condition. Access to the power control room is restricted to qualified employees.

    - Cabinets covering the electrical switch gear, fuse boxes, are made out of flame proof material. Condition of the electrical implements and switch boards are periodically verified, and the findings are documented in Inspection of safety & Electrical facility.

Do’s & Don’ts:

      - Keep all inflammable items away from burner area.

      - Keep electrical appliances and electrical points away from gas cylinder and inflammable material as far as possible.

      - Avoid using extension wires and multiple plugs.

      - Sprit lamp should be checked for leakage before lighting them.

      - All equipment should be grounded and at least once a year grounding is checked.

      - Check all the connection before turning on the main switch of machine. Switch off gas burner as soon as you smell leakage.

      - Switch “OFF” main power in case of fire. Do not pull the cord.

      - Inform gas leakage or fire accident to maintenance	department/safety department. Do not use lift in case of fire.

      - Do not clutter or block exit passage. Do not leave the gas burner unattended.

      - Do not operate any electrical switches when there is gas leakage. Do not overload electrical points.

      - Never have liquid or wet material next to electrical switches.

      - Enter the details in incident and accident register and inform to the safety head.

Fire & Evacuation Drill

    - Evacuation drill is developed by the Management representative and the illustrated evacuation plan in local language is posted in all production and office areas. Mock drill is conducted with all the employees once in a year. New employees are instructed before they start working.

Fire drill is conducted to create awareness to all the employees by training them in firefighting strategies, handling of firefighting equipments, fire characteristics of substance in use and the required firefighting method.

All of employees who are working in areas with fire risks are trained in Evacuation drills and the details are documented in employee training register.

Information And Training

New employees are given detailed training in the actual working environment and proper use and handling of machinery and chemicals.

Regular training sessions are conducted on accident and fire preventions.

Fire drills are conducted once in a year to check the adequacy of the fire alarm system and also to access the response of employees in the event of fire.

- Chemical Safety:

- Purpose: The purpose of the procedure is to explain the Chemical safety for the Employees in the Laboratory.

- Scope: The Scope covers all the Chemical safety.

Responsibility: All Staff of the laboratory.

Hazard Minimization:

- Using smaller quantities of hazardous chemicals or substituting a less hazardous chemical reduces the risk of serious exposure or spill, for additional waste reduction information. When planning your work, consider the following possibilities:

      - Substituting with less hazardous chemicals.

      - Ordering only what is required.

      - Sharing chemicals wherever possible.

Work Practices:

- Use the safety equipment and personal protective equipment (PPE).

Know the Hazard:

      - Review the hazards of the chemicals before using them. Review the Material Safety Data Sheet (MSDS) for safe handling procedures and PPE recommendations.

      - Be prepared for a spill or an exposure involving the hazardous chemical. Know the location of the nearest eyewash and emergency shower.

Keep Halls Clear:

- Keep hallways outside the lab and within the lab clear of furniture and equipment. Make sure access to the fire extinguisher and eyewash are unobstructed.

Plan Carefully and Anticipate New Hazards:

- When a new parameter / new methodology is introduced, Quality Manager formally analyzes the procedures for possible hazards and consider the consequences. Director (Lab Ops) and HODs are asked to review the hazard analysis.

Do Not Work Alone:

- Do not work alone if work requires the use of hazardous materials or hazardous procedures. At a minimum, a second person should be aware of an individual working alone in the lab and arrangements should be made for periodic checks. Excessively long work hours increase the likelihood of mistakes and accidents due to fatigue

Report Spills to HOD:

- Report all spills, accidents and injuries to HODs and record in the Accident/Incident Record. Follow good housekeeping practices. Maintain work areas in an orderly fashion. Avoid accumulation of combustible materials. Cluttered areas increase the likelihood of accidents and injuries.

Do Not Rely on Odour as an Indicator of Exposure:

- The absence of odour is not a reliable guide to a safe concentration of airborne chemical in the lab. Concentrations detectable by odour vary according to the chemical and the ability of the individual to smell the chemical. Never rely on odour to determine exposure hazard.

Food and Drink Policy:

- Never smell or taste chemicals to identify them. Wash your hands immediately after using any hazardous material and before leaving the lab. Never pipette or siphon liquids by mouth.

- Do not eat, drink, or apply cosmetics in the lab. Do not store food or drinks in a lab refrigerator or in a cold room.

Chemical Transport Disposal and Storage:

    - Transporting Chemicals within the Clinical Laboratory:

- Use safety carriers or secondary containers to transport dangerous chemicals (e.g., strong corrosives, solvents), even for short distances. Transport the chemical bottles using plastic tubs and use the lift for transit across the floors. Do not carry hazardous chemicals in stairwells.

    - Chemical Disposal:

- Do not mix hazardous wastes. Waste streams must be segregated for safe and cost-effective disposal.

    - Chemical Storage Plan

      - Chemicals are stored according to hazard class (i.e.,flammables, oxidizers, toxics, corrosives, etc.)

      - Incompatible chemicals are physically separated from each other during storage.

      - All chemicals are stored away from direct sunlight or localized heat.

      - Containers of corrosive chemicals (acids/bases) are kept in catch trays large enough to contain any spill or leakage and the trays are stored in chemical-resistant anti-corrosive cabinets.

      - All chemical containers are labeled.

      - Store hazardous chemicals at a safe reachable height for all workers in the laboratory.

      - All shelves in the cabinets are made of stainless steel of grade SS304.

      - Shelves are secured and strong enough to hold chemicals stored on them. Overloading of shelves is avoided.

      - Separate solids from liquids.

      - Acids: Store concentrated acids on lower shelves in chemical-resistant catch trays kept in a anti-corrosive cabinet. This will temporarily contain spills or leaks. Separate acids from bases and active metals such as sodium, magnesium, and potassium. Acids should be separated from chemicals which can generate toxic gases when combined (i.e., sodium).

      - Bases: Store bases away from acids. Store concentrated bases on lower shelves in chemical-resistant catch trays kept in a anti-corrosive cabinet. This will temporarily contain spills or leaks.

      - Flammables: Approved flammable storage cabinets are used to store flammable liquids. Storage of flammable and corrosive chemicals is kept in the common storage cabinets installed at the entrance of the 1st Floor lab away from the testing areas. Required quantities are issued by the stores in-charge and the same is transported by wheel cart up to ground floor lift and chemicals are transported by lift to the first floor and second floor. Small quantities (<1gallon) of flammable and corrosive chemicals are available on the working desks for daily usage.

      - Water-Reactive Chemicals: Water-reactive chemicals are stored in a cool, dry place. Do not store water-reactive chemicals under sinks or near water baths.

      - Toxics: Toxic compounds are stored according to the nature of the chemical, with appropriate security employed when necessary.

    - Hazard Chemicals Maintained in the Laboratory:

- Chemicals listed in the table below are used for processing of patient samples. The maximum storage quantities in the central stores and the departments are listed against each chemical. The chemicals should be stored in storage cabinets and should be located at ONE common point in the facility – from where the chemicals are taken by the other departments.

| S No |  | Name of Corrosive Materials |  | Storage |  |  | Disposal Procedure |  | Health Effects | First Aid Measures |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 70% ISOPROPYL ALCOHOL CONCENTRATED ALCOHOL METHANOL AMMONIA |  | Keep away from heat, spark, and open flame, sources of ignition. Keep in tightly closed containers in a well ventilated area. |  |  | Eliminate all sources of ignition and take measure to prevent run-off into drains and waterways. Wash the cleaned up area with plenty of water. |  | Contact: with skin & eyes causes irritation. Inhalation: High concentrations may cause central nervous system effects characterized by nausea, headache, dizziness etc. Ingestion: May cause gastrointestinal irritation with nausea, vomiting, and diarrhoea. | Contact: Immediately flush eyes with plenty of water; Flush skin with plenty of soap and water. Inhalation: Remove from exposure and move to fresh air immediately. Ingestion: Give 2-4 cups full of milk or water. Get medical aid. |
| 2 |  | MERCURIC IODIDE |  | Store in light resistant containers. Keep container tightly closed. Keep container in a cool, well-ventilated area. Do not store above 24°C (75.2°F). |  |  | Waste must be disposed of in accordance with federal, state and local environmental control regulations. |  | Severe over-exposure can result in death. Inflammation of the eye is characterized by redness, watering, and itching. Skin inflammation is characterized by itching, scaling, reddening, or, occasionally, blistering. | Eye & skin Contact: In case of contact, immediately flush eyes with plenty of water for at least 15 minutes. Cover the irritated skin with an emollient. Wash clothing before reuse. Get medical attention immediately. Inhalation: If breathing is difficult, give oxygen. Get medical attention. Ingestion: Loosen tight clothing such as a collar, tie, belt, or waistband. Get medical attention immediately. |
|  | 3 |  | SULFURIC ACID |  | Reacts violently with water. Keep container tightly closed. Keep container in a cool, well-ventilated area. Do not store above 23°C (73.4°F). | Sulfuric acid may be placed in sealed container or absorbed in vermiculite, dry sand, earth, or a similar material. It may also be diluted and neutralized. Be sure to consult with local or regional authorities (waste regulators) prior to any disposal. |  | Liquid or spray mist may produce tissue damage. Skin contact may produce burns. Inhalation of the spray mist may produce severe irritation of respiratory tract. |  | Eye Contact: Check for and remove any contact lenses. Immediately flush eyes with running water for at least 15 minutes. Do not use an eye ointment. Seek medical attention. Skin Contact: Wash with a disinfectant soap and cover the contaminated skin with an anti-bacterial cream. Inhalation: Allow the victim to rest in a well ventilated area. Seek immediate medical attention. |
|  | 4 |  | HYDRAZINE SULPHATE |  | Keep container dry. Keep in a cool place. Ground all equipment containing material. Corrosive materials should be stored in a separate safety storage cabinet or room. | Waste disposal should be in accordance with existing federal, state and local environmental regulations. |  | Severe over-exposure can produce lung damage, choking, unconsciousness, or death. Inflammation of the eye is characterized by redness, watering, and itching. Skin inflammation is characterized by itching, scaling, reddening, or, occasionally, blistering. |  | Gently and thoroughly wash the contaminated skin & eyes with running water and non- abrasive soap. Seek immediate medical attention. |

| 5 | ACETIC ACID | Store in cool. Dry well ventilated area away from alkaline substances. | Remove from all sources of ignition. Use water to dilute spill to non- flammable mixture. Absorb with inert material (dry sand) and place in chemical waste container. | Contact: liquid or spray mist may produce tissue damage on skin and mucus membrane of eyes. Inhalation: may produce severe tissue damage in respiratory tract Ingestion: may cause severe gastrointestinal irritation nausea, vomiting and diarrhea | Contact: wash with disinfectant soap and cover the contaminated skin with an antibacterial cream Inhalation: move to fresh air and visit emergency dept immediately Ingestion: do not induce vomiting. Rinse mouth followed by drinking a large amount of water |
| --- | --- | --- | --- | --- | --- |
| 6 | FORMALDEHYDE | Store in labeled, chemically compatible containers, away from heat and flame | Dilute with water and mop up | Contact with eye: may cause redness, watering and itching Inhalation: exposure to high concentration of vapors may result in stupor Ingestion: may cause severe gastrointestinal irritation nausea, vomiting and diarrhea | Contact with eye: Check for and remove any contact lenses. Immediately eye, skin with plenty of water Inhalation: move to fresh air and visit emergency dept immediately Ingestion: if swallowed do not induce vomiting unless directed to do so by medical personnel |
| 7 | ACETONE | Store in a cool, well ventilated location, away from any area where the fire hazards may be acute. Keep away from incompatibles |  | Slight eye, nose and throat irritant. Maybe harmful by inhalation, ingestion or through skin contact. Inhalation may cause dizziness, narcosis and coma | Contact with eye: Check for and remove any contact lenses. Immediately eye, skin with plenty of water Inhalation: move to fresh air and visit emergency dept immediately Ingestion: if swallowed do not induce vomiting unless directed to do so by medical personnel |
| S No | Name of Corrosive Materials | Storage | Disposal Procedure | Health Effects | First Aid Measures |
| 8 | HYDROCHLORIC ACID | Keep away from | Small amounts of | Hazardous and acts as an | Contact: immediately |
|  | NITRIC ACID | incompatibles such | dilute Hcl can be | irritant in case of skin | flush with plenty of |
|  | ACID ALCOHOL | as oxidizing agents, | flushed down a sink | contact, eye contact or | water and visit |
|  |  | organic materials, | with large quantity of | ingestion. Slightly | emergency dept |
|  |  | metals, alkalis, | water | hazardous in case of | Inhalation: if not |
|  |  | moisture |  | inhalation | breathing, give |
|  |  | May corrode |  |  | artificial respiration. |
|  |  | metallic surfaces. |  |  | If breathing is |
|  |  | Store in a metallic or |  |  | difficult, give oxygen. |
|  |  | coated fiber board |  |  | Visit emergency dept |
|  |  | using a strong |  |  | immediately. |
|  |  | polyethylene inner |  |  | Ingestion: if |
|  |  | package |  |  | swallowed do not |
|  |  |  |  |  | induce vomiting |
|  |  |  |  |  | unless directed to do |
|  |  |  |  |  | so by medical |
|  |  |  |  |  | personnel |
| 9. | SODIUM HYPO-CHLORITE | Keep in a tightly | Dilute sodium hypo- | Contact: Skin contact | .First Aid: Flush skin |
|  |  | closed container | chlorite should be | may produce burns | with plenty of water. |
|  |  | Stored in a cool, dry, | flushed out with | Inhalation: Inhalation of | If required, visit the |
|  |  | ventilated area. | plenty of water | the spray mist may | Emergency dept |
|  |  | Protect against |  | produce severe irritation | First Aid: Remove to |
|  |  | physical damage |  | of respiratory tract | fresh air. If breathing |
|  |  |  |  |  | difficulty persists, |
|  |  |  |  |  | visit the Emergency |
|  |  |  |  |  | dept. |
| *** Being Corrosive, please use the above chemicals in the presence of any one of the senior staff members |  |  |  |  |  |

Wherever histopathology and cyto-pathology specimens are processed - Formaldehyde and Xylene levels are monitored through an outside agency to estimate the exposure levels of the personnel working in the histopathology and cyto- pathology laboratory and the results are documented. The Formaldehyde and Xylene levels are monitored once a year in the Tissue cutting, Grossing, Tissue Processing, Cyto-pathology lab, Specimen storage and Slide and Block Storage rooms.

|  | 8 Hr Time-Weighted Exposure Limit | Action Level (8 Hr Time- Weighted Exposure) | 15 min Short Term Average Exposure Limit (STEL) |
| --- | --- | --- | --- |
| Formaldehyde | 0.75 | 0.5 | 2.0 |
| Xylene | 100 | -- | 150 |

Ventilation: Lab Areas viz., histopathology processing, grossing, tissue processing, slide and block storage and the specimen storage areas should be provided with exhaust system with input of fresh air.

- Chemical Hygiene Plan

Purpose: The purpose of the Chemical Hygiene Plan (CHP) is to minimize the exposure of hazards chemicals to personnel working in the clinical laboratory of TRUSTlab Labs.

Scope: The Chemical Hygiene Plan outlines the minimum health and safety requirements for those operations that are engaged in the "laboratory use of hazardous chemicals."

Policy: TRUSTlab has the policy for safe procurement, handling and disposal for chemical operations.

Responsibilities:

      - Director (Lab Ops)/Consultant Pathologist:

        - Ensure the compliance of CHP.

        - Implement appropriate chemical hygiene policies and practices.

        - Overall responsibility for chemical hygiene the clinical laboratory.

      - Chemical Hygiene Officer:

        - Ensure all MSDS are available for all chemicals

        - Ensure that Laboratory personnel follow the chemical hygiene rules

        - Provide adequate training for all Laboratory personnel regarding safety, chemical hygiene

Chemical hazards

    - Determine the appropriate protective equipment required; ensure that it is available; and

    - Ensure regular, formal, chemical hygiene and housekeeping inspections.

    - Basic/General Laboratory Safety Rules:

      - Know the location of emergency showers, eyewashes, first aid kits, emergency exits, telephone and fire alarm manual call point.

      - Know the location of Material Safety Data Sheets.

      - Ensure that Laboratory person trained on the Chemical Hygiene Plan.

      - No food, beverage, smokeless tobacco, or application of cosmetics is allowed in the laboratory.

      - No smoking is allowed in any area of a laboratory.

      - Always wash hands after working with chemicals, even when gloves have been used.

      - Dress appropriately. Long hair, neckties, or loose clothing should be tied back or otherwise secured.

      - No sandals, open-toed or perforated shoes are allowed to be worn in the laboratory.

      - Lab coats will be worn over short skirts (above the knee), shorts, or exposed midriffs.

      - Appropriate eye protection (safety glasses at a minimum) will be worn.

      - Wear the appropriate personal protective equipment for the chemicals.

      - When wearing contact lenses, be sure to wear safety goggle to protect your sight.

      - Laboratory area with	special or unusual hazards shall be posted with appropriate warning signs.

      - Do not directly smell or taste any chemical.

      - Do not pipette or siphon by mouth.

      - Containers shall be kept closed when not in active use.

      - When diluting strong acids, it is generally safer to add acid to water as water has a higher heat capacity and can absorb the heat generated by the dilution reaction.

      - Working with hazardous chemicals alone, especially after regular hours in a laboratory is not permitted.

LABELING:

- Labeling of chemicals / solvent is done as per relevant departmental manual. The label has to mention the type of chemical with signage as warning.

CHEMICAL PROCUREMENT AND DISTRIBUTION:

      - Ensure that a current MSDS is available or requested from the manufacturer for each chemical purchased or from other sources.

      - Substitute with less hazardous chemicals where possible.

      - Ensure  that  equipment  such  as  bottle  carriers,  carts  with  spill  containment, or specialized containers are available for use if needed.

      - Limit purchases to amounts that can be used in the next year or less.

MAINTENANCE:

      - Laboratories should be kept free of clutter.

      - Working areas should be cleaned up at the end of each operation and at the end of each day.

      - Small spills of chemicals should be cleaned up immediately.

      - The floor shall be kept clean and free of slip hazards by reasonable cleaning and immediate cleanup of spills.

      - Safety	showers,  eyewashes,	and fire extinguishers	shall be free from any obstruction that would prevent access and use.

      - Access to emergency exits shall be kept clear at all times.

PERSONAL PROTECTIVE EQUIPMENT (PPE):

      - Personnel must use proper personal protective devices when handling corrosive, flammable, bio-hazardous, and carcinogenic substances.

      - Face protection (goggles, mask, face shield, etc) is used when the possibility of splashes to the face exist.

      - Protective lab coats are worn while in the laboratory. The lab coats must be removed when leaving the lab and should be laundered in-house and never taken home by personnel.

      - All protective wear such as disposable gloves, surgical scrubs, aprons, and lab coats must be removed before leaving the technical work area.

      - Open-toed shoes, sandals, shorts and other apparel that leave skin exposed are not appropriate when working with hazardous chemicals or infectious agents.

      - All personnel are instructed in the proper use and care of disposable gloves.

      - Gloves must be worn when hands may contact infectious agents or hazardous chemicals.

      - Wash hands after removing gloves and before leaving the work area.

EYE AND FACE PROTECTION:

      - Eye and face protection shall be required where there is a reasonable probability of injury that can be prevented by such equipment.

      - Appropriate eye protection shall be worn at all times in laboratories. At a minimum, safety glasses with side shields shall be worn.

SKIN PROTECTION:

      - When working with those substances that are readily absorbed by the skin or with substances that are highly corrosive to the skin, appropriate protection shall be provided to protect the laboratory personnel from skin contact.

      - Personnel should ensure that before starting work at the bio-safety cabinets – the Ultra- violet lights are kept in “OFF” position.

      - Personnel working at the bio-safety cabinets should wear personnel protective equipments viz., head gear, face mask, goggles and gloves to protect from ultra-violet light exposure.

      - Skin protection may take the form of gloves, aprons.

      - Gloves and aprons should be selected according to the chemical resistance of the protective material to the chemical to be used.

MEDICAL CONSULTATION AND EXAMINATIONS:

      - Whenever a laboratory employee develops signs or symptoms associated with a hazardous chemical to which the employee have been exposed in the laboratory.

      - As a policy the exposure should be nil and there is no criteria or permissible levels of exposure. As a good safety practice and policy the lab strives for nil exposure.

      - Where there is exposure of certain chemicals, it shall be reported to the Director (Lab Ops)/HODs or safety officer and appropriate medical consultations and examinations will be done.

SPILLS AND ACCIDENTS:

      - Alert personnel in the immediate vicinity.

      - Confine the fire or chemical spill, if possible do so without endangering personnel.

      - Keep yourself between the emergency and an exit while attempting to confine the emergency to avoid being trapped.

      - If you have been trained to put out small fires or use an extinguisher, fight the fire if you are confident that you will be able to put it out.

      - If emergency is inside a hood, close the sash.

      - Close lab doors, if possible, to prevent spread of smoke or vapors into adjoining rooms and corridors.

      - For flammable liquid spills, shut off ignition sources, if possible.

      - Avoid unplugging equipment due to possible electrical arc between receptacle and plug. Turning off breaker will work.

      - Evacuate the emergency area. If in doubt, evacuate the building. To evacuate a building, pull the nearest fire alarm pull station on your way out.

      - Chemical Splash: follow as per MSDS if any specific instructions are available.

      - Chemical splashes in the eyes:

      - Immediately wash the eyes with potable water for at least 15 minutes.

      - Forcibly hold the eyelids open and tell the injured person	to	roll his/her eyes while continuously irrigating.

      - Do not use any substance other than potable water to wash the eyes.

      - Get medical assistance.

      - Chemical splashes on the skin:

        - Remove chemical contact with the skin by brushing off dry and water reactive chemicals and removing contaminated clothing and protective equipment that can be removed quickly (1 second or less).

        - Flush the splashed area with large amounts of potable water. Never use anything other than water or mild soap and water to clean chemicals from exposed skin.

        - Remove protective eyewear under the emergency shower as quickly as possible when chemicals have entered the eyes. In cases where the eyewear has not been breached by the chemical, remove the protective eyewear after head and face have been thoroughly washed.

        - Wash with potable water for 15 minutes or longer. Wash any part of the skin that may have had chemical contact or contact with contaminated wash water. Remove any clothing that may have come.

        - in contact with the chemical or contaminated wash water under the emergency shower.

        - Washing should give special attention to areas that may by missed such as underneath the earlobes, underneath the arms, the crotch, between the toes, the creases at the sides of the nose, a deep cleft in the chin, etc.

        - Get medical assistance. Provide MSDSs for the involved chemicals to medical personnel.

        - If a splash causes a thermal burn as well chemical burn, be sure to advise the attending medical personnel the nature of the chemical exposure.

        - After washing of the victim is completed, rescuers need to wash themselves to prevent injury from diluted chemical washed off from the victim.

TRAINING:

- Training on chemical Hygiene plan (CHP) is given to all Laboratory personnel. Refresher training is given once in a year. For new employees, training is given after induction. Training on CHP is documented.

REVIEW PERIOD

- CHP will be reviewed at least once in a year.

Evaluation of Carcinogenic Potential, Reproductive Toxicity and Acute Toxicity for Chemicals in Clinical lab

| Sr. No. | Chemicals Name | Carcinogenic Potential | Reproductive Toxicity | Acute Toxicity |
| --- | --- | --- | --- | --- |
| 1. | Glacial Acetic Acid | NA | NA | NA |
| 2. | Ammonia solution (ammonium Hydroxide) | No | No | No |
| 3. | Hydrochloric acid | No | No | No |
| 4. | Nitric Acid | NA | NA | NA |
| 5. | Sulphuric Acid | Classified I | NA | NA |
| 6. | Fomaldehyde | Classified A2 | NA | NA |
| 7. | Isopropyl Alcohol | Classified A4 | NA | NA |
| 8. | Xylene | Classified 3 | NA | NA |
| 9. | Hydrogen Peroxide | Classified A3 | NA | NA |
| 10. | Sodium hypochlorite solution | No | No | No |
| 11. | Hematology Reagents | No | No | No |
| 12. | Biochemistry Reagents | No | No | No |
| 13. | Clinical Pathology Reagents | No | No | No |
| 14. | Serology Reagents & Kits | No | No | No |
| 15. | Molecular Biology Reagents & Kits | No | No | No |

- Centrifuge Safety

- Purpose: The purpose of the procedure is to explain the Centrifuge safety for the Employees in the Laboratory.

- Scope: The Scope covers all the Centrifuge safety.

- Responsibility: All Staff of the laboratory.

- Centrifuge Safety:

Centrifuges come in three general classes: low speed (up to about 1500 rpm), high speed (up to about 5000 rpm) and for ultracentrifuge (up to about 15000 rpm). This can lead to rotor failure presenting two serious hazards: mechanical failure and dispersion of aerosols.

Approximately 90% of rotor incidents are due to user error, primarily from failure to put the lid on the rotor, failure to secure the lid, and failure to properly secure the rotor to the drive. Safety Procedures for Centrifugation:

Before Centrifugation:

      - The Sr. Technician/Group Leader must train each operator on proper operating procedures.

      - Check tubes for cracks and deformities before each use.

      - Make sure that the tubes and spindle are dry and clean.

      - Never overfill centrifuge tubes (don’t exceed ¾ full).

      - Always cap tubes before centrifugation.

      - Always balance buckets and tubes properly.

      - Check if the lid is closed on the centrifuge and secure it.

During Centrifugation:

      - Close lids at all times during operation. Never open a centrifuge until the rotor has stopped. Do not exceed safe speed.

      - The operator should not leave the centrifuge until full operating speed is attained and the

      - machine appears to be running safely without vibration.

      - Stop the centrifuge immediately if an unusual condition (noise or vibration) begins and check load balances.

After Centrifugation:

      - Allow the centrifuge to come to a complete stop before opening. Wear gloves to remove the samples.

      - Check inside of centrifuge for possible spills and leaks, and clean centrifuge thoroughly, if necessary.

      - Wash hands after removing gloves.

Centrifuging Infectious Materials or Human Samples:

      - Always wear gloves when handling tubes or rotors. Place a biohazard label on the centrifuge.

      - Wait after the run to allow aerosols to settle before opening the centrifuge. Check for possible spills or leaks.

      - In case of breakage of tube while the centrifuge is on, switch off, bring the centrifuge to a halt, using gloved hands, and remove the intact tubes. Then using forceps or any other holder, pick up all the glass fragments, discard them into the sharps disposal container.

      - Clean the inside of the centrifuge with hypochlorite solution to decontaminate the inner surface of the centrifuge.

Centrifuge Maintenance:

      - Moisture, chemicals, strong cleaning agents, and other substances can promote corrosion of centrifuge parts and cause centrifuge failure. The following are general maintenance recommendations:

      - Keep the centrifuge clean and dry.

      - Clean all spills immediately and decontaminate with 1% Sodium hypochlorite after use with biological materials.

      - Wipe the inner and outer surface once a week with a cloth soaked in 1% Sodium hypochlorite.

- Electrical Safety

- Purpose: The purpose of the procedure is to explain the Electrical safety for the Employees in the Laboratory.

- Scope: The Scope covers all the Electrical equipment’s.

- Responsibility: All Staff of the laboratory.

- Procedure: The hazards associated with the use of electricity include electrical shock and electrical fires caused by shorts and overloaded circuits or wiring. In addition, sparks from electrical equipment can serve as an ignition source for flammable or explosive vapours or combustible materials. Most incidents are a result of unsafe work practices, improper equipment use, and faulty equipment. Adherence to the following rules and procedures can significantly reduce the electrical hazards one might encounter in the workplace and ensure compliance with OHSAS (Occupation Health and Safety Assessment Series) regulations:

All electrical connection to the equipment are properly grounded to show stable voltage between 220 – 230 volts and grounded close to “0”.

Alternate power supply - UPS is connected to all the testing equipment’s and is backed with diesel generators at all the locations.

Circuit breakers as emergency stop devices are provided near the laboratory UPS and fuses are provided.

Extension cords/boards are not used as a substitute for permanent wiring.

Electrical cords or other lines are ensured not to be suspended unsupported across rooms or passageways.

All building electrical repair, splices, and wiring is performed by the electrician headed by the Head Facility.

Electrical grounding and voltage is measured for all the analytical equipment maintained by the electrician. Suitable corrective actions are taken if the reading shows defective grounding.

- Emergency Eye-Wash Procedure

- Purpose: This SOP describes the step-by-step method to be followed in the case of emergency eye wash in the event of spills to eye.

- Scope: Applicable to all staff of the laboratory in case of spillages to eye.

- Responsibility: All Lab Staff.

- Procedure:

      - Do not panic.

      - Shout out for help to allow co-workers to assist you.

      - Get to the eyewash station and turn the eyewash on.

      - Rinse both eyes with copious amounts of water for a minimum of 15 minutes.

      - Keep your eyelids open by using your hands to ensure adequate flushing of the eyes.

      - Continue rinsing eyes until emergency medical personnel arrive to assist.

      - Please note: The emergency eyewash station is only for first aid. It is not medical treatment for chemical exposures. Make certain that you seek proper medical attention.

- Things to Remember:

      - Always keep the eyewash and safety shower free of obstructions.

      - Keep the “pressure pop-off” covers (protective caps) on the eyewash stations.

      - Test eyewash stations for approximately five minutes every week.

      - Be sure that there are no electrical wires or outlets in the surrounding vicinity and the eyewash station should be easy to reach.

      - All personnel should be able to identify the location of the eyewash station before any work begins. Know the location of the eyewash station with your eyes closed.

      - Be ready to assist co-workers in the event of an accident.

- Microbiological Hazards

- Purpose: The purpose of the procedure is to explain the Microbiological Hazard for the Microbiology technicians in the Laboratory.

- Scope: The Scope covers all the Microbiological Hazards. Responsibility: All staff of the microbiology department.

Microbiology Lab Practices and Safety Rules:

      - Wash hands with disinfectant soap before entering the lab and again before leaving the lab.

      - Absolutely no food, drinks, chewing gum, or smoking is allowed in the laboratory.

      - Do not put anything in mouth such as pencils, pens, labels, or fingers.

      - Do not store food in areas where microorganisms are stored.

      - Wear Lab coat within the lab.

      - Leave protective clothing in the lab and do not wear it to other non-lab areas.

      - Avoid loose fitting items of clothing.

      - Wear appropriate shoes (sandals are not allowed) in the laboratory.

      - Keep your workspace free of all unnecessary materials.

      - Disinfect work areas in the morning with disinfectant. Laboratory equipment and work surfaces should be decontaminated with an appropriate disinfectant on a routine basis, and especially after spills, splashes, or other contamination.

      - Label everything clearly.

      - Replace caps on reagents, solution bottles, and bacterial cultures. Do not open Petri dishes in the lab unless absolutely necessary.

      - All procedures related to microbiology have to be done in the bio-safety cabinets.

      - Inoculating loops and needles should be sterilized in loop sterilizer before it is used.

      - Turn off loop sterilizer when not in use. Long hair must be tied if loop sterilizer are in use.

      - Treat all microorganisms as potential pathogens. Use appropriate care and do not take cultures out of the laboratory.

      - Wear disposable gloves when working with potentially infectious microbes or samples. If you are working with a sample that may contain a pathogen, then be extremely careful to use good bacteriological technique.

      - Sterilize equipment and materials.

      - Never pipette by mouth. Use a pipetting aid or adjustable volume pipettes.

      - Consider everything a biohazard. Do not pour anything down the sink.

      - Autoclave liquids and broth cultures to sterilize them before discarding.

      - Dispose of all solid waste material in a biohazard bag and autoclave it before discarding in the bio-medical waste.

      - Familiarize with the location of safety equipment in the lab (e.g., eye-wash station, shower, sinks, fire extinguisher, biological safety cabinet, first aid and spill kit).

      - Dispose of broken glass, razor blades, syringe needles, and sharp metal objects in the “sharps” disposal container.

      - Report all injuries or accidents immediately to the HOD/QM, no matter how small they seem.

Cleaning Spills:

      - Place caution board and call for help

      - Bring the spill kit (available at the entrance of each floor)

      - Wear appropriate PPE’s (Apron, face mask, goggles, and gloves available in the spill kit)

      - Place the absorbent pad or tissue paper over the spill.

      - Pour adequate amount of 1% sodium hypochlorite over the pad depending on the amount of spill.

      - Wait for 10-30 minutes (Depending on Minor or major spill)

      - Remove the absorbent material using tongs and discard in the yellow bag.

      - Mop the area with adequate disinfectant solution.

      - Used gloves discard in the yellow bag.

      - If broken pieces of glasses are present discard in puncture proof “SHARPS” container.

      - Document in incident and accident register.

Proper Use of Biological Safety Cabinet:

      - Bio-safety	cabinets	are	Class-A, CGMP Standards (Class-100) with 70% re- circulation and 30% exhaust.

      - All bio-safety cabinets are fitted with a virus burning units and the air is exhausted through a common hard duct to atmosphere.

      - Turn “ON” the blowers, white light, and allow to purge particulates for a minimum of 15 minutes.

      - Open the door and disinfect the interior surface of the bio-safety cabinet by moping with 70% Isopropyl Alcohol (IPA).

      - Close the door and turn “ON” the UV light for 15 minutes to keep the working place sterile.

      - Ensure that when UV light is “ON” – normal light should be in “OFF” position.

      - Turn “OFF” the UV Light, open the door and place the apparatus, consumables in the bio-safety cabinet.

      - Close the door, and turn “ON” the blower and the white light.

      - Leave the work area without any activity for 15 minutes after materials and apparatus have been placed in it.

      - Minimize movement (rapid) in and out of the bio-safety cabinet.

      - Work at least 4 inches away from the inside edge of the front vent.

      - Do not bring potentially contaminated materials out of the cabinet until they have been surface decontaminated.

      - Keep a container for discarding waste or spillage in right side corner with appropriate amount of 1% Hypochlorite Solution.

      - If a procedure requires the use of a flame use a burner with a pilot light so that the flame can be ignited only when required.

      - Leave the work area without any activity for 5 minutes after completion of work.

      - Switch “ON” the UV light for at-least 15 minutes after handling cultures in the bio-safety cabinet.

      - Remove all materials and apparatus from bio safety cabinet and disinfect the interior surface by moping with 70% Isopropyl Alcohol.

      - Close the door turn “OFF” blowers and lamp and turn “ON” the UV Light for 5 minutes. Ensure to turn “OFF” UV Light after 5 minutes.

      - Expose Settle Plate once a week for sterility check.

- Fumigation

- Purpose: This procedure explains the role of fumigation, method of fumigation of the microbiology room and checking the effectiveness of the fumigation procedure.

- Scope: The Scope covers the Microbiology Department.

- Responsibility: The HOD is responsible for this process.

- Procedure: This procedure is done to reduce the load of microorganisms which will otherwise interfere with the culture test results and prove hazardous to the staff of the microbiology department.

Perform this procedure once a week.

Take 20 ml of Incidur solution or other disinfectant solution meant for fogging is diluted in 1 litre of water and poured in to the fogging machine. Machine should be switched on for 20- 30 minutes. All the liquid fogged in the room, doors and windows should be closed tightly and allow the fog to remain in contact with air for 20-30 minutes.

The room can be open for use after 2hrs.

Ideally 200-250 square feet will be fogged effectively with 20 ml of disinfectant solution. Double strength of disinfectant can be used in case of air born contamination is expected. After fumigation, collect sample swabs from various locations in the room (Work bench, bio- safety cabinets, laminar air flow chamber, floor, AC grills) and subject them to culture using blood agar (BA).

Expose these plates to the air in different areas of the room for a period of 15 minutes so that the floating air particles carrying the microbial load settle down.

Collect swabs from the floor, working table and bio-safety hood and streak them onto separate BA plates and incubate them as above.

Take care not to generate any air current. Incubate the Blood agar plates for 48 hrs. at 370C.

Evaluate the sterility and effectiveness of fumigation based on the colony count and also the type of bacteria grown.

Examine the culture for growth. If the values are found to be <10 colonies the fumigation is successful. If the values are found to be >10 colonies – the fumigation process is repeated.

Start the routine work only if culture is negative for organisms. Record the details in the Fumigation and Room Sterility Register.

- Freezer And Refrigerator Safety

- Purpose: The purpose of the procedure is to explain the Do’s and Don’ts for freezer and Refrigerator for the Employees in the laboratory.

- Responsibility: All Staff of the laboratory.

- Procedure:

Do’s:

      - All material kept inside the freezer should be recorded in the inventory basis.

      - All spills inside should be cleaned with water and 1% hypochlorite solution as per spill management policy.

Don’ts:

      - No flammable substance to be kept in the refrigerator. No food items to be kept in the refrigerator.

        - Freezer Refrigerator doors not to be left open for a long time UNATTENDED.

- Mecury Reduction and Spill Planning and Prevention

- Purpose: This procedure describes the step-by-step method for reduction of mercury in the laboratory.

- Scope: This SOP will be used only for reduction of mercury.

- Responsibilities: Laboratory Staff.

- Procedure:

Policy: TRUSTlab Labs makes every effort to prevent a mercury spill from occurring due to the health effects of mercury vapor, the cost associated with the clean-up and the difficulty in removing mercury vapor from certain areas. The best way to prevent a release of elemental mercury is not to have it! Proper disposal or reclamation of the mercury is strongly recommended.

If mercury-containing devices are used, following precautionary measures are followed: Always store mercury in unbreakable container.

Store in well-ventilated area.

When using mercury-containing devices, do so over plastic containment tray.

Make sure container is of sufficient size to contain the greatest amount of mercury that may be spilled.

Transfers of mercury should be done under an appropriate exhaust hood and over a plastic containment tray. Make sure container is of sufficient size to contain the greatest amount of mercury that may be spilled.

Maintain an appropriate spill kit in area where mercury is used or stored. Know and follow all manufacturer directions.

Whenever possible, choose non-mercury containing devices.

If you use fluorescent lights, remember they are considered universal waste and should be managed as such (40 CFR 273).

Work surfaces should have few crevices and carpeted areas should never be used.

- Radioactive Material Handling

- Purpose This is to prevent any uptake of radioactive material into the human body by inhalation or ingestion, as well as limit external radiation exposure to personnel, and prevent further spread of contamination.

- Scope: Applicable to TRUSTlab Labs. Responsibility: All staff of the laboratory.

- Procedure:

If contamination is detected, evaluate the extent of the contamination.

Precautions: Spreading of radioactive material beyond the spill area can easily occur by the movement of personnel involved in the spill or clean-up effort. Prevent spreading of radioactive material and contamination by confining all movement of personnel until they have been monitored and found to be free of contamination. And NEVER risk external nor internal exposure to save equipment or an experiment.

Severity of Spill: The decision to call an incident a major spill versus a minor spill is one that depends on many incident specific variables such as the isotope spilled, the amount spilled, number of individuals affected, and the likelihood of spread of contamination. For this reason we offer guidelines to help with the decision, not definitions. First assess contamination and injury of personnel! Then decide if the lab can handle the situation safely alone, or needs the assistance of emergency personnel. If no one has been injured or contaminated and you feel capable of handling the spill yourself, perhaps this is minor spill, again depending on the isotope spilled and amount.

If anyone is injured in any way or highly contaminated, consider this to be a major spill and call radiation safety immediately!

Supplies needed to clean up a radiological spill: Plastic bags, “Caution Radioactive Material” tape, absorbent material (i.e. paper towels, blue pads), decon detergent like “rad con”, rope or tape. Also a must is protective clothing, to include, but not limited to, disposable gloves, footwear, lab coats, and safety goggles. And you will need a portable survey meter and/or materials for taking swipes.

For MINOR Radiological Spills:

      - Attend to any contaminated personnel and remove them from the spill area. People first, then cleanup!

      - Alert people in the immediate area of the spill and close off the spill area, keeping uninvolved people OUT!

      - If volatile materials are involved, close all windows and shut off the ventilation, heating and air conditioning units, to the room, and leave fume hoods on to exhaust the contaminated air. Notify Radiation Safety of the spill to the safety officer.

      - Wear protective equipment including safety goggles; double layer of disposable gloves, changing the outer layer of gloves frequently; shoe covers; and long-sleeve lab coat as you clean up the spill.

      - Always move from the outside of the spill inward; from least contaminated areas to most contaminated areas!

      - Place absorbent paper towels or blue pads over liquid spills. Place dampened towels over spills of solid materials, being very careful NOT to breathe any dust coming from the spilled material. Be sure whatever material you chose to dampen the towels with will not react in any way with the spilled material!

      - Place towels in a plastic bag and seal for transfer to a radioactive waste container. With gamma ray emitters, use forceps to pick up the paper towels. As you do this, do not press hard, but do not leave any liquids behind. Be sure to add contaminated gloves or other contaminated disposable material to rad waste container.

      - For further decontamination, use a mild soapy solution or a commercial decontaminant such as “rad con” or “radiac wash” to clean up the remains of the spill.

      - Monitor area, hands, and shoes for contamination with an appropriate survey meter or method,

      - i.e. Geiger counter or swipes. Recall some isotopes can only be detected with swipes. Remove anything contaminated and decon or store for decay.

      - Repeat cleanup/decon until contamination is no longer detected.

      - Draw a map and take extensive swipes of the entire area, remembering to include floors, equipment, handles, etc. Document final survey, showing area to be free of contamination and that proper spill procedures were followed. Please see attached sample log sheets; keep these in your records. DOCUMENT EVERYTHING!

For MAJOR Radiological Spills:

      - Attend to injured persons, if any, and remove them from the spill area. People first, then cleanup! Medical attention ALWAYS comes first.

      - Alert people to evacuate, but monitor all individuals and let no one leave the area until they have been shown to be free of contamination. Pay close attention to shoes, you do not want to track contamination elsewhere! Keep others out of the area!

      - If volatile materials are involved, shut off ventilation, heating and air conditioning equipment, to the room, and switch on fume hoods to exhaust the contaminated air and keep the room under negative pressure. Seal off the room.

      - Evaluate the situation, and at earliest convenience, document the initial assessment. Call the Safety Officer (SO) for assistance.

      - Document names of potentially contaminated personnel and have them stay in one area, away from the spill, until they have been monitored and shown free of contamination. Remove contaminated clothing and wash contaminated skin in warm, soapy water, being careful as to not damage the skin. If contamination of skin persists, induce sweating by covering the area with plastic, or a disposable glove, if on the hand. Wash the affected area again to remove any contamination that was released by the perspiration.

      - NOTE: Contamination on hands can be greatly reduced by trimming the fingernails. Also, open wounds and hard to reach places should be irrigated profusely with lukewarm water and clean with a swab; soap and detergent; and possibly a soft brush, if needed. Avoid the use of organic solvents, which may increase skin penetration.

      - Close doors and prevent entrance into affected area, keeping ALL uninvolved people OUT! Distinguish clean vs. contaminated areas. Have person knowledgeable of incident and area assist emergency personnel.

      - ALWAYS move from outside of the spill, inward; from least contaminated areas to most contaminated areas!

      - Draw map and take extensive swipes of the area, remembering to include floors, handles, equipment, etc. Document final survey, showing area to be free of contamination and that proper spill procedures were followed. DOCUMENT EVERYTHING!

      - Special Situations:

        - In the case of a fire: Follow fire emergency procedures.

        - Immediately notify persons in room and building.

        - Activate the nearest alarm pull station.

        - Call 9999 from a safe location to report the emergency.

        - Take everyone to a safe place and do not let anyone leave there until they have been surveyed to be free of contamination.

        - When the major emergency is no longer a factor, decon the room as normal.

      - Be sure to document everything!

      - In the case of a spill involving biological materials that are radioactive:

      - Decontaminate the biological material using an approved disinfectant while following the procedures outlined in the Biological Spill Procedures section of the “Emergency Response and Incident Reporting Guide”.

      - Decontaminate the remaining radioactive material while following normal procedures.

      - Be sure to document everything!

      - In summary:	This acronym may help you remember the highlights of what to do in a radiological spill incident:

        - S top what you are doing.

        - P resume you are contaminated.

        - I nform others in the immediate area of the spill.

        - L ocalize the spilled material(s).

        - L abel the area as contaminated.

- Handling Of Hazardous Materials

- Purpose: This procedure describes the handling and storage of hazardous materials used in TRUSTlab Labs.

- Scope: This procedure is applicable to all areas using the hazardous materials.

- Definition:

- Hazardous material: A hazardous material is a material, which is capable of producing harmful physical or health effects.

Harmful physical effects include fire, sudden release of pressure, explosion, and other violent reactions. Harmful health effects include acute conditions and chronic conditions.

Acute conditions develop soon after over-exposure to hazardous materials and include burns, rashes, respiratory distress, convulsions, and possibly even death.

Chronic conditions develop after long-term exposure to hazardous materials and include cancers, nervous system disorders, and damage to other organ systems.

Description: Hazardous materials may generally be assigned to one or more of the following classifications.

Flammable liquid - any liquid having a flash point below 1000	(37.8  C); i.e., at 1000

or less the Combustible liquid- any liquid having a flash point between 100 and 2000 (37.8-93.3 C). Liquid produces enough vapors to ignite if exposed to an ignition source.

Flammable solid - a substance that can cause a fire through friction, absorption of moisture or spontaneous chemical changes and, when ignited, will burn so vigorously that it creates a hazard.

Oxidizer - a substance that readily yields oxygen or other electron acceptor to stimulate the combustion of organic matter (fuel).

Corrosive - a liquid that corrodes steel (SAE 1020) at a rate greater than 6.35 mm (0.250 in.) at a test temperature of 130 than 12.5.

(55  C) or has a pH less than 2 or greater

Organic Peroxide - an organic compound containing the chemical bond -0- 0- (oxygen joined to oxygen).

Poison - a substance so toxic that it presents a risk to life or health.

Explosive- any chemical compound, mixture or device that reacts or decomposes with substantial instantaneous release of gas and heat.

Compressed Gas - a substance in gas or liquid form contained in a vessel under pressure. This includes cylinders and aerosol cans. These substances may be flammable, non- flammable, or poisonous.

Cryogenics - substances which are extremely cold such as liquid nitrogen, liquid helium and dry ice. These substances may also become asphyxiation hazards if spilled in non-ventilated areas.

Radioactive - any material having a specific activity greater than 0.002 micro curies per gram (uCi/g).

Biomedical - tissues, organs, and blood from humans. Syringes, needles, sharps, and other objects containing materials above.

Policy:

    - Storage of Hazardous Materials:

      - Hazardous materials must be stored based on their compatibility, not simply in alphabetical order. Store materials of the same hazard together i.e. flammables with flammables and oxidizers with oxidizers.

      - Hazardous substances should be stored in an orderly manner with older products most accessible and the newer products least accessible.

      - Good housekeeping must be practiced in areas where hazardous products are stored.

      - All hazardous materials must be properly labeled including their exact contents, hazardous properties, date of receipt, and if appropriate, date of expiration.

      - Hazardous substances should be stored in original containers in which they were packaged at the manufacturing plant. If this is not practical, these products should be transferred according to manufacturers' recommendations into containers that are constructed to withstand the effects of the product over the maximum storage time.

      - Incompatible materials must not be stored such that they may come in contact with each other.

Handling Hazardous Materials: Protective Measures:

      - Whenever it is feasible, engineering controls must be used to reduce employee exposures to hazardous materials. The two most common engineering controls are the use of local exhaust and general ventilation. These measures limit an employee's exposure to airborne contaminants.

      - When engineering controls are not available, or they fail to adequately reduce hazards, other personal protective equipment is required. Examples of personal protective equipment include: safety glasses, hearing protection, gloves, respirators, etc.

      - Personal protection devices must be provided and worn in accordance with the manufacturer's recommendations indicated on the label of the product or as stated in the Material Safety Data Sheet (MSDS) for the product.

Spill Procedures:

- Hazardous chemical spills can be handled effectively. Spill procedures should include the following:

      - The potential location of possible spills.

      - The quantities of material that might be released.

      - Chemical and physical properties of the material. This information may be obtained from the Material Safety Data Sheet or label.

      - Hazardous properties of the material (consult the MSDS).

      - The types of personal protection equipment that may be needed for cleanup. Location and contents of spill kits that should be made available where possible. These kits might include the following:

      - Neutralizing agents such as sodium carbonate, sodium bicarbonate or sodium bisulfate. Absorbents such as vermiculite, "super sorb," or absorbent pillows or dikes. Paper towels, rags and sponges may be used, but caution should be exercised because some chemicals may ignite upon contact with them.

      - Plastic scoops and shovels, disposable mops, disposable protective clothing and containers to receive the spilled material and all items used in the cleanup.

      - The following general procedure may be used, but should be tailored to the individual needs of the handlers and the specific hazard associated with the hazardous material.

      - If the spilled material is flammable, turn off ignition and heat sources. Attend to any person who may have been contaminated.

      - Notify individuals in the area about the spill.

      - Evacuate nonessential personnel.

      - Avoid breathing vapors of spilled material. Establish an exhaust or ventilation, if it is safe to do so. Air handling units are not to be used because they re-circulate the hazardous vapors.

- Ergonomics

- Purpose: The purpose of this procedure to implement an ergonomics program to prevent temporary or permanent staff injury. This can occur when incorrect or repetitive body movements are performed during normal operations. TRUSTlab Labs has established a process for developing task specific guidelines to assist in recognizing and controlling potential ergonomic hazards.

- Scope: Applicable to all Ergonomic requirements of the laboratory

- Responsibility: HOD and Lab Technicians.

- Procedure:

The person designated to ensure that proper ergonomic techniques are implemented for this Ergonomic Program is the Safety Officer. This person shall:

Recognize the potential of ergonomic stressors that could adversely impact staff health and safety.

Implement corrective actions to reduce the duration, frequency, and severity of exposure to ergonomic stressors (establish controls such as addressing proper work-related techniques or provide on the job training).

Ensure staff reporting ergonomic-related symptoms are referred to the nearest medical facility for surveillance or treatment,

Perform worksite analysis or surveys to validate whether efforts to eliminate or reduce ergonomic risk factors have been effective to the extent feasible,

All new staff shall receive additional job specific training in accordance with this Facility’s written training program;

    - Worksite Analysis:

- Be aware of common contributing ergonomic-related problems or conditions at other ergonomic facilities as they may also be a concern for this Program. Ergonomic worksite analysis emphasizes an evaluation of work place practice controls and operational tasks.

- These may include the following:

      - TRUSTlab Labs strive to keep the workplace free of hazards.

      - If hazards exist, TRUSTlab Labs eliminates which may cause death or serious physical harm.

      - Potential hazards been recognized.

      - There is a feasible means of abatement for those hazards.

      - Staff reporting ergonomic-related symptoms.

      - Staff ensures proper ergonomic improvements are implemented in their specific work area(s).

      - Staff consistently follows suggested ergonomic improvements.

Review Injury and Illness Records:

- If access is available, the Safety Officer is responsible for maintaining health and safety records review staff injury and illness records for potential ergonomic-related problems. These records may include:

| Employee’s compensation claims | First aid logs |
| --- | --- |
| Group health insurance records | Records of staff complaints or grievances |
| Absentee and turnover records | First report of injury/OSHA 300 log |

Identify Potential Musculoskeletal Disorders:

- After review of the above information sources, identify entries that may indicate the presence of musculoskeletal disorders (MSDs). While there is no single diagnosis for MSDs, the conditions identified below are commonly included in the range of conditions that potentially constitute an MSD

| Tendonitis | deQuervain’s disease |
| --- | --- |
| Tenosynovitis | Ganglion cyst |
| Epicondylitis | Thoracic outlet syndrome |
| Carpal tunnel syndrome | Sprains, strains, or tears |
| Bursitis | Pain |
| Tendonitis | deQuervain’s disease |
| Tenosynovitis | Ganglion cyst |
| Epicondylitis | Thoracic outlet syndrome |
| Carpal tunnel syndrome | Sprains, strains, or tears |
| Bursitis | Pain |

Categorize Hazards: Once recognized, categorize the MSD cases as related to work tasks to pinpoint where patterns or trends exist.

Seek Staff Input: Request information from staff about the existence of ergonomic problems related to particular jobs or tasks. This is accomplished by:

      - Speaking with staff

      - Conducting symptom surveys

      - Providing staff with questionnaires

      - Ensuring effective housekeeping to minimize the potential for slips, trips, and falls

Hazard Control Recommendation Plan:

- If ergonomic hazards are identified, draft a written hazard control plan and update it as needed, including:

      - Descriptions of specific jobs

      - Details of all potential hazards associated with the job task

      - Possible approaches to controlling the hazard

      - Descriptions highlighting the effectiveness of each control approach.

Ergonomic-related Hazards and Controls: The following is a list of Ergonomic tasks, potential ergonomic hazards, and possible control methods. Note: not all hazards have been identified. Some solutions may work for multiple situations but may not always work for each task or individual.

| Sr. No. | Task | Body Positions/ | Work Practices/ | Proper Equipment |
| --- | --- | --- | --- | --- |
| 1. | Seating | Feet should rest | Avoid sitting at the | Use a footrest if feet |
|  |  | flat on the floor | edge of the seat, sit | do not reach the floor. |
|  |  | or a footrest. | all the way back into | If back support is not |
|  |  | Chair should | the seat to provide | adequate or if the |
|  |  | provide adequate | back support. | seat pan is too long, |
|  |  | low back and | Get out of chair at | try a rolled up towel |
|  |  | thigh support. | least every half hour | or a back support |
|  |  | Front edge of | to help relieve stress | cushion to provide |
|  |  | chair should not | on back. Before | support. Remove or |
|  |  | press up against | starting work, make | adjust armrests that |
|  |  | back of knees. | sure chair is | hinder work activities. |
|  |  |  | properly adjusted. |  |
|  | 2. | Pipetting | Maintain straight wrists. Keep elbows close to body. | Keep waste bins, beakers, etc., as close as possible. Take frequent micro-breaks away from pipetting (at least every 15-30 seconds). Share workload between right and left hands. Rotate pipetting tasks with other employees as feasible. alternate activities to avoid continuous pipetting for long periods. Occasionally | Use shorter pipette and pipette-tips. Choose pipettes that require minimal hand and finger effort. For highly repetitive jobs, utilize automated processes or multi- channel pipettes where feasible. See seating. |  |

| 3. | Test Tube Handling | Maintain straight wrists Work / elbows close to body avoid reaching upward or stooping low | Arrange tubes to minimize reaching/twisting. Share workload between right and left hands. Take adequate breaks away from handling activity (even short several second "micro- breaks" help. Use both hands to open tubes | Use upside-down containers to raise tube racks when needed. Use vortex mixer rack instead of holding tubes by hand. Use cap removers to help minimize pinch gripping. To avoid forearms resting on sharp edges, pad edges or use a cushion to pad forearm. |
| --- | --- | --- | --- | --- |
| 4. | Microscope use | Maintain straight wrists Avoid tilted head/ neck postures | Take frequent micro breaks to rest eyes (momentarily close eyes or focus on far away objects to vary focal length). Keep lens clean and in good condition. Spread microscope work throughout the day or rotate microscope | Raise and stand microscope at a slight tilt to allow a more upright head/ neck posture. To avoid forearms resting on sharp edges, pad edges or use a cushion to pad forearm. See seating (above). |
| 5. | Hand Tool Use | Maintain straight wrists Avoid pinch gripping tools when possible | Take occasional micro breaks away from tool use (at least every 15- 30 minutes) Share workload between right and left hands. | Choose the right tool for the job. Ensure tools are in proper working order. Increase size of tool handles where possible to minimize gripping effort. |
| 6. | General Work Tips | Minimize use of awkward body postures | For any continuous/ repetitive task, take frequent micro- breaks away from the primary activity. Arrange work scheduling to allow occasional alternating of tasks. Rotate tasks intermittently between left and right hands to avoid overuse of any one side. For highly continuous/ repetitive tasks, consider a rotation of employees to help safely distribute workload. | When purchasing equipment models that adjust in size are preferable. Use the proper equipment for the task. Know how to properly use the equipment. Where feasible, use automated processes to reduce/ eliminate high repetition or forces. |

- Management Of Incidents And Accidents

- Purpose: The purpose of the procedure is to explain the management of Incident and Accidents for the Employees in the laboratory.

- Scope: The Scope covers all the management of Incident and Accidents.

- Responsibility: All Staff of the laboratory.

Procedure:

Needle Stick Injury (Occupational Exposure to Blood-borne Viral Infections):

- In cases of exposure to blood or body fluids, the following procedures should be followed:

      - Do not panic.

      - Wash hands immediately in running water for 10 min. (Don’t squeeze).

      - Report to the concerned HOD; get the patient’s HIV, HbsAg and HCV status from history. If not done do it at once.

      - Fill	up	the	needle	stick	injury/splashes	form (Available	in	all departments).

      - Report and Consult Safety Officer (Don’t Delay).

      - Get the evaluation report from the Head of the Department for PEP (To be effective, post exposure prohylaxis {PEP} should be started within 2 hrs. Hence don’t delay)

      - Follow-up of viral screening is done at the end of 6 weeks, 3 months and 6 months as per NACO and OSHA guidelines and documented.

      - Immediate Contact Procedure:

      - The Head Clinical Laboratory Services should be informed of the incident.

      - The Head Clinical Laboratory Services should immediately refer to the Infection Control Nurse.

Other Types of Exposure:

      - Splashes to the nose, mouth or skin, should be flushed with plenty of water.

      - Eyes should be irrigated with clean water saline or sterile irritants.

Sharps Disposal:

General Principles for Handling and use of Sharps:

Avoid sharps usage wherever possible.

      - Do not bend or break needles by hand.

      - Sharps must not be passed directly from hand to hand.

      - Never leave sharps lying around; dispose them carefully.

      - Do not keep syringes, needles or any other sharp objects in pockets.

      - NEVER RE-SHEATH/RE-CAP USED NEEDLES.

      - Dispose the used sharps into sharps disposal container.

Management of Blood/Body fluid Spills:

    - Bio hazard Spill:

      - Place caution board and call for help.

      - Bring the spill kit (available at the entrance of each floor).

      - Wear appropriate PPE’s (Apron, face mask, goggles, and gloves available in the spill kit).

      - Place the absorbent pad or tissue paper over the spill.

      - Pour adequate amount of 1% sodium hypochlorite over the pad depending on the amount of spill.

      - Wait for 10-30 minutes (Depending on Minor or major spill)

      - Remove the absorbent material using tongs and discard in the yellow bag.

      - Mop the area with adequate disinfectant solution.

      - Used gloves discard in the yellow bag.

      - If broken pieces of glasses are present discard in puncture proof “SHARPS” container.

      - Document in incident and accident register.

    - Hazmat Spill:

      - Place the caution board and call for help.

      - Bring the spill kit (From the respective department).

      - Wear appropriate PPE’s (Apron, face mask, goggles, gloves and gum boots)

      - Place the absorbing universal pad or pillow (if major) over the spill.

      - Wait till the spill gets absorbed.

      - Remove the absorbent material using dust-pan and brush from outwards to inwards and discard in the black bag.

      - If broken pieces of glasses are present discard in puncture proof “SHARPS” container.

      - If the spill emits toxic gases or fumes close the doors for traffic. Open the windows if possible, for ventilation of the noxious gas and open the door after complete evacuation of noxious gas.

      - Mop the area with adequate disinfectant solution.

      - Used gloves discard in the yellow bag.

      - Document in incident and accident register.

    - Management of Mercury Spillage:

      - Place caution board and call for help.

      - Wear appropriate PPE’s (Apron, gloves and mask etc.,).

      - Locate the spill.

      - Use torchlight to see the extent of spill

      - Remove wrist watch, gold ornaments and keep it safe.

      - Take water in the plastic container.

      - Group the entire mercury balls to one place with the help of plastic paper card.

      - Draw all the mercury beads with the help of plastic syringe and dispose in plastic container/ Pour sand over it and collect it in a bin through dust- pan

      - Sprinkle sulphur / zinc powder which will stain the mercury darker and easily visible to collect small and tiny beads.

      - Dispose all in the plastic container. Put the plastic container in the zip lock cover.

      - Wipe the area with tissue paper.

      - Label and hand over to the house keeping department for storage and discard.

      - Ventilate the area by opening the window.

      - Document in the incident and accident register.

    - Management of Broken / Spilled samples

      - Wear disposable gloves throughout the procedure.

      - Broken/spilled sample should be covered with an absorbent material soaked in 1% hypochlorite solution.

      - After 10 minutes, the breakage and the cloth should be cleared away into a dust pan

      - The area should be swabbed with disinfectant.

      - The broken material and swabs should be placed in a “contaminated” waste container for autoclaving.

      - The dust pan should be autoclaved or placed in disinfectant for 24 hours.

Never Miss: All incident /accident including near miss is essential events and must be reported and the details should be entered in the Incident/accident register.

- General Housekeeping Procedure

- Purpose: The purpose of the procedure is to explain the General housekeeping Procedure for the Employees in the Laboratory.

- Responsibility: All housekeeping and technical staff of the laboratory.

Procedure:

      - Arrange all records and other documents in a specified order in a specified area of the laboratory.

      - Arrange and place all the consumables in their respective places at suitably identified locations.

      - Do not place personal belongings in the department work area.

      - Dust all equipment and tables at the beginning of the day and wipe them with a soft cloth.

      - Sweep the floor of the department and mop with disinfectant.

      - Clean all glass doors with a soft cloth in the afternoon.

      - Empty the Dustbins twice in a day.

      - Housekeeping Staff is Responsible for:

      - Floor cleaning.

      - Toilet cleaning.

      - Emptying the Dustbins

      - Collection and Removal of Bio-medical waste.

      - Cleaning – Daily and Weekly: The cleaning schedule followed in the laboratory is as follows:

| Daily Cleaning |  | Responsibility |
| --- | --- | --- |
| 0900 Hrs – 1000 Hrs | Sweeping, Mopping and Dusting | Housekeeping |
| 1000 Hrs – 1200 Hrs | Sink cleaning, Dustbins, Sample transport trays and toilet cleaning | Housekeeping |
| 1500 Hrs– 1600 Hrs | Sweeping | Housekeeping |
| 1600 Hrs – 1800 Hrs | Glassware Cleaning | Housekeeping |
|  | Sample Transport Lift | Housekeeping |
|  | Test tube racks and pipette stand washing | Lab Staff |
|  | Table Tops and Equipment | Lab Staff |
| Weekly Cleaning |  |  |
| Saturday | Defrost of Refrigerator | Lab Staff |

Cleaning Agents: The cleaning agents used for cleaning is as follows:

| Bench top (working tables) | 1% Hypo |
| --- | --- |
| Sink | Soap Solution |
| Toilets | Lysol and Harpic |
| Trays/Dustbins/Test tube racks/Pipette Stands etc | Soap Solution |
| Floor Cleaning | Lysol |
| Glass cleaning | Colin |

    - Preparation Of 1% Sodium Hypochlorite Preparation:

      - Prepare 1% of Sodium hypochlorite by adding 250ml of concentrated 4% Sodium hypochlorite to 750 ml of Distilled water and should be used within 6 hours.

      - Prepare 1% of Sodium hypochlorite by adding 200ml of concentrated 5% Sodium hypochlorite to 800 ml of Distilled water and should be used within 6 hours.

      - Fill the small Sodium hypochlorite bottles till the mark. Do not fill above the mark, as it would leak.

      - Stability:

      - Diluted Sodium hypochlorite is stable only for 6 hrs, hence prepare the Sodium hypochlorite as & when needed & fill the bottles.

Cleaning of the work surface:

      - Clean all surfaces of the hood, with 1% hypochlorite, water and finally with 70% isopropanol. Remove the discard box and place it in hazardous waste.

      - UV should be ON for 20 min before the every batch of the RNA isolation.

      - Cleaning with Sodium hypochlorite & spirit:

      - All the work surfaces must be wiped with diluted Sodium hypochlorite and dried with spirit, before & after work.

Decontamination Procedure:

    - Decontamination of Centrifuges:

- Clean the surfaces and inside with 1% sodium hypochlorite once a week by the laboratory staff.

    - Decontamination of Dustbin:

- Daily wash with DETERGENT SOAP and rinse with 1% sodium hypochlorite.

    - Cleaning of Premises:

- The Premises including furniture, windows and doors are cleaned on daily basis before:

      - 1000 and 1500 HRS by housekeeping staff.

    - Cleaning of Glassware:

- All used glassware like conical flasks, test tubes, beakers, measuring cylinders are cleaned daily using the soap solution and dried before using by housekeeping staff.

    - Cleaning of Staining Jar:

- Heavily stained jars are discarded. Jars are cleaned once a week using detergent soap and dried.

    - Cleaning of Sample Transport Lift:

- The inside area of the sample transport lift is cleaned twice a day using 70% IPA.

    - Cleaning of Sample Transporting trays:

- The plastic trays are cleaned once a day or and when contaminated using soap solution and dried before using by housekeeping staff.

    - Cleaning of Dust Bins - Biomedical Waste Collection:

- The containers are cleaned daily and as and when found contamination with soap solution, rinse with 1% sodium hypochlorite followed by drying by the housekeeping staff

    - Cleaning of Test Tube Racks and Pipette Stands:

- Test tube racks are cleaned once a week and as and when contaminated by the housekeeping staff with soap solution and dried.

    - Cleaning of Test tubes:

- Soak the tubes in the soap solution for 30 minutes. Then remove and wash in water for 3 times. Dry them before use.

    - Preparation of 1% Hypochlorite

- 1% sodium hypochlorite solution is prepared by dissolving 25 ml of 4% sodium hypochlorite solution in 75 ml of water and a contact period between the chemicals of 30 minutes is allowed.

- Microbiological Waste

- Purpose: The purpose of this procedure is to maintenance of proper Microbiological Waste.

- Scope: Management of Microbiological waste handling to in the Laboratory.

- Responsibility: All Laboratory Staff.

- Procedure

Microbiology Lab Practices and Safety Rules:

      - Wash hands with disinfectant soap before entering the lab and again before leaving the lab.

      - Absolutely no food, drinks, chewing gum, or smoking is allowed in the Laboratory.

      - Do not put anything in mouth such as pencils, pens, labels, or fingers.

      - Do not store food in areas where microorganisms are stored.

      - Wear Lab coat and Safety glasses within the lab. Leave protective clothing in the lab and do not wear it to other non-lab areas.

      - Avoid loose fitting items of clothing. Wear appropriate shoes (sandals are not allowed) in the Laboratory.

      - Keep your workspace free of all unnecessary materials.

      - Disinfect work areas before and after use with 1 % Sodium Hypochlorite. Laboratory equipment and work surfaces should be decontaminated with an appropriate disinfectant on a routine basis, and especially after spills, splashes, or other contamination.

      - Label everything clearly.

      - Replace caps on reagents, solution bottles, and bacterial cultures. Do not open Petri dishes in the lab unless absolutely necessary.

      - Inoculating loops and needles should be flame sterilized in a Bunsen burner before it is used.

      - Turn off Bunsen burners when not in use. Long hair must be tied if Bunsen burners are in use.

      - Treat all microorganisms as potential pathogens. Use appropriate care and do not take cultures out of the Laboratory.

      - Wear disposable gloves when working with potentially infectious microbes or samples. If you are working with a sample that may contain a pathogen, then be extremely careful to use good bacteriological technique.

      - Sterilize equipment and materials.

      - Never pipette by mouth. Use a pipetting aid or adjustable volume pipettes.

      - Consider everything a biohazard. Do not pour anything down the sink. Autoclave liquids and broth cultures to sterilize them before discarding.

      - Dispose of all solid waste material in a biohazard bag and autoclave it before discarding in the regular trash.

      - Familiarize with the location of safety equipment in the lab (e.g., eye-wash station, shower, sinks, fire extinguisher, biological safety cabinet, first aid kit, emergency gas valve, etc.).

      - Dispose broken glass in the broken glass container.

      - Dispose of razor blades, syringe needles, and sharp metal objects in the “sharps” container.

      - Report all injuries or accidents immediately to the Director of LAB / Quality Manager, no matter how small they seem.

      - Cleaning Spills:

      - Report spills and accidents immediately to the Director of LAB/ Quality Manager and follow the procedures given below.

      - Wear a lab coat, disposable gloves, safety glasses or a face shield, and if needed, approved respiratory equipment.

      - Soak a paper towel(s) in an appropriate disinfectant (fresh 1% sodium hypochlorite solution) and place around the spill area.

      - Working from the outer edges into the center, clean the spill area with fresh towels soaked in the disinfectant. Be sure to decontaminate any areas or surfaces that have been affected by the spill. Allow 10 minutes contact time.

      - Place the paper towels and gloves into a biohazard bag and autoclave these materials to sterilize them.

      - Dispose contaminated clothing properly.

      - Wash hands with a disinfectant soap.

- Bio-Medical Waste Handling And Disposal

- Purpose: The purpose of this procedure is to give instructions to the housekeeping staff and technical staff of the laboratory regarding the manner of handling and segregation of general and biomedical waste generated from within the lab prior to disposal and the method of disposing the blood and urine samples after completing the tests.

- Scope: Biomedical waste at the laboratory includes the following components:

      - Human blood and all body fluids

      - Urine and Stool Samples.

      - Human body parts.

      - Cotton, used test tubes, sample collection tubes and containers.

- Responsibility: The housekeeping staff and all the technical staff of the laboratory are responsible for this process.

  - General Procedure:

      - Segregate all biomedical waste according to their type and dispose them in respective colour- coded bags containing the bio hazard symbol.

      - Do not reuse the following in the lab - Syringes, non vacutainer tubes, polypropylene tubes, Cover slips, Plastic sample containers, Micropipette tips, etc.

      - Cover all urine and stool containers with lids and the test tubes containing blood samples with cork.

      - Discard all used reagent vials in to containers lined with Blue bags.

      - Dispose the used needles in puncture-proof container containing 1% sodium hypochlorite solution.

      - Dispose all soiled cotton and tissue into a separate bin lined with Yellow colored bags.

      - Do not discard left over specimen directly in to the sink without disinfecting them with sodium hypochlorite.

      - At the end of the day, drain all the used disinfectant into the sink and flush it with water.

      - Pack all the treated biomedical waste into Black bags well and leave them for final disposal by the Maridi.

      - Keep the processed samples separately in a designated area and transfer them to the larger Red colored bags.

      - Discard the non-biomedical general waste in the dustbin lined with Blue colored bags and leave it for final disposal through Corporation authorities.

      - Clean spills of blood & body fluids using 1% sodium hypochlorite solution and allow a minimum contact period of 30 minutes.

      - Clean the workbenches with 1% sodium hypochlorite solution at the beginning and end of the day.

Decontamination of Lab Waste:

      - All used slides, tips and plastic ware are soaked in 1% Sodium Hypochlorite solution before they are released for disposal.

      - All liquid waste fluid from the auto analyzers is poured into the sink	after decontamination for 20 minutes in 1% sodium hypochlorite.

Collection and Segregation of Solid Waste:

| Sr. No. | Colour/Type of Bag for Disposal | Biomedical Waste to be Disposed |
| --- | --- | --- |
| 1 | White Jar (Puncture Proof Container) | Needles, scalpels, slides, nails, blades, sharp items and other broken glass items. |
| 2 | Red | Waste generated from disposable plastics items such as tubing, bottles, intravenous tubes and sets, syringes, gloves, and vacutainers with their |
| 3 | Yellow | Body parts, placenta, human tissue specimens, surgical waste, cotton bandages and gauzes, dressings, solid and infectious waste. |
| 4 | Blue/Cardboard boxes with blue colored marking | Discarded and contaminated glass including medicine vials and ampoules. |

Storage and Disposal of Solid Waste:

      - As when containers are filled 2/3rd, the containers are tied up and labeled with details of the location collected in the temporary storage area of the laboratory.

      - The segregation is done at the point of waste generation and collected in the Morning and Afternoon by the House keeping staff for biomedical waste treatment at autoclave room.

      - The biomedical waste is temporarily stored after treatment at dedicated area allocated in the laboratory premises.

      - The authorized collection and treatment agent collects the same within 48hrs in the afternoon for treatment at their unit.

      - The temporary storage provided in well secured against spillage, access and theft.

      - Collect and store biomedical waste separately prior to handing it over the same Common Bio-medical Waste Treatment and Disposal Facilities (CBWTF). Use a dedicated collection bin labelled as “Covid-19” to store COVID-19 waste and kept separately in a temporary storage room prior to handing over to authorized staff of CBWTF. Biomedical waste collected from these rooms should go directly into CBWTF collection van.

      - In addition to mandatory labelling bags/containers used for collecting biomedical waste from sample collection rooms, BSL2 facility and PPE doffing area should be labelled as “COVID-19 waste”. This marking would enable CBWTFs to identify the waste easily for priority treatment and disposal immediately upon the receipt.

      - General waste not having contamination should be disposed as solid waste as per SWM rules, 2016.

Collection and Disposal of Liquid Waste:

      - All body fluids and urine specimens are discarded after diluting with 1% sodium hypo chlorite into the drain.

Disposal of Chemicals:

      - Used chemicals – Xylene and Alcohol: Used Xylene and Alcohol is stored in containers with properly labeling and handed over to the bio- medical agency after recording in the log book.

      - Used Paraffin Wax - is stored in yellow bags with properly labeling and handed over to the bio-medical agency after recording in the log book.

      - Used Formalin - is discard in running water.

      - Unused chemicals after expiry date or short expiry less than 3 months are sent in a black cover to stores for returning or appropriate disposal.

Sewage Treatment Plant (STP) – Central Lab Facility:

      - STP is installed at the laboratory premises to treat the liquid waste (sewage and lab waste).

      - All the liquid waste from the facility is routed to the common sewage tank from there the waste is pumped to the STP.

      - After treatment the treated waste is routed through the sand and carbon filter and then stored in a separate treated water tank.

      - Water from the treated water tank is used for landscaping and gardening purposes.

- Bomb Threat

- Purpose: This procedure outlines information about handling Bomb threat. Scope: This procedure is applied when an emergency like bomb threat occurs. Responsibility:

      - Management

      - Safety committee members

      - All staff

  - Procedure on receiving a Bomb Threat:

      - Although threats can come in almost any form (letters, memos, writing on a wall, e-mail, etc.), most threats come over the telephone. A strict and consistent procedure should be followed. The person who receives the threat must record as much information as possible. Contingency arrangements may allow a call to be traced.

      - Use common sense: if the threat is very specific and a short time is indicated before the bomb will explode, it may be advisable to get everyone out of the building as quickly as possible and let the police do the searching.

      - When an unspecific, less than eminent bomb threat is made, an initial search of the building should usually be conducted in one of three ways.

      - A complete search of the entire premises by management, supervisory, or administrative personnel.

      - A complete search of the entire premises by trained teams developed by management.

      - An overt search of the entire premises by employees in their working area.

      - Each method has its own criteria for speed and thoroughness. This should be weighed against the potential risk involved. A search by employees of their own

      - work areas is the most thorough type of search. A system can be set up for each employee to search his or her area overtly or covertly on a predetermined signal (do not use electronic or radio equipment to signal).

      - NOTE: The local police departments will assist in searching, and may insist on being accompanied by an employee. The Safety Officer is usually designated for this chore, but it could be anyone with good knowledge of the building’s layout.

      - Instructions to the people conducting the search should be clear, but always remind them:

        - Do Not Touch a suspected device - Immediately report a suspected bomb device to the Police!

        - Do Not Assume it is the only device planted in the area.

        - Do Not Change the Environment: Do not turn on or off water, gas or anything with electricity.

      - Searches should cover: The search should be systematic, encompassing the entire room like:

        - Public access areas

        - Evacuation routes

        - Lobbies

        - Restrooms

        - Hallways

        - Stairwells

        - Main office areas

        - Floor-to-waist level

        - Waist-to-chin level

        - Chin-to-ceiling

        - Inside and above false ceilings

        - Note: Searching a false ceiling may be physically impossible. Leave this to a trained search team that has been developed by management.

        - Remember: If a suspected device is located, do not touch.

        - Contact the supervisor in charge and the Police.

Evacuation: During a bomb threat, the decision to evacuate is made by the COO/Chairperson of the safety committee.

      - This decision may be determined by a number of factors: Category of warning, specific or nonspecific Prevalence of bomb threats in the community within a recent time frame, and any previous publicity the possibility of carrying out an effective search without evacuation.

      - You also need to consider how much of an evacuation is in order:

        - Complete evacuation

        - Partial evacuation

        - To an internal area

        - To a safe outside area

        - No evacuation

      - When total evacuation is chosen: It is imperative to search evacuation routes before evacuation is undertaken. Have people taken to an area away from the premises to avoid possibility of being struck by debris. Use the same exit plans as you would for fire alarms with supervisors possibly remaining behind to search the premises.

      - If a suspicious device or package is located, law enforcement will  direct the evacuation.

Letter/Parcel Bomb Detection: At times, the postal service is used to deliver explosive devices. This is a threat that can be minimized by training people to detect if a package might contain explosives.

If You Suspect a Letter or Parcel Might Be Explosive: Do not touch or further handle it. Isolate the area and contact a supervisor for appropriate assistance. Remember, alertness could save your life.

Typical Signs to Watch for in Letter or Package Bombs: Unusual or unexpected point of origin, an indecipherable address or no return address at all Inaccuracies in your address or in titles Unusually restrictive markings that are not a normal part of your business dealings (for example, "personal,” "to be opened only by,” "do not delay delivery”) Excessive weight or thickness for envelope or package size and/or excessive postage Improvised labels or obviously disguised script Unusual odors. The feel of springiness Metallic components or stiffeners in letters, protruding wire, string or metal foil Oily or greasy stains on packaging or excessive wrapping, binding and taping materials Small holes Unbalanced or lopsided letters and parcels.

- Civil Disorder

- Purpose: This Procedure outlines the safety precautions and management of any public disturbance involving acts of violence by a group of three or more persons causing immediate danger, damage, or injury to the property or person of another individual.

- Scope: The scope covers all the personal employed in TRUSTlab labs.

- Responsibility: Management and Employees

Procedure:

      - Alert employees in the area of the situation.

      - Alert the security to close the entrance gates.

      - Lock all doors; secure all files, documents and equipment.

      - Inform to the police.

      - Avoid provoking or obstructing anyone participating in a disturbance or demonstration. Assist the campus police when they arrive by supplying them with all additional information and ask others to do the same.

      - The campus police will assess the situation upon their arrival and conduct any search necessary or disperse demonstrators as necessary.

      - Once the Crowd is dispersed, necessary security guards and police personal are placed till such time it is considered safe for the staff for re-entering into the campus.

- Contingency Plan

In case of shortage of reagents or manpower or breakdown of instrument:

      - All reagents are ordered in a proper planned manner, keeping lead period of supply in account.

      - Staffs are trained as per the job responsibility defined, together with cross training, along with valuation. This would ensure that there is back-up in case of any requirement.

      - Staff roster and leaves are planned in such a way that there is adequate staff available in the laboratory.

      - In case of shortage of reagents or manpower or instrument breakdown, all the samples are outsourced to a NABL accredited lab for processing.

      - Outsourced results are transcribed and reports released.

Lab Equipments:

      - Installed equipment’s are designed to prevent contact between the operator and infectious material.

      - Constructed of materials that are impermeable to liquids, resistant to corrosion and meet structural requirements.

      - Equipment’s are free of burrs and sharp edges.

      - Easy to maintain and operate.

Use of plastic apparatuses is in practice and use of glass apparatuses is minimized.

Notify all clients of laboratories about emergency situation (which is not under control) in the laboratory through Neuberg website and ensure the clients/patients that the laboratory services will be restored as soon as emergency situation is under control.

Processing and reporting of samples is halted for some time and as soon as situation gets better emergency samples processed and reports released to concerned patient.

Disaster Preparedness:

In the event of natural calamities like cyclone/ earthquake, Government Agency issues frequent warning signals through various media to alert on the seriousness. Accordingly, precautionary measures will be taken & action will be taken.

Electrical wiring is carried out in such a way which prevents any electrical accident.

Ambulance and other mode of transport vehicles will be stationed for emergency evacuation.

- Laboratory Safety Training and Information

- Purpose: The purpose of the procedure is to explain the Laboratory safety training and information for staff for the Employees in the Laboratory.

- Responsibility: Lab Director (Lab Ops.) and HODs of the laboratory.

Procedure: The Director (Lab Ops.) and HODs is responsible for ensuring that the staff receives proper training on the lab safety aspects. All the new staff is trained on the lab safety aspects during induction and refresher programs are arranged twice a year.

The contents of the lab safety training program generally are:

      - Details, location, and availability of laboratory safety manual.

      - Pre and Post Prophylaxis for needle stick injuries.

      - General information on physical and health hazard of hazardous chemicals, signs and symptoms of exposure, and measures staff can take to protect themselves.

      - General information on safety equipment and personal protective equipment.

      - Proper use of laboratory bio-safety hoods.

      - Biomedical waste management.

      - Common fire hazard and causes and your role in preventing fires.

      - What to do in case of a fire and what to do if tapped by a fire.

      - Fire protection systems.

      - Civil Disorder

      - Natural Disaster

- Reference

NACO guidelines

CDC guidelines