SOP – 001

PROCEDURE FOR

URINE MICROSCOPIC EXAMINATION

- TABLE OF CONTENTS

TABLE OF CONTENTS	2

APPROVAL AUTHORITY	3

RECORD OF AMENDMENTS	4

DISTRIBUTION LIST	5

LIST OF ABBREVIATIONS	6

1.	Objective	7

2.	Scope	7

3.	Responsibilities	7

4.	Definition	7

5.	Test Principle	7

6.	Specific Performance Characteristics	7

7.	Type of Sample	7

8.	Type of Container & Additives	8

9.	Patient Preparation	8

10.	Equipment & Reagents	8

11.	Safety	8

12.	Calibration Procedure	9

13.	Test Procedure	9

14.	Quality Control Procedure	9

15.	Interferences & Cross Reactions	10

16.	Result Calculation/Interpretation	10

17.	Biological Reference Interval	10

18.	Reportable Interval	10

19.	Procedure when Results are not in Range	10

20.	Critical Alert Values	10

21.	Laboratory Clinical Interpretation	10

22.	Potential Source of Variability	10

23.	References	10

- APPROVAL AUTHORITY

| Prepared by: | Head of Department | Dr. Abdul Aleem Name | Date: 14/11/2024 |
| --- | --- | --- | --- |
|  |  | Signature |  |
| Reviewed & Approved by: | Laboratory Head | Dr. Abdul Aleem Name | Date: 14/11/2024 |
|  |  | Signature |  |
| Issued by: | Quality Manager | Dr. Sai Varun Name | Date: 14/11/2024 |
|  |  | Signature |  |

- RECORD OF AMENDMENTS

| S. No. | Ver. No. | Ver. Date | Rev. No. | Rev. Date | Description of Changes |
| --- | --- | --- | --- | --- | --- |

| Document Copy No. | Name / Designation of the holder of the document copy | Type of Copy | Version No. | Revision No. |
| --- | --- | --- | --- | --- |
| 01/10 | Managing Director | Controlled Copy | 01 | 00 |
| 02/10 |  |  |  |  |
| 03/10 |  |  |  |  |
| 04/10 |  |  |  |  |
| 05/10 |  |  |  |  |
| 06/10 |  |  |  |  |
| 07/10 |  |  |  |  |
| 08/10 |  |  |  |  |
| 09/10 |  |  |  |  |
| 10/10 |  |  |  |  |

- DISTRIBUTION LIST

- LIST OF ABBREVIATIONS

| S. No. | Abbreviations | Expansion |
| --- | --- | --- |
| 1 | °C | Degree Centigrade |
| 2 | CP | Clinical Pathology |
| 3 | CV | Coefficient of Variation |
| 4 | EQAS | External Quality Assurance Scheme |
| 5 | HBV | Hepatitis B Virus |
| 6 | HIV | Human Immunodeficiency Virus |
| 7 | HOD | Head of Department |
| 8 | IQC | Internal Quality Control |
| 9 | MSP | Management System Procedure |
| 10 | N/A | Not Applicable/Available |
| 11 | pH | Potential of Hydrogen |
| 12 | QC | Quality Control |
| 13 | QM | Quality Manager |
| 14 | QNS | Quantity Not Sufficient |
| 15 | SOP | Standard Operating Procedure |
| 16 | TRF | Test Requisition Form |
| 17 | TDPL | TRUSTlab Diagnostics Private Limited |

- Objective

- The purpose of this SOP is to define a procedure for the microscopic examination of urine deposits. This SOP also provides information about the procedural principles, clinical significance and other details.

- Scope

- This procedure applies to all staff of the Clinical Pathology Department tasked with the duty of performing these tests.

- Responsibilities

It is the responsibility of the technicians of the Clinical Pathology Department to comply with this SOP.

HOD shall ensure implementation.

The Quality Manager is responsible for monitoring, reviewing and ensuring compliance with this procedure.

All other TRUSTlab staff involved in this process (including contract personnel) are responsible for complying with this procedure.

- Definition

- Microscopic urine examination, a part of a urinalysis, involves looking at urine under a microscope to identify and count various components like cells, casts, crystals, and microorganisms.

- Test Principle

- The insoluble solid particle in suspension that is present in urine is separated by centrifugation. The sediment is then used for microscopic examination.

- Specific Performance Characteristics

- N/A.

- Type of Sample

Urine is the primary specimen for the test.

Collect a minimum 15 ml of a random sample of urine.

Process the samples within 1 hour of collection. If delay is anticipated instruct patients to store the sample at 2-8°C in the refrigerator for a maximum of up to 4 hours until transport to the laboratory.

Do not accept if the sample volume is inadequate or if samples are received without an ID number.

- Type of Container & Additives

- Sterile screw-capped container.

- Patient Preparation

Instruct patients to collect urine samples in a clean dry container (preferably one provided by TRUSTlab).

Instruct patients to void urine directly into the container and while collecting allow the first portion of urine to escape.

- Equipment & Reagents

Equipment

      - Centrifuge

      - Microscope

      - Cover Slips

      - Slide

      - Test tubes

Reagents

- N/A.

- Safety

Follow universal precautions like wearing suitable gloves and eye/face protection while handling all reagents, samples and operating the instrument.

Universal precautions imply that all body fluids are treated as if known to be infectious for Human Immunodeficiency Virus (HIV), Hepatitis B Virus (HBV) or other Blood borne Pathogens.

Dispose of all waste in accordance with applicable national and/or local regulations.

Some reagents contain sodium azide, which may react with copper or lead plumbing to form explosive metal azides. Use caution in disposing of these reagents. If disposing to drain, flush with large volumes of water to prevent azide buildup.

Waste material containing patient samples or biological products should be considered biohazardous when disposing or treating.

Clean up all spills immediately and thoroughly. Disinfect the area for any spills involving biohazardous materials. Dispose of all contaminated materials appropriately.

- Calibration Procedure

- N/A.

- Test Procedure

Precautions:

      - A thin preparation under the cover slip without any air bubble should be made.

      - The specimen should be mixed well before the sample is taken for microscopic examined.

Procedure:

      - Take 3–5 ml of urine in a test tube and centrifuge at 3000 rpm for 5 minutes.

      - Decant the supernatant and re-suspend the sediment in the remaining urine drops.

      - Place 1 drop of the re-suspended sediment on a glass slide, and cover it with a cover slip, ensuring no air bubbles are trapped.

      - Examine under the microscope, using 10X objective first, and 45X objective next, in at least 10 fields.

      - Take the average and express the result in numbers per high power field (HPF).

      - Count the numbers of pus cells, RBCs, Epithelial cells and casts if present, and enter the findings.

      - Also look for crystals, bacteria and mucus threads.

- Quality Control Procedure

Internal Quality Control

- As defined in the procedure Urine IQC (TDPL/CP/SOP-002).

External Quality Control

- EQAS: Metropolis EQAS

- Interferences & Cross Reactions

- N/A.

- Result Calculation/Interpretation

The presence of casts indicates renal disorder and is associated with proteinuria.

Uniform turbidity of the urine is due to the presence of pus cells and bacteria.

Presence of unorganized elements of urinary sediments.

Increased number of WBC’s indicates renal infection.

Hematuria: If RBCs are present in the urine.

Increased number of epithelial cells indicates renal degeneration.

- Biological Reference Interval

- Negative.

- Reportable Interval

- N/A.

- Procedure when Results are not in Range

- N/A.

- Critical Alert Values

- N/A.

- Laboratory Clinical Interpretation

A microscopic examination of urinary sediment for pus cells, bacteria and casts is often used to correlate a positive test for urine protein.

This examination aids in laboratory diagnosis of renal and other disorders.

- Potential Source of Variability

- N/A.

- References

      - Methods and interpretations-MLT by Ramanik Sood 5th edition Page 62-68

      - Procedure for Managing Internal QC Program (TDPL/QA/MAN-001: MSP-013)

      - NABL 112