SOP – 003

PROCEDURE FOR

TITRATION, AVIDITY AND SPECIFICITY OF ANTIBODY REAGENTS

- TABLE OF CONTENTS

TABLE OF CONTENTS	2

APPROVAL AUTHORITY	3

RECORD OF AMENDMENTS	4

DISTRIBUTION LIST	5

LIST OF ABBREVIATIONS	6

1.	Objective	7

2.	Scope	7

3.	Responsibilities	7

4.	Definition	7

5.	Test Principle	7

6.	Specific Performance Characteristics	7

7.	Type of Sample	8

8.	Type of Container & Additives	8

9.	Patient Preparation	8

10.	Equipment & Reagents	8

11.	Safety	8

12.	Calibration Procedure	9

13.	Procedure	9

14.	Quality Control Procedure	11

15.	Interferences & Cross Reactions	11

16.	Result Calculation	11

17.	Biological Reference Interval	11

18.	Reportable Interval	11

19.	Procedure when Results are not in Range	11

20.	Critical Alert Values	11

21.	Laboratory Clinical Interpretation	11

22.	Potential Source of Variability	11

23.	References	11

Appendix 1: Titration of Antibody Reagent in Blood Grouping Worksheet	12

Appendix 2: Avidity of Antibody Reagent in Blood Grouping Worksheet	12

Appendix 3: Specificity of Antibody Reagents in Blood Grouping	12

- APPROVAL AUTHORITY

| Prepared by: | Head of Department | Dr. Abdul Aleem Name | Date: 14/11/2024 |
| --- | --- | --- | --- |
|  |  | Signature |  |
| Reviewed & Approved by: | Laboratory Head | Dr. Abdul Aleem Name | Date: 14/11/2024 |
|  |  | Signature |  |
| Issued by: | Quality Manager | Dr. Sai Varun Name | Date: 14/11/2024 |
|  |  | Signature |  |

- RECORD OF AMENDMENTS

| S. No. | Ver. No. | Ver. Date | Rev. No. | Rev. Date | Description of Changes |
| --- | --- | --- | --- | --- | --- |

| Document Copy No. | Name / Designation of the holder of the document copy | Type of Copy | Version No. | Revision No. |
| --- | --- | --- | --- | --- |
| 01/10 | Managing Director | Controlled Copy | 01 | 00 |
| 02/10 |  |  |  |  |
| 03/10 |  |  |  |  |
| 04/10 |  |  |  |  |
| 05/10 |  |  |  |  |
| 06/10 |  |  |  |  |
| 07/10 |  |  |  |  |
| 08/10 |  |  |  |  |
| 09/10 |  |  |  |  |
| 10/10 |  |  |  |  |

- DISTRIBUTION LIST

- LIST OF ABBREVIATIONS

| S. No. | Abbreviations | Expansion |
| --- | --- | --- |
| 1 | AHG | Anti-Human Globulin |
| 2 | AMR | Analytical Measurement Range |
| 3 | °C | Degree Centigrade |
| 4 | CV | Coefficient of Variation |
| 5 | CRR | Clinical Reportable Range |
| 6 | EDTA | Ethylenediamine Tetra-acetic Acid |
| 7 | EQAS | External Quality Assurance Scheme |
| 8 | HOD | Head of Department |
| 9 | HBV | Hepatitis B Virus |
| 10 | HIV | Human Immunodeficiency Virus |
| 11 | HM | Haematology |
| 12 | IgG | Immunoglobulin G |
| 13 | IgM | Immunoglobulin M |
| 14 | MSP | Management System Procedure |
| 15 | N/A | Not Applicable/Available |
| 16 | NS | Normal Saline |
| 17 | pH | Potential of Hydrogen |
| 18 | QC | Quality Control |
| 19 | QM | Quality Manager |
| 20 | QNS | Quantity Not Sufficient |
| 21 | rpm | Revolutions per minute |
| 22 | SD | Standard Deviation |
| 23 | SOP | Standard Operating Procedure |
| 24 | TRF | Test Requisition Form |
| 25 | TDPL | TRUSTlab Diagnostics Private Limited |

- Objective

- The purpose of this SOP is to describe the procedure for testing the quality of ABO reagents for potency (avidity and titration) and specificity. This SOP also provides information about the test principle, clinical significance and other details.

- Scope

- This procedure applies to all staff of the Haematology Department tasked with the duty of performing these tests.

- Responsibilities

It is the responsibility of the technicians of the Haematology Department to comply with this SOP.

HOD shall ensure implementation.

The Quality Manager is responsible for monitoring, reviewing and ensuring compliance with this procedure.

All other TRUSTlab staff involved in this process (including contract personnel) are responsible for complying with this procedure.

- Definition

- Titration: This term refers to determining the amount of an antibody that can bind to an antigen.

- Avidity: This is the total binding strength between a multivalent antibody and multiple antigenic epitopes, essentially the overall strength of the interaction. Avidity can be measured using methods like titration, where serial dilutions of an antibody are tested against an antigen, and the avidity index (AI) can be calculated by comparing the results with and without a denaturant.

- Test Principle

- The test is based on the antigen-antibody reaction depending on the presence of the antigen on red cells and antibodies in the anti-sera reagent. The test is performed by tube agglutination method.

- Specific Performance Characteristics

- N/A.

- Type of Sample

- Known pooled cells of A, B & O-Positive Blood groups.

- Type of Container & Additives

- K2 EDTA anticoagulant vacutainer.

- Patient Preparation

- N/A.

- Equipment & Reagents

Equipment

      - Gloves

      - Droppers

      - Test tube rack

      - Centrifuge

Reagents & others

      - Anti-A anti-sera

      - Anti-B anti-sera

      - Anti-D anti-sera (IgM)

      - Anti-D anti-sera (IgG)

      - Anti Human globulin

      - Pooled A1 red cells

      - Pooled B red cells

      - Pooled O-Positive red cells

- Safety

Follow universal precautions, like wearing suitable gloves and eye/face protection, while handling all reagents, samples and operating the instrument.

Universal precautions imply that all body fluids are treated as if known to be infectious for Human Immunodeficiency Virus (HIV), Hepatitis B Virus (HBV) or other Blood borne Pathogens.

Dispose of all waste in accordance with applicable national and/or local regulations.

Some reagents contain sodium azide, which may react with copper or lead plumbing to form explosive metal azides. Use caution in disposing of these reagents. If disposing to drain, flush with large volumes of water to prevent azide buildup.

Waste material containing patient samples or biological products should be considered biohazardous when disposing or treating.

Clean up all spills immediately and thoroughly. Disinfect the area for any spills involving biohazardous materials. Dispose of all contaminated materials appropriately.

- Calibration Procedure

- N/A.

- Procedure

Titration of the Antibody Reagent

      - Take antibody reagent A and A pooled cells.

      - The reagent should be clear on visual inspection.

      - Take 12 test tubes labelled as 1/1, 1/2, 1/4, 1/8, 1/16, 1/32, 1/64, 1/128, 1/512, 1/1024 and place them in test tube rack in sequential order.

      - Take 100 µl of normal saline in each test tube from the labelled as ½ onwards.

      - Add 100 µl of Anti-A sera in 1/1 and 1/2 test tube and mix.

      - Transfer 100 µl of mixed sample from tube labelled as 1/2 to tube labelled as 1/4.

      - Repeat the same procedure of transferring 100 µl from adjacent tubes to next tubes till last tube.

      - Transfer 100 µl from the last tube to one glass tube and preserve it for further testing.

      - Add 50 µl of 5% pooled A cells in all tubes.

      - Centrifuge all the tubes at 1000 rpm for 1 minute.

      - Look for clumping grossly and note down the titration value of reagent on tube just before 1+ clumping.

      - Repeat the same procedure for antibody reagent B with pooled B cells, for anti D (IgM), D (IgG) & AHG with pooled O Rh D (Positive) cells.

      - Data is entered in the formats given in Appendix 1 for titration value of reagents.

    - Avidity of the Antibody Reagent

      - Prepare 50% pooled cells of A, B & O.

      - Take 100 µL of sediment of red cells Add 3 ml of normal saline and mix well.

      - Centrifuge at 2000 rpm for 5 minutes. Decant the supernatant.

      - To the remaining cell button repeat steps 2, 3 and 4, thrice.

      - After the final wash to prepare 50% suspension of red cells, add 1-1.5 ml of normal saline until the solution turns to dark red color.

      - Take 3 clean slides labelled as A, B and D.

      - On slide labelled as A, place 50 µl of pooled cells and 50 µl of anti A reagent side by side.

      - Set ready a timer and start while mixing.

      - Stop the timer when visible agglutination is seen and note down the time taken to form agglutination.

      - Repeat the same procedure for other reagents with respective pooled cells.

      - Refer to Appendix 2 for the avidity of reagents used.

    - Specificity testing of the antibody reagent

      - 2-5% of pooled red cell suspension is to be used for this test.

      - Take 3 test tubes for each antibody reagent.

      - Label these tubes as A1, B and O respectively.

      - Add 1 drop of A1, B and O pooled cells in tubes labelled as A1, B and O respectively.

      - Add 1 drop of Anti-A reagent in each tube.

      - Mix each tube by gentle shaking.

      - Centrifuge the tubes at 1000 rpm for 1 minute.

      - Remove the tubes from the centrifuge.

      - Re-suspend the red cells button.

      - Read macroscopically for agglutination.

      - Grade and record test results.

      - There should be reaction with corresponding antisera only, not with others.

      - Minimum 2+ reactions should be given by antisera with corresponding cell.

      - Interpret results and compare to previous reports.

      - Repeat the same for other reagents as well.

      - Please refer to Appendix 4 for specificity of reagents.

- Quality Control Procedure

- N/A.

- Interferences & Cross Reactions

- N/A.

- Result Calculation

- N/A.

- Biological Reference Interval

- N/A.

- Reportable Interval

- N/A.

- Procedure when Results are not in Range

- N/A.

- Critical Alert Values

- N/A.

- Laboratory Clinical Interpretation

- N/A.

- Potential Source of Variability

- N/A.

- References

      - Dr. S.K. Saran. Transfusion medicine, technical manual 2nd edition.

- Appendix 1: Titration of Antibody Reagent in Blood Grouping Worksheet

| Date | Tube with different Titration | Lot. No | 1/1 | 1/2 | 1/4 | 1/8 | 1/16 | 1/32 | 1/64 | 1/128 | 1/256 | 1/512 | 1/1024 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Anti-A |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Anti-B |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Anti-D |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Anti-AB |  |  |  |  |  |  |  |  |  |  |  |  |
|  | AHG |  |  |  |  |  |  |  |  |  |  |  |  |

- Appendix 2: Avidity of Antibody Reagent in Blood Grouping Worksheet

| - Date | - Antibody Reagent | - Company | - Lot # | - Expiry Date | - Time | - Performed by | - Reviewed by | - Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Appendix 3: Specificity of Antibody Reagents in Blood Grouping

| Date/Time | Appearance |  |  |  |  |  |  |  |  |  |  |  | Performed by | Reviewed by | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Anti-A |  |  | Anti-B |  |  | Anti-D IgM | Anti-D IgG | Anti-A1 |  |  | Anti-H |  |  |  |
|  | A | B | O | A | B | O | O | O | A | B | O | O |  |  |  |