SOP – 004

PROCEDURE FOR

DIRECT COOMBS TEST – TUBE METHOD

- TABLE OF CONTENTS

TABLE OF CONTENTS	2

APPROVAL AUTHORITY	3

RECORD OF AMENDMENTS	4

DISTRIBUTION LIST	5

LIST OF ABBREVIATIONS	6

1.	Objective	7

2.	Scope	7

3.	Responsibilities	7

4.	Definition	7

5.	Test Principle	7

6.	Specific Performance Characteristics	7

7.	Type of Sample	8

8.	Type of Container & Additives	8

9.	Patient Preparation	9

10.	Equipment & Reagents	9

11.	Safety	9

12.	Calibration Procedure	10

13.	Test Procedure	10

14.	Quality Control Procedure	10

15.	Interferences & Cross Reactions	11

16.	Result Calculation	11

17.	Biological Reference Interval	11

18.	Reportable Interval	11

19.	Procedure when Results are not in Range	11

20.	Critical Alert Values	11

21.	Laboratory Clinical Interpretation	11

22.	Potential Source of Variability	12

23.	References	12

- APPROVAL AUTHORITY

| Prepared by: | Head of Department | Dr. Abdul Aleem Name | Date: 14/11/2024 |
| --- | --- | --- | --- |
|  |  | Signature |  |
| Reviewed & Approved by: | Laboratory Head | Dr. Abdul Aleem Name | Date: 14/11/2024 |
|  |  | Signature |  |
| Issued by: | Quality Manager | Dr. Sai Varun Name | Date: 14/11/2024 |
|  |  | Signature |  |

- RECORD OF AMENDMENTS

| S. No. | Ver. No. | Ver. Date | Rev. No. | Rev. Date | Description of Changes |
| --- | --- | --- | --- | --- | --- |

| Document Copy No. | Name / Designation of the holder of the document copy | Type of Copy | Version No. | Revision No. |
| --- | --- | --- | --- | --- |
| 01/10 | Managing Director | Controlled Copy | 01 | 00 |
| 02/10 |  |  |  |  |
| 03/10 |  |  |  |  |
| 04/10 |  |  |  |  |
| 05/10 |  |  |  |  |
| 06/10 |  |  |  |  |
| 07/10 |  |  |  |  |
| 08/10 |  |  |  |  |
| 09/10 |  |  |  |  |
| 10/10 |  |  |  |  |

- DISTRIBUTION LIST

- LIST OF ABBREVIATIONS

| S. No. | Abbreviations | Expansion |
| --- | --- | --- |
| 1 | AHG | Anti-Human Globulin |
| 2 | °C | Degree Centigrade |
| 3 | CV | Coefficient of Variation |
| 4 | EDTA | Ethylenediamine Tetra-acetic Acid |
| 5 | EQAS | External Quality Assurance Scheme |
| 6 | HOD | Head of Department |
| 7 | HM | Haematology |
| 8 | IgG | Immunoglobulin G |
| 9 | IQC | Internal Quality Control |
| 10 | LISS | Low Ionic Strength Salt Solution |
| 11 | MSP | Management System Procedure |
| 12 | N/A | Not Applicable/Available |
| 13 | NS | Normal Saline |
| 14 | pH | Potential of Hydrogen |
| 15 | QC | Quality Control |
| 16 | QM | Quality Manager |
| 17 | QNS | Quantity Not Sufficient |
| 18 | RBC | Red Blood Cells |
| 19 | rpm | Revolutions per minute |
| 20 | SOP | Standard Operating Procedure |
| 21 | TRF | Test Requisition Form |
| 22 | TDPL | TRUSTlab Diagnostics Private Limited |

- Objective

- The purpose of this SOP is to describe the procedure for performing the Direct Coombs Test to detect antibodies on the surface of patient blood cells. This SOP also provides information about the test principle, clinical significance and other details.

- Scope

- This procedure applies to all staff of the Haematology Department tasked with the duty of performing these tests.

- Responsibilities

It is the responsibility of the technicians of the Haematology Department to comply with this SOP.

HOD shall ensure implementation.

The Quality Manager is responsible for monitoring, reviewing and ensuring compliance with this procedure.

All other TRUSTlab staff involved in this process (including contract personnel) are responsible for complying with this procedure.

- Definition

- The Direct Coombs test, also known as the Direct Antiglobulin Test (DAT), is a blood test used to detect antibodies or complement proteins that are bound to the surface of red blood cells (RBCs). These antibodies can be a sign of certain diseases or drug reactions that cause the body to attack its own RBCs, potentially leading to hemolytic anemia. A positive result indicates that antibodies or complement are present on the RBCs, suggesting a possible cause for anemia or other blood-related issues.

- Test Principle

The red cells sensitized to the incomplete antibody, when subjected to the anti-Human globulin causes agglutination. This test is performed on the washed cells of the patient. Tube agglutination method.

- Specific Performance Characteristics

- N/A.

- Type of Sample

A minimum of 3 ml of whole blood

      - Collect the blood sample by venipuncture observing universal precautions.

      - Allow samples to clot adequately before centrifugation.

      - Keep tubes stoppered and upright at all times.

      - Do not use samples that have been stored at room temperature for longer than 8 hours.

      - Tightly cap and refrigerate specimens at 2 to 8°C if the assay is not completed within 8 hours.

      - Freeze samples at or below -20°C if the sample is not assayed within 48 hours.

      - Freeze samples only once and mix thoroughly after thawing.

      - Before placing samples on the system ensure that:

        - Samples are free of fibrin or other particulate matter.

        - Samples are free of bubbles.

Sample Stability: 48 hours at 2 – 8℃.

Sample Retention: Sample is retained for 24 hours at 2 – 8℃.

Sample Rejection Criteria:

      - Quantity not sufficient (QNS)

      - Sample received in wrong vacutainer

      - No TRF; Grossly incomplete TRF; Errors in labeling

      - Grossly haemolysed specimens

      - Highly lipemic specimens

      - Highly icteric specimens

      - Sample received beyond stability period

      - Incomplete TRF details or labelling errors are clarified with Customer/Doctor/ Patient before sample is accepted for processing. If critical information is still not obtained, the sample is rejected.

- Type of Container & Additives

- K2 EDTA anticoagulant vacutainer.

- Patient Preparation

The phlebotomists should be courteous and compassionate towards the patients and their attendants. Ask the patient to sit comfortably. Maintain calmness and silence during blood drawing to avoid mistakes.

Check the patient’s identification; ask the patient for his or her full name, address, identification number, and/or date of birth.

No specific preparation is required.

- Equipment & Reagents

Equipment

      - Test tubes 12x5mm

      - Pipettes, Droppers

      - Calibrated Centrifuge

      - Microscope

Reagents

      - LISS (Low ionic strength salt solution)

      - Coombs reagent (Anti human globulin-AHG)

      - Negative control – Pooled O cells 2-5% red cell suspension

      - Positive control – Washed O positive cells (2-5% red cell suspension) sensitized to diluted Anti-D (IgG) anti sera. When these cells are seen under a microscope there should be no agglutination. However, with addition of AHG, there is 2+ clumping.

- Safety

Follow universal precautions, like wearing suitable gloves and eye/face protection, while handling all reagents, samples and operating the instrument.

Universal precautions imply that all body fluids are treated as if known to be infectious for Human Immunodeficiency Virus (HIV), Hepatitis B Virus (HBV) or other Blood borne Pathogens.

Dispose of all waste in accordance with applicable national and/or local regulations.

Some reagents contain sodium azide, which may react with copper or lead plumbing to form explosive metal azides. Use caution in disposing of these reagents. If disposing to drain, flush with large volumes of water to prevent azide buildup.

Waste material containing patient samples or biological products should be considered biohazardous when disposing or treating.

Clean up all spills immediately and thoroughly. Disinfect the area for any spills involving biohazardous materials. Dispose of all contaminated materials appropriately.

- Calibration Procedure

- N/A.

- Test Procedure

      - Take patient sample and prepare the patient washed cells (2-5% red cell suspension).

      - Take three test tubes labeled as test, positive and negative.

      - To the tube labeled as test add two drops of prepared patient cells suspension

      - To the tube labeled as Negative add two drops of negative control cell suspension

      - To the tube labeled as Positive add two drops of positive control cell suspension

      - Add two drops of Coombs serum (Anti Human Globulin) to all the three tubes.

      - Incubate all for 15 minutes at 37°C.

      - Centrifuge at 1000 rpm for 1 minute, if necessary.

      - Then observe for agglutination macroscopically and if negative examine under the microscope.

- Manual Method:

      - Prepare 5% suspension of red cells in isotonic saline.

      - Add two drops of AHG and mix well.

      - Centrifuge for 1 min for 1000 rpm.

      - Very gently resuspend the cell button and observe for agglutination microscopically.

- Quality Control Procedure

Internal Quality Control

    - Procedure:

        - Negative control – Pooled O cells 2-5% red cell suspension

        - Positive control – Washed O positive cells (2-5% red cell suspension) sensitized to diluted Anti-D (IgG) anti sera.

        - When these cells are seen under a microscope there should be no agglutination.

        - However, with addition of AHG, there is 2+ clumping.

External Quality Control

- EQAS participation –- CMC EQAS

- Interferences & Cross Reactions

- N/A.

- Result Calculation

- N/A.

- Biological Reference Interval

- Negative

- Reportable Interval

- Negative, Positive (+ to 4+), Mixed field.

- Procedure when Results are not in Range

- N/A.

- Critical Alert Values

- N/A.

- Laboratory Clinical Interpretation

- Scoring of results in red cell agglutination tests

| Symbol | Agglutination score | Description |
| --- | --- | --- |
| 4+ or C (complete) | 12 | Cell button remains in 1 clump |
| 3+ | 10 | Cell button dislodges into several large clumps. Macroscopically visible. |
| 2+ | 8 | Cell button dislodges into many small clumps. Macroscopically visible. |
| 1+ | 5 | Cell button dislodges into finely granular clumps. Macroscopically just visible. |
| (+) or w (weak) | 3 | Cell button dislodges into fine granules. Only visible microscopically |
| (-) | 0 | Negative results: All cells are free and evenly distributed |

- Potential Source of Variability

- N/A.

- References

      - Practical Hematology, Dacie and Lewis, Tenth edition.

      - Procedure for Managing Internal QC Program (TDPL/QA/MAN-001: MSP-013).