SOP – 001

PROCEDURE FOR

FIXATION OF TISSUE

- TABLE OF CONTENTS

TABLE OF CONTENTS	2

APPROVAL AUTHORITY	3

RECORD OF AMENDMENTS	4

DISTRIBUTION LIST	5

LIST OF ABBREVIATIONS	6

1.	Objective	7

2.	Scope	7

3.	Responsibilities	7

4.	Definition	7

5.	Test Principle	7

6.	Specific Performance Characteristics	7

7.	Type of Sample	8

8.	Type of Container & Additives	8

9.	Patient Preparation	8

10.	Equipment & Reagents	8

11.	Safety	9

12.	Calibration	9

13.	Test Procedure	10

14.	Quality Control Procedure	11

15.	Interferences & Cross Reactions	11

16.	Result Calculation	12

17.	Biological Reference Interval	12

18.	Reportable Interval	12

19.	Troubleshooting	12

20.	Critical Alert Values	12

21.	Laboratory Clinical Interpretation	12

22.	Potential Source of Variability	12

23.	References	12

- APPROVAL AUTHORITY

| Prepared by: | Head of Department | Dr. Abdul Aleem Name | Date: 14/11/2024 |
| --- | --- | --- | --- |
|  |  | Signature |  |
| Reviewed & Approved by: | Laboratory Head | Dr. Abdul Aleem Name | Date: 14/11/2024 |
|  |  | Signature |  |
| Issued by: | Quality Manager | Dr. Sai Varun Name | Date: 14/11/2024 |
|  |  | Signature |  |

- RECORD OF AMENDMENTS

| S. No. | Ver. No. | Ver. Date | Rev. No. | Rev. Date | Description of Changes |
| --- | --- | --- | --- | --- | --- |

| Document Copy No. | Name / Designation of the holder of the document copy | Type of Copy | Version No. | Revision No. |
| --- | --- | --- | --- | --- |
| 01/10 | Managing Director | Controlled Copy | 01 | 00 |
| 02/10 |  |  |  |  |
| 03/10 |  |  |  |  |
| 04/10 |  |  |  |  |
| 05/10 |  |  |  |  |
| 06/10 |  |  |  |  |
| 07/10 |  |  |  |  |
| 08/10 |  |  |  |  |
| 09/10 |  |  |  |  |
| 10/10 |  |  |  |  |

- DISTRIBUTION LIST

- LIST OF ABBREVIATIONS

| S. No. | Abbreviations | Expansion |
| --- | --- | --- |
| 1 | AMR | Analytical Measurement Range |
| 2 | °C | Degree Centigrade |
| 3 | CRR | Clinical Reportable Range |
| 4 | EQAS | External Quality Assurance Scheme |
| 5 | HOD | Head of Department |
| 6 | IQC | Internal Quality Control |
| 7 | LOD | Limit of Detection |
| 8 | LOQ | Limit of Quantitation |
| 9 | MSP | Management System Procedure |
| 10 | NABL | National Accreditation Board for Testing and Calibration Laboratories |
| 11 | N/A | Not Applicable/Available |
| 12 | pH | Potential of Hydrogen |
| 13 | QC | Quality Control |
| 14 | QM | Quality Manager |
| 15 | SOP | Standard Operating Procedure |
| 16 | TRF | Test Requisition Form |
| 17 | TDPL | TRUSTlab Diagnostics Private Limited |

- Objective

Fixation of tissues is an essential prerequisite for successful processing, cutting and staining of tissues. This SOP addresses the process of fixation of tissues in a way to preserve them in as life-like a manner as possible and prevent autolytic changes. Main objective is to maintain clear and consistent morphological features of the excised tissues.

This also addresses decalcification of bony tissues before they are subjected to the routine formalin fixation and processing.

- Scope

- This procedure applies to all staff of the Histopathology Department of TRUSTlab, tasked with the duty of performing these tests.

- Responsibilities

It is the responsibility of the technicians of the Histopathology Department to comply with this SOP.

HOD shall ensure implementation.

The Quality Manager is responsible for monitoring, reviewing and ensuring compliance with this procedure.

Any other TRUSTlab staff involved in this process (including contract personnel) are responsible for complying with this procedure.

- Definition

- Fixation: This is a process by which the constituents of the cells and therefore of the tissues are fixed in a physical and partly also in a chemical state so that they will withstand the subsequent treatment with various reagents with a minimum loss of normal architecture, or any significant distortion/autolysis.

- Test Principle

N/A.

- Specific Performance Characteristics

Uncertainty of measurement: NA

Accuracy: NA

Limit of quantitation (LOQ): NA

Limit of detection (LOD): NA

Analytical measurement range (AMR): NA

Clinical reportable range (CRR): NA

Sensitivity: NA

Specificity: NA

- Type of Sample

Specimens received at TRUSTlab.

Sample Retention: Sample is retained for 30 days.

Sample Rejection Criteria:

      - Wrongly labeled specimen

      - No tissue in the container

      - No fixative in the specimen container

- Type of Container & Additives

- Formalin-fixed specimen in sealed containers.

- Patient Preparation

N/A.

- Equipment & Reagents

Equipment

- Electronic weighing balance.

Reagents

      - Formalin

      - Disodium hydrogen ortho phosphate

      - Sodium dihydrogen ortho phosphate

      - Picric Acid

      - Glacial acetic acid

Preparation of Reagents

      - 10 % Buffered Neutral Formalin

- For the preparation of 5 liters of 10 % buffered neutral formalin:

        - Initially measure 32.5gm of disodium hydrogen ortho phosphate and 20gm of sodium di hydrogen ortho phosphate.

        - Add them to 4500 ml of distilled/de-ionized water and mix well.

        - To this, add 500ml of concentrated formalin and mix well. Adjust the pH to 6.8 - 7.2.

        - Details of volume and pH of formalin prepared are maintained in the Consumption Sheet on MS Teams. (Preparation of 10% NBF formalin)

      - Bouin’s Fluid

        - Saturated aqueous picric acid	75 ml

        - 40% Formalin                             	25 ml

        - Glacial acetic acid                          5 ml

Reagent Storage Condition Stability

- All reagents are stable at Room Temperature until the expiry date mentioned on the reagent package.

Additional Requirements

- Plastic Containers

- Safety

Wear gloves while handling specimens or reagents. Do not eat, drink, smoke, apply cosmetics, or handle contact lenses in areas where these materials are handled.

Always, open the specimen containers under fume hood to prevent formalin fumes from entering the work atmosphere

Follow all the standard universal precautions and biosafety measures as given in SOP.

- Calibration

Calibration Schedule

- The electronic Weighing Balance is calibrated once a year by an NABL accredited external agency.

- Test Procedure

Decalcification

      - Definition: In order to obtain satisfactory paraffin sections of bone or any other heavily mineralized tissue it is essential to remove the minerals and soften the tissue. The process of demineralization of tissues with acids or chelating agents especially calcium is decalcification/ demineralization.

      - Decalcifying Agent: 5% Nitric Acid (Aqueous Nitric Acid)

        - Nitric Acid:                     05 ml

        - Distilled water               95 ml

      - Before decalcification, the tissue should be fixed properly in 10 % buffered formalin.

      - The volume of the nitric acid should be at least 20 times the volume of the tissue section and   the section should be immersed completely in the acid.

      - Assessment of completion of decalcification

- Assessment of the completion can be ascertained by probing the tissue block with a needle, knife or fingernail in order to detect any gritty fragments of calcium.

      - Neutralization of tissue following acid decalcification

- Adequate water rinsing of decalcified tissue for 30 minutes (for small biopsies) and 1- 4 hours (for larger tissues).

Formalin Fixation

      - Formalin Fixation is the choice of fixation for all routine biopsies in TRUSTlab’s Histopathology department.

      - The tissue specimens after receipt in the laboratory are checked for patient identification with the labeling on the specimens, apart from registration number history etc.

      - The container is checked for the presence or absence of formalin along with the tissue, and if absent, it is documented.

      - If formalin is absent in container, it is replaced with 10% buffered neutral formalin prepared in the department.

      - Gross appearance of medium and large tissues is noted down.

      - All hollow specimens with a lumen are cut open at this stage for proper fixation.

      - Large solid specimens are serially sliced and packed with absorbent cotton in between for better penetration of the fixative. The specimen is then kept for fixation in adequate fixing fluid.

      - Duration:

        - 2 - 6 hours for small biopsies.

        - 6 to 24 hours for larger biopsies after cutting them open or slicing them.

      - Amount of Fixative: Should be approximately 10 times the volume of the specimen.

Bouin’s Fluid Fixation

      - This reagent of fixation is used for small tissue biopsies, mainly for testicular biopsy.

      - Duration: Variable from a few hours up to 18 hours.

- Quality Control Procedure

Internal Quality Control

- The Specimen Collection, Storage & Transportation Manual (TDPL/QA/MAN-003) provides the information on specimen handling and transport of specimens to the laboratory.

- Once the sample reaches the NRL, technicians check the specimen containers for the presence of the fixative.

- If the fixative is inadequate or is absent in the container, fresh fixative is added, as required.

- Quality of fixation is also checked by the pathologist at the time of reporting the slides.

- For all compromised specimen (partially autolysed tissue), a note is added in the final report, stating the issue.

External Quality Control

- EQAS participation –- RML EQAS

- Interferences & Cross Reactions

- N/A.

- Result Calculation

- N/A.

- Biological Reference Interval

- N/A.

- Reportable Interval

- N/A

- Troubleshooting

The yellow color of picric acid can be removed with changes in 50 and 70 % alcohol

Excess picrate formed can be removed easily after dewaxing the sections.

If large specimens are not sliced the deeper portions may be autolysed.

The details for the same are mentioned in the instructions for sample collection for histopathology in the SOP of Primary sample collection.

- Critical Alert Values

- N/A.

- Laboratory Clinical Interpretation

- N/A.

- Potential Source of Variability

- N/A.

- References

      - Bancroft JD, Gamble M: Theory and Practice of Histological Techniques, 5th edition 2002, Churchill Livingstone U.K., 85-108.

      - K. Lakshminarayanan: Histological Techniques - A Practical Manual, Bhalani Publishing House, Mumbai, 22-31.

      - Procedure for Managing Internal QC Program (TDPL/QA/MAN-001: MSP-013).