SOP – 004

PROCEDURE FOR

HEMATOXYLIN & EOSIN STAINING

- TABLE OF CONTENTS

TABLE OF CONTENTS	2

APPROVAL AUTHORITY	3

RECORD OF AMENDMENTS	4

DISTRIBUTION LIST	5

LIST OF ABBREVIATIONS	6

1.	Objective	7

2.	Scope	7

3.	Responsibilities	7

4.	Definitions	7

5.	Principle	7

6.	Specific Performance Characteristics	8

7.	Type of Sample	8

8.	Type of Container & Additives	8

9.	Patient Preparation	9

10.	Equipment & Reagents	9

11.	Safety	10

12.	Calibration	10

13.	Procedure	10

14.	Quality Control Procedure	12

15.	Interferences & Cross Reactions	13

16.	Result Calculation	13

17.	Biological Reference Interval	13

18.	Reportable Interval	13

19.	Troubleshooting	13

20.	Critical Alert Values	14

21.	Laboratory Clinical Interpretation	14

22.	Potential Source of Variability	14

23.	References	14

- APPROVAL AUTHORITY

| Prepared by: | Head of Department | Dr. Abdul Aleem Name | Date: 14/11/2024 |
| --- | --- | --- | --- |
|  |  | Signature |  |
| Reviewed & Approved by: | Laboratory Head | Dr. Abdul Aleem Name | Date: 14/11/2024 |
|  |  | Signature |  |
| Issued by: | Quality Manager | Dr. Sai Varun Name | Date: 14/11/2024 |
|  |  | Signature |  |

- RECORD OF AMENDMENTS

| S. No. | Ver. No. | Ver. Date | Rev. No. | Rev. Date | Description of Changes |
| --- | --- | --- | --- | --- | --- |

| Document Copy No. | Name / Designation of the holder of the document copy | Type of Copy | Version No. | Revision No. |
| --- | --- | --- | --- | --- |
| 01/10 | Managing Director | Controlled Copy | 01 | 00 |
| 02/10 |  |  |  |  |
| 03/10 |  |  |  |  |
| 04/10 |  |  |  |  |
| 05/10 |  |  |  |  |
| 06/10 |  |  |  |  |
| 07/10 |  |  |  |  |
| 08/10 |  |  |  |  |
| 09/10 |  |  |  |  |
| 10/10 |  |  |  |  |

- DISTRIBUTION LIST

- LIST OF ABBREVIATIONS

| S. No. | Abbreviations | Expansion |
| --- | --- | --- |
| 1 | AMR | Analytical Measurement Range |
| 2 | °C | Degree Centigrade |
| 3 | CRR | Clinical Reportable Range |
| 4 | DPX | Di-butyl Phthalate in Xylene |
| 5 | DW | Distilled water |
| 6 | EQAS | External Quality Assurance Scheme |
| 7 | HCL | Hydrochloric Acid |
| 8 | H&E | Hematoxylin and Eosin |
| 9 | HOD | Head of Department |
| 10 | HPE | Histopathological Examination |
| 11 | IQC | Internal Quality Control |
| 12 | LOD | Limit of Detection |
| 13 | LOQ | Limit of Quantitation |
| 14 | MSP | Management System Procedure |
| 15 | NABL | National Accreditation Board for Testing and Calibration Laboratories |
| 16 | N/A | Not Applicable/Available |
| 17 | pH | Potential of Hydrogen |
| 18 | QC | Quality Control |
| 19 | QM | Quality Manager |
| 20 | SOP | Standard Operating Procedure |
| 21 | TRF | Test Requisition Form |
| 22 | TDPL | TRUSTlab Diagnostics Private Limited |

- Objective

- This document describes a detailed procedure for hematoxylin and eosin staining of the tissue sections after processing and microtomy so that they can be examined under the microscope for a diagnosis.

- Scope

- This procedure applies to all staff of the Histopathology Department of TRUSTlab, tasked with the duty of performing tests involving these procedures.

- Responsibilities

It is the responsibility of the technicians of the Histopathology Department to comply with this SOP.

HOD shall ensure implementation.

The Quality Manager is responsible for monitoring, reviewing and ensuring compliance with this procedure.

Any other TRUSTlab staff involved in this process (including contract personnel) are responsible for complying with this procedure.

- Definitions

- Regressive Staining: The tissue is deliberately over-stained and then de-stained (differentiated) until the proper end point is reached.

- Principle

Biological staining is the union between a coloured dye and tissue substrate, which resists simple washing. Several mechanisms are involved in this chemical reaction which include electrostatic bonding, Van der Waal forces, covalent and hydrogen bonding, hydrophobic bonding and dye aggregation.

The commonly used tissue stain procedure in pathology is hematoxylin and eosin stain – manual method.

Hematoxylin itself is not a stain. The major oxidation product is hematein, a natural dye that is responsible for the colour properties. Hematein can be produced by natural oxidation or chemical oxidation. Hematoxylin stains the nucleus of the cells. Eosins are acid xanthene or phthalein dyes. The most commonly used eosin dye is eosin Y. Eosin combines with the alum hematoxylin to demonstrate the general histological architecture of a tissue. Eosin distinguishes between the cytoplasm of different types of cells and between different types of connective tissue fibers and matrices by staining them differing shades of red and pink.

Hematoxylin and eosin staining is based on the Regressive Staining principle.

- Specific Performance Characteristics

Uncertainty of measurement: NA

Accuracy: NA

Limit of quantitation (LOQ): NA

Limit of detection (LOD): NA

Analytical measurement range (AMR): NA

Clinical reportable range (CRR): NA

Sensitivity: NA

Specificity: NA

- Type of Sample

Tissue sections prepared from the paraffin blocks as described in TDPL/HP/SOP-003.

Sample Stability: The paraffin blocks are stable at RT for the duration of the archival period (10 years).

Sample Retention: 10 years.

Sample Rejection Criteria:

      - Irretrievable broken paraffin blocks.

      - No tissue present in the paraffin block.

      - Tissue sections with holes making the visualization of the diagnostic area very difficult for a definitive opinion.

      - Crumbled sections.

      - Unlabelled sections.

- Type of Container & Additives

- Paraffin Blocks.

- Patient Preparation

N/A.

- Equipment & Reagents

Equipment

      - Automated Staining: Leica Autostainer XL

      - Manual Staining: Staining troughs with lids

Reagents

      - Hematoxylin (Commercially procured)

      - 1% Alcoholic Eosin (One gram of Eosin Y stain powder +20 ml of DW+80ml of absolute/isopropyl alcohol)

      - 1% Acid Alcohol (495 ml of 70% isopropyl alcohol + 5.0ml of HCL)

        - To prepare 70% isopropyl alcohol, take 350 ml of isopropyl alcohol + 150 ml of DW.

      - Isopropyl Alcohol

      - Xylene

      - DPX

Reagent Storage Condition Stability

- All reagents mentioned above are stable at Room Temperature until the expiry date mentioned on the reagent package.

Additional Requirements

      - Glass slides

      - Cover slips

      - Tissue paper

      - Gauze piece

      - Labels

      - Pen

      - Slide rack

      - Slide trays

- Safety

- Safety precautions as mentioned in this SOP should be followed.

All tissues and chemicals should be treated as potentially biologically hazardous.

While handling tissues and the various chemicals, the universal safety precautions mentioned in TRUSTlab’s Laboratory Safety Manual (TDPL/QA/MAN-004) must be observed.

Xylene is an irritant to the skin and eyes. Use gloves while handling. If exposure occurs, wash with mild soap and water. Wash eyes with ample amount of water.

Xylene is explosive. Vapour accumulation may flash and/or explode if ignited. Keep ignition sources, open flames etc. away from these fumes.

- Calibration

Calibration Schedule

- N/A.

- Procedure

- Manual Staining

      - Label the glass slides with the HPE number and the year as mentioned on the    corresponding paraffin block.

      - Place the glass slide with the tissue section onto a filter paper in an inclined position so that the hot water of the floatation bath is drained out.

      - Place this slide on the slide warmer/incubator (temperature 65°C to 80°C for 15-20 minutes so that the slides are dry and the paraffin around the section starts melting.

      - Place the above slide in a slide rack and keep the rack in the Xylene I trough for de-paraffinisation. This trough contains an adequate amount of xylene so that the slides are completely immersed in it.

      - The slide rack is kept immersed in the Xylene I bowl for 5-10 minutes.

      - Transfer the slide rack into the Xylene II trough and keep immersed for the next 5-10 minutes.

      - Transfer the slide rack into the Xylene III trough and keep immersed for the next 5-10 minutes.

      - Transfer this slide rack into Alcohol I trough filled with adequate amount of Isopropyl alcohol so that the slides are completely immersed in the alcohol. Keep the slides in the alcohol for 3-5 minutes.

      - Transfer this slide rack into Alcohol II trough filled with an adequate amount of Isopropyl alcohol so that the slides are completely immersed in the alcohol. Keep the slides in the alcohol for 3-5 minutes.

      - Transfer this slide rack into Alcohol III filled with an adequate amount of Isopropyl alcohol so that the slides are completely immersed in the alcohol. Keep the slides in the alcohol for 3-5 minutes.

      - Transfer the above slide rack into an empty trough and keep the slides under running tap water.

      - Take care that the running tap water does not fall directly onto the tissue section of the slide.

      - Keep the slide in running tap water for 5 minutes.

      - Transfer the slide rack with the slides into hematoxylin trough filled with adequate quantity of hematoxylin and keep for 3-5 minutes.

      - Transfer this slide rack into an empty trough under running tap water for 5 minutes.

      - Dip the slides once into the trough filled with 1% Acid Alcohol.

      - Transfer the slide rack into the empty trough under running tap water for 10 minutes, so that bluing of the section occurs.

      - Dip this slide rack one to two times in the trough filled with 1% Eosin Y.

      - Dip the slides once in a separate trough of isopropyl alcohol and labeled as Alcohol I (this trough is different from the trough in Step h) so that the slides are completely immersed in alcohol for dehydration.

      - Transfer the slide into the Alcohol II trough for a single dip. (This trough is different from the one mentioned in Step i).

      - Transfer the slide into the Alcohol III trough for a single dip. (This trough is different from the one mentioned in Step j)

      - Blot the slides with gauze piece on the side opposite to the section and also on the side with the section in between the sections.

      - Dip the slides 1-2 times in xylene troughs I and II (These troughs are different from the one mentioned in steps e and f).

      - Mount the slides with the coverslip and DPX and label them.

- For mounting and labeling:

- Put DPX on the entire length of the cover slip avoiding the edges.

- Place the slides which are still moist with xylene on cover slip with sections facing the DPX.

- Gently press the coverslip and align it so that the edges of the cover slip and the glass slide are overlapping each other or the edge of the cover slip is on the slide itself.

- Wipe off the excess DPX. After staining and mounting is completed, a Trustlab label (with biopsy or cytology number and year) is stuck on to the glass slide.

      - Arrange all the slides of a particular case serially (alphabetically if any TRF has such sections) in a slide tray. Arrange the TRFs as per the sequence of the slides in the tray and submit the TRFs and the slide tray for reporting to the pathologist.

- Quality Control Procedure

Internal Quality Control

      - The quality of staining is assessed and graded by the pathologist daily and the same is entered in the IQC-Histopathology Form (TDPL/HP/FOM-003).

      - When the staining is not within the acceptable limits, the stains are either filtered or changed and the same is documented in the Quality Handling of H&E Stain (TDPL/HP/FOM-001).

      - Changing the solutions:

        - Once a week or when the solution is less for the jar, whichever occurs first.

        - Change the Alcohol and xylene whenever it turns turbid and there is change in color.

      - Filter the stain whenever precipitates are formed, or as and when instructed by the pathologist.

      - The quality of staining after a lot-change is assessed by the pathologist and the same is entered in the Quality Handling of H&E Stain (TDPL/HP/FOM-001).

External Quality Control

- N/A

- Interferences & Cross Reactions

Improper sections

Interferences due to the staining:

- Light staining

- Dark staining

- Stain particles

If the force of the tap water is high and falling directly onto the slides the part of section might get disrupted.

Inadequate mounting medium leads to a brown refractile colour of the sections and   interferes with the interpretation.

Excess of mounting medium causes the slippage of the cover slip.

Bubbles seen on the sections under microscope due to improper mounting.

- Result Calculation

- N/A.

- Biological Reference Interval

- N/A.

- Reportable Interval

- N/A

- Troubleshooting

| Problem | Reason | Solution |
| --- | --- | --- |
| Light Stain | -The sections are kept in H and E stain for lesser time -Freshly prepared hematoxylin | - Increase the time of the slides in the stain bowl. - In case the hematoxylin is freshly prepared keep the slides in the hematoxylin stain for longer time up to 15 minutes |
| Dark Stain | Section is thick/Over-stained | Refer to faults in section cutting reduce the time in Hematoxylin |
| Stain Precipitates | Crystals are formed in the stain solution | Filter the stain periodically at least once a week |
| Thin dark blue layer is seen on the sections under microscope | A layer forms on the surface of the hematoxylin stain after few days | Filter the stain periodically at least once a week |
| Refractile and oily appearance of the sections | Improper de-paraffinisation | De-paraffinisation in xylene should be done properly |
| Sections washed off | Improper tissue processing | - If possible, take more sections of the specimen - Refer to faults of poor processing in TDPL/HP/SOP-002 |

- Critical Alert Values

- N/A.

- Laboratory Clinical Interpretation

The sections are examined by a pathologist and impressions/interpretations are given accordingly.

- Potential Source of Variability

- N/A.

- References

      - Lakshminarayanan K. Immuno histochemistry. In Lakshminarayanan K: ed. Histological Techniques - A Practical Manual. Bhalani Publishing house, 2006:72-87.

      - Bancroft JD, Gamble M: Theory and Practice of Histological Techniques, 5th edition 2002, Churchill Livingstone U.K., 125-138

      - Procedure for Managing Internal QC Program (TDPL/QA/MAN-001: MSP-013)

      - Laboratory Safety Manual (TDPL/QA/MAN-004)