SOP – 005

PROCEDURE FOR

SPECIAL STAINS

- TABLE OF CONTENTS

TABLE OF CONTENTS	2

APPROVAL AUTHORITY	3

RECORD OF AMENDMENTS	4

DISTRIBUTION LIST	5

LIST OF ABBREVIATIONS	6

1.	Objective	7

2.	Scope	7

3.	Responsibilities	7

4.	Definitions	7

5.	Principle	8

6.	Specific Performance Characteristics	8

7.	Type of Sample	8

8.	Type of Container & Additives	9

9.	Patient Preparation	9

10.	Equipment & Reagents	9

11.	Safety	10

12.	Calibration	10

13.	Procedure	11

14.	Quality Control Procedure	13

15.	Interferences & Cross Reactions	13

16.	Result Calculation	14

17.	Biological Reference Interval	14

18.	Reportable Interval	14

19.	Troubleshooting	14

20.	Critical Alert Values	14

21.	Laboratory Clinical Interpretation	14

22.	Potential Source of Variability	15

23.	References	15

- APPROVAL AUTHORITY

| Prepared by: | Head of Department | Dr. Abdul Aleem Name | Date: 14/11/2024 |
| --- | --- | --- | --- |
|  |  | Signature |  |
| Reviewed & Approved by: | Laboratory Head | Dr. Abdul Aleem Name | Date: 14/11/2024 |
|  |  | Signature |  |
| Issued by: | Quality Manager | Dr. Sai Varun Name | Date: 14/11/2024 |
|  |  | Signature |  |

- RECORD OF AMENDMENTS

| S. No. | Ver. No. | Ver. Date | Rev. No. | Rev. Date | Description of Changes |
| --- | --- | --- | --- | --- | --- |

| Document Copy No. | Name / Designation of the holder of the document copy | Type of Copy | Version No. | Revision No. |
| --- | --- | --- | --- | --- |
| 01/10 | Managing Director | Controlled Copy | 01 | 00 |
| 02/10 |  |  |  |  |
| 03/10 |  |  |  |  |
| 04/10 |  |  |  |  |
| 05/10 |  |  |  |  |
| 06/10 |  |  |  |  |
| 07/10 |  |  |  |  |
| 08/10 |  |  |  |  |
| 09/10 |  |  |  |  |
| 10/10 |  |  |  |  |

- DISTRIBUTION LIST

- LIST OF ABBREVIATIONS

| S. No. | Abbreviations | Expansion |
| --- | --- | --- |
| 1 | AFB | Acid Fast Bacilli |
|  | AMR | Analytical Measurement Range |
| 2 | °C | Degree Centigrade |
| 3 | CRR | Clinical Reportable Range |
| 4 | DPX | Di-butyl Phthalate in Xylene |
| 5 | DW | Distilled water |
| 6 | EQAS | External Quality Assurance Scheme |
| 7 | HOD | Head of Department |
| 8 | HPE | Histopathological Examination |
| 9 | IQC | Internal Quality Control |
| 10 | LOD | Limit of Detection |
| 11 | LOQ | Limit of Quantitation |
| 12 | MSP | Management System Procedure |
| 13 | NABL | National Accreditation Board for Testing and Calibration Laboratories |
| 14 | N/A | Not Applicable/Available |
| 15 | PAS | Periodic Acid Schiff |
| 16 | pH | Potential of Hydrogen |
| 17 | QC | Quality Control |
| 18 | QM | Quality Manager |
| 19 | SOP | Standard Operating Procedure |
| 20 | TRF | Test Requisition Form |
| 21 | TDPL | TRUSTlab Diagnostics Private Limited |
| 22 | ZN | Ziehl Neelsen |

- Objective

This document describes the detailed procedures for special stains used in histopathology. These procedures are used to demonstrate carbohydrates, fats, connective tissue elements, various pigments or various organisms like H. Pylori, Acid fast bacilli and fungi so that a definitive diagnosis is made.

Broadly, the special stains are useful in the following conditions:

      - Inflammatory Lesions: Ziehl Neelsen stain for Acid fast bacilli, Periodic Acid Schiff (PAS).

      - Lymph node Biopsies: PAS for histiocytes and vascular basement membrane

      - Liver Biopsies: PAS for glycogen and, trophozoites of E. Histolytica, reticulin for demonstration of fibrosis.

      - Kidney Biopsies: PAS for basement membrane.

      - Gastrointestinal Biopsies: Giemsa stain for identification of H. Pylori organisms.

- Scope

- This procedure applies to all staff of the Histopathology Department of TRUSTlab, tasked with the duty of performing tests involving these procedures.

- Responsibilities

It is the responsibility of the technicians of the Histopathology Department to comply with this SOP.

HOD shall ensure implementation.

The Quality Manager is responsible for monitoring, reviewing and ensuring compliance with this procedure.

Any other TRUSTlab staff involved in this process (including contract personnel) are responsible for complying with this procedure.

- Definitions

Histopathology: It is a science of studying the structural changes brought about in the human body by a disease, to identify the disease and its cause; so that appropriate treatment can be given to the patient.

Toning: Treatment of sections with various solvents/solutions to produce good differentiation and sharpness of stain especially when there is subsequent counterstaining.

- Principle

Giemsa Stain: Giemsa stain is a Romanowsky stain with a combination of azure II and eosin. Azure is blue violet and stains acidic cell components. Eosin is red and stains

Periodic Acid Schiff: Substances containing vicinal glycol groups are oxidised by periodic acid to form dialdehydes, which combine with Schiff’s reagent to form an insoluble magenta coloured compound. Periodic acid is the oxidant of choice as it does not over-oxidise the formed aldehydes.

Periodic Acid Schiff with Diastase: Some of the polysaccharides are best demonstrated by their selective removal following specific enzyme digestion. Diastase selectively removes glycogen and results in loss of PAS staining.

Ziehl Neelsen Stain: Mycobacteria such as tubercle bacilli have a lipid-rich cell wall capable of taking up strong phenol dye solutions so that they retain the subsequent differentiation in acid or alcohol, i.e. they are acid or alcohol fast. Most other organisms lose the dye and take up a counter stain. ZN stain distinguishes acid fast bacilli such as M. Tuberculosis and M. Leprae from other non-acid fast bacilli.

- Specific Performance Characteristics

Uncertainty of measurement: NA

Accuracy: NA

Limit of quantitation (LOQ): NA

Limit of detection (LOD): NA

Analytical measurement range (AMR): NA

Clinical reportable range (CRR): NA

Sensitivity: NA

Specificity: NA

- Type of Sample

Tissue sections prepared from the paraffin blocks as described in TDPL/HP/SOP-003.

Sample Stability: The paraffin blocks are stable at RT for the duration of the archival period (10 years).

Sample Retention: 10 years.

Sample Rejection Criteria:

      - Tissue sections with holes making the visualization of the diagnostic area very difficult for a definitive opinion.

      - Crumbled sections.

      - Sections where the area of interest for special stains is lost/absent.

      - Uneven sections.

      - Unlabeled sections.

      - Tear or scratch across the section.

      - No tissue in paraffin block or section on slide.

- Type of Container & Additives

- Paraffin Blocks.

- Patient Preparation

N/A.

- Equipment & Reagents

Equipment

- N/A.

Reagents

      - Periodic Acid Schiff: 1% Periodic acid, Schiff’s reagent.

      - Ziehl Neelsen Stain: Carbol fuchsin, methylene blue, 1% acid alcohol (350 ml isopropyl alcohol + 150 ml of DW + 5 ml HCL).

- Lugol’s Iodine, Liquid paraffin, Carbol fuchsin, 5% Sulfuric acid for modified Ziehl Neelsen staining.

      - Hematoxylin (counter stain), 1% acid alcohol for differentiation.

      - General for all stains: Mounting medium (DPX), Xylene

Reagent Storage Condition Stability

      - Stock solution of Schiff`s reagent stored in refrigerator at 2-8°C till the solution is consumed entirely.

      - Working solutions of Geimsa are prepared fresh every time the test is performed.

      - The rest of all the reagents are stable at Room Temperature until the expiry date mentioned on their package.

      - Freshly prepared solutions are documented in Freshly prepared reagents log - F/QSP/50.39.

Additional Requirements

      - Incubator

      - Coplin Jars

      - Glass slides

      - Cover slips

      - Labels

      - Pen

      - DPX

- Safety

- Safety precautions as mentioned in this SOP should be followed.

All tissues and chemicals should be treated as potentially biologically hazardous.

While handling tissues and the various chemicals, the universal safety precautions mentioned in TRUSTlab’s Laboratory Safety Manual (TDPL/QA/MAN-004) must be observed.

Xylene is an irritant to the skin and eyes. Use gloves while handling. If exposure occurs, wash with mild soap and water. Wash eyes with ample amount of water.

Xylene is explosive. Vapour accumulation may flash and/or explode if ignited. Keep ignition sources, open flames etc. away from these fumes.

- Calibration

Calibration Schedule

- N/A.

- Procedure

- Manual Staining

Label the slide with HPE number of the section with diamond pencil. Take the thin sections (test and control) on a slide.

Keep the slide in the incubator till the paraffin around the section melts.

De-paraffinization of the section: Dip the slide in 1st xylene jar for 5 minutes, 2nd xylene jar for another 5 minutes, then transfer it to 3rd Xylene jar for another 5 minutes.

Bringing section to water: Keep the above slide in 1st jar of isopropyl alcohol for 5 minutes and then transfer to the 2nd jar of isopropyl alcohol for another 5 minutes.

Perform the corresponding special stain procedure as mentioned below.

    - Periodic Acid Schiff (McManus Method)

      - 0.5 % Periodic acid (Shelf life: 1 year) - 0.5 gm periodic acid + 100 ml DW

      - Schiff’s reagent (Shelf life: 6 months) - 1 gm Basic fuchsin + 100 ml of boiling DW.

      - Shake for 5 minutes and cool down to 50°C.

      - Filter and add 20 ml of 1 N HCL or 2 ml of Conc HCL.

      - Cool and add 1 gm sodium/potassium meta bisulphite.

      - Store in a dark place for 18-24 hours.

      - Add 2 gm activated charcoal.

      - Shake and mix for 1 minute.

      - Filter the charcoal, and store in the refrigerator (4-8°C) in a dark container.

      - This solution is straw colored. When the color changes, discard it. Make a fresh solution.

      - Follow steps 13.1 to 13.4.

      - Put periodic acid on the section. Keep for 5 to 10 minutes.

      - Wash thoroughly on tap water. Rinse in DW

      - Put Schiff’s solution (which has been brought to the RT) on the section. Keep for 15 minutes.

      - Wash thoroughly in running water for 5 minutes.

      - Keep the slide in hematoxyline jar for 1 to 2 minutes to counter stain the section.

      - Wash thoroughly in running water for 5 minutes

      - Dehydrate, clear and mount in DPX (refer to 13.6, to 13.8)

    - Modified Ziehl–Neelsen Stain

      - Deparaffinize sections with xylene and liquid paraffin mixture.

- (2 parts of xylene and 1 part of liquid paraffin)

- 2 changes of 12 minutes each

      - Drain, wipe excess of oil and blot to opacity (The residual oil helps to prevent shrinkage and injury to the sections. It also prevents the removal of acid-fast material from the organisms).

      - Treat the sections for 5 minutes in Lugols Iodine.

      - Wash in running tap water.

      - Bleach the sections with 5% sodium thio sulphate for 3 – 5 minutes.

      - Wash in running tap water for 5 minutes.

      - Carbol fuschin for 30minutes at room temperature.

      - Wash with running tap water until excess of stain runs out.

      - Decolourize the slides individually for 10 minutes with 5% sulfuric acid.

      - Wash in tap water for 10 minutes.

      - Counter stain with Harris hematoxylin for 15 seconds.

      - Wash in running tap water for 5 minutes.

      - Blot and dry slides.

      - Mount with DPX.

    - Giemsa stain for H.Pylori

      - Follow steps 13.1 to 3.4.

      - Cover the slides with a working Giemsa solution for 25 minutes.

      - Wash with tap water.

      - Blot and Dry slides, clear in xylene and mount with DPX.

Dehydrating the section: Dip the slides in 1st Isopropyl alcohol and 2nd Isopropyl alcohol jars 3 times. These jars are separate from those used in step 4.9.4.

Clearing the sections: Dip the slides in xylene once and then mount.

Mounting the sections:

      - Put DPX on the entire length of the coverslip avoiding the edges.

      - Place the slides on coverslip with sections facing the DPX.

      - Gently press the coverslip and align it so that the edges of the coverslip and the glass slide are overlapping each other or the edge of the coverslip is on the slide itself.

      - Wipe off the excess DPX with tissue paper.

Labelling the slide:

      - After mounting the sections, paste Trustlab slide labels with HPE number of the section and the name of the special stain on the slide.

      - The positive control slide is labeled with the name of the special stain and the date of the procedure.

Submit the stained sections along with corresponding positive control, H&E-stained sections and the TRF to the Pathologist for reporting.

- Quality Control Procedure

Internal Quality Control

      - The quality of staining is assessed and graded by the pathologist daily and the same is entered in the IQC-Histopathology Form (TDPL/HP/FOM-003).

      - Perform the staining procedure simultaneously on a section known to be positive for the corresponding special stain (positive control) with every batch.

      - Quality of staining is determined by the reporting Pathologist on microscopy.

      - QC failure:

        - No stain in the positive control.

        - Internal controls in stains like PAS, ZN are negative with no stain in the positive control slide.

      - Corrective Actions for QC failure:

        - Repeat the corresponding test along with positive control.

        - Document the same in Repeat/ rejection logbook.

External Quality Control

- N/A

- Interferences & Cross Reactions

If the section is not completely covered with a reagent, staining is affected.

Sections with holes, scratches across them.

Formation of stain precipitates.

Incomplete dehydration of the stained section.

Incomplete clearing of the stained section.

Air bubbles during mounting.

Sections washed off during the procedure.

- Result Calculation

- N/A.

- Biological Reference Interval

- N/A.

- Reportable Interval

- N/A

- Troubleshooting

- GMS and Reticulin stain: The high alkalinity of the silver nitrate solution tends to detach the sections from the slides. Take the section well to an albuminized / poly L lysine coated slide by through drying.

- Critical Alert Values

- N/A.

- Laboratory Clinical Interpretation

Results of the special stain sections are interpreted on microscopy considering the findings on H & E stained sections and positive control slide.

Interpretation of Special Stains:

| Special Stain | Control | Interpretation |
| --- | --- | --- |
| Giemsa for H. Pylori | Known-positive tissue | Small comma-shaped dark blue organisms are seen at the luminal surface of the mucous glands. |
| PAS | Liver, colon, appendix | Positive structures - Magenta color Nucleus - Blue/Violet color |
| ZN | Known-positive tissue | Acid fast bacilli- Red color Other organisms- Blue color |

PAS positive structures: Basement membrane, fungi, chitin, collagen (weakly), reticulin, neutral mucins, glycogen, starch, platelets.

Details of special stains done including control performance are captured in Special stains log.

- Potential Source of Variability

- N/A.

- References

      - Lakshminarayanan K. Immuno histochemistry. In Lakshminarayanan K: ed. Histological Techniques - A Practical Manual. Bhalani Publishing house, 2006:72-87.

      - Bancroft JD, Gamble M: Theory and Practice of Histological Techniques, 5th edition 2002, Churchill Livingstone U.K., 125-138

      - Procedure for Managing Internal QC Program (TDPL/QA/MAN-001: MSP-013)

      - Laboratory Safety Manual (TDPL/QA/MAN-004)