SOP – 007

PROCEDURE FOR

PROCESS FLOW

- TABLE OF CONTENTS

TABLE OF CONTENTS	2

APPROVAL AUTHORITY	4

RECORD OF AMENDMENTS	5

DISTRIBUTION LIST	6

LIST OF ABBREVIATIONS	7

1.	Objective	8

2.	Scope	8

3.	Responsibilities	8

4.	Definitions	8

5.	Principle	8

6.	Specific Performance Characteristics	8

7.	Type of Sample	9

8.	Type of Container & Additives	10

9.	Patient Preparation	10

10.	Equipment & Reagents	10

11.	Safety	10

12.	Calibration	11

13.	Procedure	11

14.	Quality Control Procedure	17

15.	Interferences & Cross Reactions	17

16.	Result Calculation	17

17.	Biological Reference Interval	17

18.	Reportable Interval	17

19.	Troubleshooting	17

20.	Critical Alert Values	17

21.	Laboratory Clinical Interpretation	17

22.	Potential Source of Variability	18

23.	References	18

APPENDIX – I	18

APPENDIX – II	18

APPENDIX – III	19

- APPROVAL AUTHORITY

| Prepared by: | Head of Department | Dr. Abdul Aleem Name | Date: 14/11/2024 |
| --- | --- | --- | --- |
|  |  | Signature |  |
| Reviewed & Approved by: | Laboratory Head | Dr. Abdul Aleem Name | Date: 14/11/2024 |
|  |  | Signature |  |
| Issued by: | Quality Manager | Dr. Sai Varun Name | Date: 14/11/2024 |
|  |  | Signature |  |

- RECORD OF AMENDMENTS

| S. No. | Ver. No. | Ver. Date | Rev. No. | Rev. Date | Description of Changes |
| --- | --- | --- | --- | --- | --- |

| Document Copy No. | Name / Designation of the holder of the document copy | Type of Copy | Version No. | Revision No. |
| --- | --- | --- | --- | --- |
| 01/10 | Managing Director | Controlled Copy | 01 | 00 |
| 02/10 |  |  |  |  |
| 03/10 |  |  |  |  |
| 04/10 |  |  |  |  |
| 05/10 |  |  |  |  |
| 06/10 |  |  |  |  |
| 07/10 |  |  |  |  |
| 08/10 |  |  |  |  |
| 09/10 |  |  |  |  |
| 10/10 |  |  |  |  |

- DISTRIBUTION LIST

- LIST OF ABBREVIATIONS

| S. No. | Abbreviations | Expansion |
| --- | --- | --- |
| 1 | AMR | Analytical Measurement Range |
| 2 | °C | Degree Centigrade |
| 3 | CRR | Clinical Reportable Range |
| 4 | DC | Decalcify |
| 5 | EQAS | External Quality Assurance Scheme |
| 6 | HOD | Head of Department |
| 7 | HPE | Histopathological Examination |
| 8 | IQC | Internal Quality Control |
| 9 | LOD | Limit of Detection |
| 10 | LOQ | Limit of Quantitation |
| 11 | MS | More Sections |
| 12 | MSP | Management System Procedure |
| 13 | NABL | National Accreditation Board for Testing and Calibration Laboratories |
| 14 | N/A | Not Applicable/Available |
| 15 | PAS | Periodic Acid Schiff |
| 16 | pH | Potential of Hydrogen |
| 17 | QC | Quality Control |
| 18 | QM | Quality Manager |
| 19 | SOP | Standard Operating Procedure |
| 20 | TRF | Test Requisition Form |
| 21 | TDPL | TRUSTlab Diagnostics Private Limited |

- Objective

- The purpose of this SOP is to establish guidelines to formulate methods, techniques and management practices to handle all the specimens received in the Histopathology department (Includes Cytopathology) of TRUSTlab, for testing.

- Scope

- This procedure applies to all staff of the Histopathology Department of TRUSTlab, tasked with the duty of performing tests involving these procedures.

- Responsibilities

It is the responsibility of the technicians of the Histopathology Department to comply with this SOP.

HOD shall ensure implementation.

The Quality Manager is responsible for monitoring, reviewing and ensuring compliance with this procedure.

Any other TRUSTlab staff involved in this process (including contract personnel) are responsible for complying with this procedure.

- Definitions

- Histopathology: It is a science of studying the structural changes brought about in the human body by a disease, to identify the disease and its cause; so that appropriate treatment can be given to the patient.

- Principle

- N/A.

- Specific Performance Characteristics

Uncertainty of measurement: NA

Accuracy: NA

Limit of quantitation (LOQ): NA

Limit of detection (LOD): NA

Analytical measurement range (AMR): NA

Clinical reportable range (CRR): NA

Sensitivity: NA

Specificity: NA

- Type of Sample

Sample types for histopathology include:

    - Biopsy for histopathological examination (small, moderate, large, radical).

    - Paraffin Blocks for second opinion

    - Stained slides for second opinion

    - Blocks for special stains

Sample types for cytopathology include:

    - Gynaec Cytology smears (PAP smears)

    - Fluids

    - FNAC’s

    - Sputum

    - Imprint smears

    - Stained slides for second opinion

Sample Stability: The paraffin blocks are stable at RT for the duration of the archival period (10 years).

Sample Retention: Fluid – 3 days; Tissue – 30 days; Slide & Block – 3650 days

Sample Rejection Criteria & Follow-up Action:

      - Mismatch of name & type of specimen: All samples with mismatch of name &/ sample type details or with unlabeled specimen containers are not processed. Further attempts are made, as per the instructions of the pathologist to reach the clinician or the customer for clarification, so that processing can be started. The same is documented on the TRF. TAT for such samples shall commence from the time clarifications are received.

      - Samples with mismatch of only age and sex (and with matching other demographics) are directly put forward for processing.  Further attempts are made to reach the clinician or the customer for clarification of the same. Reports are released on receipt of the clarifications. TAT for such samples starts from the time clarifications are received.

      - Completely autolysed specimen.

      - If the specimen is completely autolysed the referral doctor is contacted to confirm as to whether the sample is to be processed or not. Based on the communication provided, the sample will be processed further (or) rejected. Once rejected, the billing counter (reception) person is contacted for the refund of the charges collected for the same giving reasons for rejection. Details of rejection are logged in the reject log of the section.

      - Irretrievably broken slides. (Refer to Appendix II)

      - Samples are processed as per the details given below.

      - For samples where no clinical details are procured (even after multiple attempts), processing of samples is carried out. However, reporting of these cases shall be done only after two weeks, following sample acceptance, as permissible (e.g. with only microscopic findings or with only provisional diagnosis or with a remark at the end of the report) and report is released.

- Type of Container & Additives

- Sealed Jars/Containers; Paraffin Blocks.

- Patient Preparation

N/A.

- Equipment & Reagents

Equipment

- N/A.

Reagents

- N/A.

- Safety

- Safety precautions as mentioned in this SOP should be followed.

All tissues and chemicals should be treated as potentially biologically hazardous.

While handling tissues and the various chemicals, the universal safety precautions mentioned in TRUSTlab’s Laboratory Safety Manual (TDPL/QA/MAN-004) must be observed.

Xylene is an irritant to the skin and eyes. Use gloves while handling. If exposure occurs, wash with mild soap and water. Wash eyes with ample amount of water.

Xylene is explosive. Vapour accumulation may flash and/or explode if ignited. Keep ignition sources, open flames etc. away from these fumes.

- Calibration

Calibration Schedule

- N/A.

- Procedure

Pre-Examination Procedures:

- It includes aspects of sample receipt and sample documentation:

    - Sample Receipt

      - Samples are handed over to the Accession Department by the riders who collect the samples from all the collection centres.

      - Accesstion registers the sample in LIMS and then intimates the department via a phonecall.

      - Upon receiving the phone call, the technician on duty goes to Accession, receives the sample, verifies the details and returns to the department with it.

      - “Department Receive” on LIMS is then updated.

      - Technician then checks the labels of biopsy containers for the presence of at least 2 identifiers. Identifiers may be, but not limited to, the following - Patient name, ID, age, sex, doctor name, clinical details etc. Identifiers on the specimen containers shall match the data provided (i.e., Doctors prescription, TRF'S / Registration receipt).

      - For STAT samples or verbal requests for additional tests – Any request for urgent provision of final report (histopathology / cytopathology) is addressed, as suitable. The request form is labeled as “Urgent report” on the top and the pathologist is intimated. Every attempt is made to furnish the final report as early as possible.

      - Any other verbal request is duly addressed as per the nature of the request and after discussion with the pathologist

    - Sample Documentation

      - All the histopathology and cytopathology samples are sorted out by the same technician who receives the samples & proceeds with documentation.

      - Unique biopsy or cyto no is generated from the respective accession logs, and the TRF’s and specimen are labeled with the same.

      - Each test request form bears a unique ID as exemplified here:

        - TLHP1231/A/25: where TLHP stands for TRUSTlab Histopathology, 1231 is the biopsy number, A is the suffix for number of sections like A, B C, D etc. and 25 denotes the year of sample receipt.

        - CY100/25: where CY stands for Cytology, 100 is the gynaec/non-gynaec cytology number and 25 denotes the year of sample receipt.

      - The same no is then noted on the TRF, additional papers including prescription(s)/ clinical details info/radiographs and the specimen container.

      - When only unstained/stained slide(s) for histopathology/cytopathology is received, the procedure described below is followed -

      - The barcode on the wrapper is stapled or fixed to the TRF with the generated unique ID number. The same number is inscribed on the slides.

      - The same process is carried out for gynaec conventional pap smears and for LBC fluid.

      - Then a similar process is followed for biopsy samples i.e., all the histopath test request forms are given unique ID's, labels are written with specific biopsy number and are pasted on to the respective container, with specific subscripts, wherever required

Examination Procedure

    - Histopathology

      - General Instructions:

        - Other specific columns in the histopath request form including requisition, patient sample details, label on container, formalin status, state of samples received are all now updated.

        - The containers for biopsy samples are then verified for the presence of fixative (Formalin). For samples without fixative, the same is reflected in the TRF and fresh 10% buffered neutral formalin is added to the container or they are placed in containers with formalin till the time of grossing. The same is also reflected in the final report of the sample. If the specimen is completely autolysed, follow the rejection criteria mentioned in this document.

        - Samples received in polythene covers are transferred into suitable containers and an adequate volume of 10% buffered neutral formalin is added to them and then appropriately labeled.

        - All the request forms are then brought up to the pathologist, who initials them and suggests a further course of action as appropriate. In the absence of clinical details /site of biopsy, all attempts are made to procure the same from the referring consultant.

        - All the TRF`s with other attachments including prescription(s)/ clinical details info/ radiographs, if any, and raw data sheet are kept in a serial order along with the specimen container for grossing.

        - The unique histopathology ID for every specimen is maintained throughout the processing and examination steps

        - De-paraffinization of the section: Dip the slide in 1st xylene jar for 5 minutes, 2nd xylene jar for another 5 minutes, then transfer it to 3rd Xylene jar for another 5 minutes.

      - Grossing

        - All biopsy samples are grossed as per the laid down SOP (TDPL/HP/SOP-006).

        - At grossing, the number of sections taken is captured on the reverse of the TRF. Further, each section in the tissue cassette is accompanied by a label bearing the unique histopathology number inscribed in pencil.

        - Specific embedding instructions, if any, are captured in separate embedding instructions book by the technician assisting at grossing.

        - After grossing of individual specimen, specific columns in TRF including grossing done by, time of grossing and other details are updated by the same technician who assists in grossing.

        - All the tissue cassettes with sections and appropriate labels are taken in to the basket which is then loaded on to the tissue processor, by 1:30 pm every day.

      - Embedding

        - At embedding and blocking, the same label, bearing the unique histopathology number inscribed in pencil, is embedded in to the wax block in embedding ring. Specific instructions in embedding instruction book are adhered to by the technician on embedding duty who further initials the embedding instructions book.

        - With a marker pen, the unique ID is written on the surface of the embedding ring with biopsy number and registration number on one side of the case and with month and year on the other side of the case.

      - Microtomy

        - At microtomy, biopsy number and last four digits of barcode number are inscribed with a diamond pencil on the slides on one edge. Then the tissue ribbons are taken on to the slides by the concerned technician.

        - Slides with ribbons are forwarded for further staining.

      - Staining and Submission

        - For specific aspects of staining, refer to TDPL/HP/SOP-004.

        - After staining and mounting is completed, a Trustlab label (with biopsy number and year) is stuck on to the glass slide.

        - The stained slides together with request forms are submitted for reporting by pathologist(s).

      - Slide(s) & / Block(s) for Opinion

        - The same process is followed for slides and blocks which are received for second opinion. After the biopsy no is generated for the slides, they are put up to the pathologists for opinion and the paraffin blocks are included in the batch of blocks which are for cutting.

    - Cytopathology

      - For cytopathology, fluid specimen or slides are given unique number on receipt and after confirmation of the details in the TRF.

      - Fluids for cytology are processed in centrifuge and slides are prepared; Smears of FNA for cytology are directly stained with H&E and MGG (and PAP stain, if required and as suggested by pathologist).

      - LBC fluid is processed in the LBC instrument and slides are prepared for PAP staining; Conventional PAP smears are directly subjected to PAP straining.

      - All slides bear the last four digits of SIN/Barcode number inscribed at one end and respective cytology number at other end.

      - After staining and mounting is completed, a TRUSTlab label (with respective cytology number and year typed on it) is stuck onto the glass slide.

      - The stained slides together with request forms are submitted for reporting by pathologists(s)

      - The slides put up for the reporting, along with the TRF`s, are checked for appropriateness of unique ID number on the glass slide which is correlated with that on the TRF by the pathologist who shall further ensure specimen identity at reporting.

      - The pathologist examines the slides under the microscope and documents his/her findings in the microscopic description section of TRF.

      - A note is made in the IQC-Histopathology Form (TDPL/HP/FOM-003) about quality of processing, cutting and staining of sections in histopathology and in the Daily IQC Data Monitoring Form (TDPL/GE/FOM-012) for the processing of cytology and gynaec specimens.

      - All attempts are made to reach a final impression. When a final opinion is made the same is written on the TRF sheet and is signed off by the pathologist, putting the date of reporting.

      - In all difficult cases and malignancies, the slides are also reviewed by other pathologists on duty and a consensus opinion is given. When there is a disparity in the opinion a differential diagnosis is provided. The names of the pathologists who have opined are reflected in the final report which is released.

      - For cases where special stains are required or for bony specimens with soft tissue an interim report is issued. Final report is issued only after the special stains slide(s)/ decalcified sections are examined.

Post Examination Procedures

    - General

      - The TRF`s with the report is then given to the data entry operator for draft of the reports on PCS. Verification of the report for any errors and final authentication is done by the Pathologist on whose name the report is being released.

      - At final authentication, the reports uploaded and checked by the pathologist for all details and the correctness of the report, which is then released electronically by the authorized signatory.

      - The final report includes all the details to identify the patient, unique histopathology number, with the details of the gross findings, no and type of sections studied, microscopic findings in brief highlighting the salient diagnostic features along with a Final Impression. Comments /notes /advice, as deemed necessary, are included in the final report.

      - The reports after authentication are ready to be taken out as hard copies by the collection centers at their respective places / data entry operators at the main laboratory.

      - The released reports are archived as per the regulatory requirements electronically.

      - After reporting, the slides are kept in department temporary archive for 1 month and then transferred to the main slide storage cabinets in the slide storage room for 5 years from date of registration.

      - The TRFs are archived in the department for 10 yrs.

    - Retention of Samples

      - Biopsy:

- The containers of “All embedded specimen” are kept in the sample storage area in the section till the time of release of report and all other samples are kept in the tissue storage area till 15 days from the date of dispatch of reports, after which they are sent for discard through the contracted agency hired by the laboratory for disposal of wastes. Refer below for details of discarding process.

      - Cytology:

- Fluids are stored in the refrigerator at 2-8°C temperature for 72 hours from the time of registration.

      - Slides and blocks:

        - All cytology slides, Gynaec PAP smears and smears prepared from fluid(s) submitted will be retained for 10 years from the date of registration.

        - Slides of FNA shall be retained for 10 years from the date of registration.

        - Slides prepared from cell blocks shall be retained for 10 years from the date of registration.

        - All paraffin blocks, other than those issued to patient, shall be retained for 10 years

        - Request for slides and blocks is duly reciprocated after the receipt of the request form from the branches duly signed off by the patient and branch manager. For walk in patients, slides and blocks are issued after taking the signature of the concerned in the logbook. (See Appendix for format for issue of slides and blocks).

    - Discard of Biopsy Specimen

      - After the lapse of retention period following release of reports, the biopsy specimens are discarded.

      - The process of discarding includes:

        - identifying the respective samples,

        - discarding formalin in the drain under running tap water

        - packing the biopsy specimen in yellow bags which are to be transferred to the point from where contracted agency hired by the laboratory for disposal of wastes picks up the bags for further disposal.

        - Details of discarded specimen are updated in histopathology register.

- Quality Control Procedure

- N/A,

- Interferences & Cross Reactions

N/A.

- Result Calculation

- N/A.

- Biological Reference Interval

- N/A.

- Reportable Interval

- N/A

- Troubleshooting

- N/A.

- Critical Alert Values

- N/A.

- Laboratory Clinical Interpretation

- N/A.

- Potential Source of Variability

- N/A.

- References

      - Theory and Practice of Histological Techniques - John D. Bancroft, 8th edition

      - Rosai and Ackerman`s Surgical Pathology - Juan Rosai, 10th Edition

      - NABL 112: Specific criteria for accreditation of medical laboratories

      - CAP Anatomic pathology and cytopathology checklist

      - Procedure for Managing Internal QC Program (TDPL/QA/MAN-001: MSP-013)

      - Laboratory Safety Manual (TDPL/QA/MAN-004)

- APPENDIX – I

- Samples without TRFs

        - Samples without RF`s are received by the technical staff on duty in the section. They are kept in a separate tray meant for the same in the section only to prevent loss of specimen in the accession area.

        - Once the TRF is received for the sample, the same procedure described earlier is followed.

        - TAT for such specimen’s starts from the time the TRF is received in the section.

- APPENDIX – II

- Retrieval of Broken Slides

        - If the slides are broken into a few pieces, and if the section of interest is retrievable, the coverslip on the broken bits is removed by dipping in xylene.

        - An intact slide with a thin layer of DPX over it is taken.

        - The broken bits are rearranged on the slide with DPX over it.

        - DPX on the slides is allowed to dry.

        - Mounting is done with DPX.

        - Slides are labeled and put up for submission.

- APPENDIX – III

- Work instructions for issuing Slides and Blocks

        - On request for slides and blocks by the consultant, the same are retrieved by the technical staff and put up for the Pathologist for selection.

        - For small biopsy specimens, either the slide or the block only will be issued. One of them is archived for any future referral.

        - For moderate and large specimens, select sections with areas of interest and all other blocks are issued. This ensures that the patient receives either slides or blocks for the total no of bits examined and the section also has an entire set of slides/blocks of the case.

        - Slides are placed in slide mailers and sealed safely and blocks in self-sealing envelopes and are together and handed over to the patient/attendant

        - The same is entered in the log book maintained for the purpose in the section and the signature of the person receiving the same is taken.