SOP - 001

PROCEDURE FOR

GRAM STAINING

- TABLE OF CONTENTS

TABLE OF CONTENTS	2

APPROVAL AUTHORITY	3

RECORD OF AMENDMENTS	4

DISTRIBUTION LIST	5

LIST OF ABBREVIATIONS	6

1.	Objective	7

2.	Scope	7

3.	Responsibilities	7

4.	Definition	7

5.	Test Principle	7

6.	Specific Performance Characteristics	7

7.	Type of Sample	8

8.	Patient Preparation	8

9.	Type of Container & Additives	8

10.	Equipment & Reagents	8

11.	Safety	8

12.	Calibration Procedure	9

13.	Test Procedure	9

14.	Quality Control Procedure	10

15.	Interfering Substances	11

16.	Result Calculation	11

17.	Biological Reference Interval	12

18.	Reportable Interval	12

19.	Procedure when Results are not in Range	12

20.	Critical Alert Values	12

21.	Laboratory Clinical Interpretation	12

22.	Potential Source of Variability	12

23.	References	13

- APPROVAL AUTHORITY

| Prepared by: | Head of Department | Dr. Sadiya Shahnaz Name | Date: 14/11/2024 |
| --- | --- | --- | --- |
|  |  | Signature |  |
| Reviewed & Approved by: | Laboratory Head | Dr. Abdul Aleem Name | Date: 14/11/2024 |
|  |  | Signature |  |
| Issued by: | Quality Manager | Dr. Sai Varun Name | Date: 14/11/2024 |
|  |  | Signature |  |

- RECORD OF AMENDMENTS

| S. No. | Ver. No. | Ver. Date | Rev. No. | Rev. Date | Description of Changes |
| --- | --- | --- | --- | --- | --- |

| Document Copy No. | Name / Designation of the holder of the document copy | Type of Copy | Version No. | Revision No. |
| --- | --- | --- | --- | --- |
| 01/10 | Managing Director | Controlled Copy | 01 | 00 |
| 02/10 |  |  |  |  |
| 03/10 |  |  |  |  |
| 04/10 |  |  |  |  |
| 05/10 |  |  |  |  |
| 06/10 |  |  |  |  |
| 07/10 |  |  |  |  |
| 08/10 |  |  |  |  |
| 09/10 |  |  |  |  |
| 10/10 |  |  |  |  |

- DISTRIBUTION LIST

- LIST OF ABBREVIATIONS

| S. No. | Abbreviations | Expansion |
| --- | --- | --- |
| 1 | ATCC | American Type Culture Collection |
| 2 | EDTA | Ethylenediaminetetraacetic Acid |
| 3 | EQAS | External Quality Assurance Scheme |
| 4 | IAMM | Indian Association of Medical Microbiologists |
| 5 | MI | Microbiology |
| 6 | NABL | National Accreditation Board for Testing and Calibration Laboratories |
| 7 | rpm | Revolutions per minute |
| 8 | SOP | Standard Operating Procedure |
| 9 | TDPL | TRUSTlab Diagnostics Private Limited |

- Objective

  - This SOP describes the procedural steps for Gram staining of smears to report the presence of microorganisms in the given clinical specimen or from a bacterial colony.

- Scope

This procedure applies to all staff of the Microbiology Department tasked with the duty of performing these tests.

- Responsibilities

It is the responsibility of the technicians of the Microbiology Department to comply with this SOP.

HOD shall ensure implementation.

The Quality Manager is responsible for monitoring, reviewing and ensuring compliance with this procedure.

All other TRUSTlab staff involved in this process (including contract personnel) are responsible for complying with this procedure.

- Definition

Gram’s staining is a method of differentiating bacterial species into two large groups Gram-positive and Gram-negative, based on the chemical and physical properties of their cell walls.

- Test Principle

The Gram reaction is dependent on the permeability of the bacterial cell wall and cytoplasmic membrane, to the dye-iodine complex. Gram positive bacteria retain the primary stain due to multilayered peptidoglycan, decreased space between the molecules of cell wall & acidic cytoplasm and stain purple violet. Gram negative bacteria do not retain the dye-iodine complex due to dissolution of outer lipopolysaccharide membrane of cell wall and damage of cytoplasmic membrane and they take the counter stain pink.

- Specific Performance Characteristics

- Stains bacteria to give Gram’s reaction – Gram positive or Gram negative.

- Type of Sample

- Cerebrospinal fluid, sputum, body fluids, pus, exudates, tissues, pure bacterial colony, urine, endotracheal secretions, bronchoalveolar lavage fluid.

- Patient Preparation

- Refer to the primary sample collection manual.

- Type of Container & Additives

- Sterile, leak proof, capped containers, sterile swabs, syringes.

- Equipment & Reagents

Calibrated Inoculation loop

Microscope

Slides

Gram stain kit

Bunsen burner

Culture Media

Marker

Centrifuge

Alcohol spirit

Cotton/ tissue paper

Biosafety Cabinet

- Safety

The use of disposable gloves and proper biohazardous clothing is STRONGLY RECOMMENDED while running the test.

In case there is a cut or wound on the hand, DO NOT PERFORM THE TEST.

Do not smoke, drink or eat in areas where specimens or kit reagents are being handled.

Tests are for in vitro diagnostic use only and should be run by competent personnel only.

Do not pipette by mouth.

All materials used in the assay and samples should be decontaminated in 1% sodium hypochlorite solution for 30-60 min. They should be disposed of in accordance with established safety procedures.

Wash hands thoroughly with soap or any suitable detergent, after the use of the kit. Consult a physician immediately in case of accident or contact with eyes, in the event that contaminated material is ingested or comes in contact with skin punctures or wounds.

Spills should be decontaminated promptly with Sodium Hypochlorite or any other suitable disinfectant.

Suitable infection prevention precautions to be followed.

- Calibration Procedure

- Biosafety Cabinet and centrifuge calibration is done on an annual basis by an NABL accredited calibration agency. Necessary maintenance measures are undertaken for all the equipment.

- Test Procedure

Pre-assay Instructions

      - Slides should be clean and devoid of dust and particles. Slides should be cleaned with dry tissue paper and if it still feels oily or mucky, run the slide through flame to clear the oil debris and clean it dry with tissue paper.

      - Frosted slides may be preferred to label the slides appropriately with patient details by glass marking pencil or slides numbered 1, 2… on the reverse of slide with date and tallying worksheet with patient details.

      - Smears should be made on the middle third portion of the slide spread 1-3 cm in diameter.

      - Smears should not be made too thick or thin to allow sufficient penetration of stains.

      - All specimens should be thoroughly mixed before making smear, to create homogeneity.

      - All sterile body fluids need to be centrifuged at 1000 rpm for 15 minutes or 2000 rpm for 10 minutes, or 3000 rpm for 5 minutes in sterile test tubes. The sediment is taken to make the smears after decanting the supernatant into a sample container. A 1 cm diameter smear is made from the deposit. If the sample volume is less, the sample is smeared on the slide and once it dries, a second smear is made on the first smear. This process is repeated three to five times.

      - For smears from sputum, pus or body fluids – purulent portion is used to prepare the smear.

      - For tissues, a small tissue is placed in a sterile petri dish or clean glass slide and a minute piece is sliced with the help of a sterile glass blade. The small piece is smeared on the glass slide or crushed between two glass slides to get the smear.

      - A tiny portion of the bacterial or fungal yeast colony is touched with an inoculating wire to make a smear in a drop of saline solution.

Procedure

      - Make a thin smear of the specimen on a clean-labeled micro slide and allow it to air-dry.

      - Fix the smear by gentle heat by passing the slide 3 or 4 times through the flame of Bunsen burner so that the smear does not wash off during the staining procedure.

      - Flood the smear with the Crystal Violet Stain. Let it stand for 1 minute.

      - Pour off the stain. Wash with water.  Shake off the excess water.

      - Flood with Gram's Iodine solution. Let it stand for 1 minute.

      - Pour off.  Wash with water. Shake off the excess water.

      - Pick up the slide and while holding it in a slanting position, pour decolouriser over the smear just until the purple colour runs off. Wash quickly with tap water. Usually this is sufficient for satisfactory decolourization, though thick smears may require repetition of this step. Shake off the excess water.

      - Flood the smear with Saffranine. Let stand for 1 minute.

      - Pour off the stain. Wash with water. Shake off the water. Drain.

      - Culture smears may be blotted dry.

- Quality Control Procedure

Internal Quality Control

      - On a slide, a mixed smears Staphylococcus aureus ATCC 29213 (Gram positive cocci) and ATCC Escherichia coli ATCC 25922 (Gram negative bacilli) are made.

      - The slide with control and test smears is stained by Gram’s staining.

      - The appearance of purple-colored Gram-positive cocci and pink-colored Gram-negative bacilli in the control smear indicate proper staining technique and stained test smear is compared with it.

      - Positive control ATCC Strains of Staphylococcus aureus (29213).

      - Negative control ATCC Strains of Escherichia coli (25922).

External Quality Control

      - EQAS participation – IAMM EQAS three times a year.

- Interfering Substances

- Sample collection process and transportation should be done appropriate to the sample to avoid sample contamination with the commensal flora and environmental contaminants. Adequate aseptic precautions are to be taken to avoid contamination during smear and staining process. Timing deviation in any of the steps leads to erroneous staining. Care to be taken during fixing of smear. Gram-positive organisms lose their ability to retain the crystal violet and stain Gram-negative due to over decolorizing or due to too old Grams iodine or old culture or due to overheating during the fixation process. Gram negative organism may appear as Gram positive due to inadequate decolorisation.

- Result Calculation

Presence or absence of Host Cells – Epithelial cells, pus cells, lymphocytes and quantification - scanty, few, moderate, plenty.

Presence or absence of microorganisms – Gram reaction, size, shape, arrangement.

Presence of capsule, spore, hyphae like structures; inside or outside host cells and quantification - scanty, few, moderate, plenty.

Grading of Gram Stain of Sputum

- NOTE: Total Score should be a positive integer.

- Biological Reference Interval

- N/A.

- Reportable Interval

- 4 Hours

- Procedure when Results are not in Range

- When the results are found inappropriate to the sample type, the sample is re-stained or checked with a repeat sample as per individual sample and growth discretion.

- Critical Alert Values

- All significant organisms observed in samples from sterile body sites is informed as critical alert.

- Laboratory Clinical Interpretation

- Specific clinical interpretation is given as per sample type and microscopic picture.

- Potential Source of Variability

- The following factors play a crucial role in the result variations and appropriate care should be taken to eliminate them:

prior antibiotic usage by patient

sample type

sample collection

sample transport temperature

time to sample collection and inoculation

portion of sample used for inoculation

culture media

loop size and sterility

temperature and time of incubation.

- References

Practice of Medical Microbiology – Cruickshank

Diagnostic Microbiology by Monica Cheesbrough