SOP - 010

PROCEDURE FOR

MAINTAINING ATCC CULTURE

- TABLE OF CONTENTS

TABLE OF CONTENTS	2

APPROVAL AUTHORITY	3

RECORD OF AMENDMENTS	4

DISTRIBUTION LIST	5

LIST OF ABBREVIATIONS	6

1.	Objective	7

2.	Scope	7

3.	Responsibilities	7

4.	Definition	7

5.	Test Principle	7

6.	Specific Performance Characteristics	7

7.	Type of Sample	7

8.	Patient Preparation	7

9.	Type of Container & Additives	7

10.	Equipment & Reagents	8

11.	Safety	8

12.	Calibration Procedure	8

13.	Test Procedure	8

14.	Quality Control Procedure	10

15.	Interfering Substances	10

16.	Result Calculation	10

17.	Biological Reference Interval	10

18.	Reportable Interval	10

19.	Procedure when Results are not in Range	10

20.	Critical Alert Values	10

21.	Laboratory Clinical Interpretation	11

22.	Potential Source of Variability	11

23.	References	11

- APPROVAL AUTHORITY

| Prepared by: | Head of Department | Dr. Sadiya Shahnaz Name | Date: 14/11/2024 |
| --- | --- | --- | --- |
|  |  | Signature |  |
| Reviewed & Approved by: | Laboratory Head | Dr. Abdul Aleem Name | Date: 14/11/2024 |
|  |  | Signature |  |
| Issued by: | Quality Manager | Dr. Sai Varun Name | Date: 14/11/2024 |
|  |  | Signature |  |

- RECORD OF AMENDMENTS

| S. No. | Ver. No. | Ver. Date | Rev. No. | Rev. Date | Description of Changes |
| --- | --- | --- | --- | --- | --- |

| Document Copy No. | Name / Designation of the holder of the document copy | Type of Copy | Version No. | Revision No. |
| --- | --- | --- | --- | --- |
| 01/10 | Managing Director | Controlled Copy | 01 | 00 |
| 02/10 |  |  |  |  |
| 03/10 |  |  |  |  |
| 04/10 |  |  |  |  |
| 05/10 |  |  |  |  |
| 06/10 |  |  |  |  |
| 07/10 |  |  |  |  |
| 08/10 |  |  |  |  |
| 09/10 |  |  |  |  |
| 10/10 |  |  |  |  |

- DISTRIBUTION LIST

- LIST OF ABBREVIATIONS

| S. No. | Abbreviations | Expansion |
| --- | --- | --- |
| 1 | ATCC | American Type Culture Collection |
| 2 | MI | Microbiology |
| 3 | NABL | National Accreditation Board for Testing and Calibration Laboratories |
| 4 | SOP | Standard Operating Procedure |
| 5 | TDPL | TRUSTlab Diagnostics Private Limited |

- Objective

  - This SOP describes the procedural steps to find the traceability of maintaining the ATCC culture in the Laboratory.

- Scope

This procedure applies to all staff of the Microbiology Department tasked with the duty of performing these tests.

- Responsibilities

It is the responsibility of the technicians of the Microbiology Department to comply with this SOP.

HOD shall ensure implementation.

The Quality Manager is responsible for monitoring, reviewing and ensuring compliance with this procedure.

All other TRUSTlab staff involved in this process (including contract personnel) are responsible for complying with this procedure.

- Definition

- An ATCC strain refers to a strain of microorganism, such as a bacterium, yeast, or fungus, that has been deposited with the American Type Culture Collection (ATCC).

- Procedural Principle

- N/A

- Specific Performance Characteristics

- N/A.

- Type of Sample

- N/A.

- Patient Preparation

- N/A.

- Type of Container & Additives

- N/A.

- Equipment & Reagents

      - Laminar air flow

      - Bunsen burner

      - Inoculation loop

      - Microscope

- Safety

Observe all precautions, as done for handling any biological specimen.

Wear face mask, apron and disposable gloves, while transferring and handling the specimen

Inoculate the specimen under Biosafety cabinet only.

Spills should be decontaminated promptly with Sodium Hypochlorite or any other suitable disinfectant.

Suitable infection prevention precautions to be followed.

- Calibration Procedure

- N/A.

- Procedure

Input

    - ATCC strains of bacteria

      - Escherichia coli - ATCC 25922

      - Pseudomonas aeruginosa - ATCC 27853

      - Staphylococcus aureus - ATCC 29213

      - Enterococcus faecalis – ATCC 29212.

    - Culture media / Agar slants prepared in house.

    - Inoculation loop

Output

    - Isolation of pure culture.

    - Full characterization of each strain to establish identity.

    - Careful preservation and storage protocols to maintain the culture safely and effectively.

Monitoring and Measurement

    - Properly maintained records and registers.

    - Checking of Culture media prepared in house for their efficiency in supporting growth.

    - To check the commercially acquired antimicrobial discs.

Preservation of ATCC Stock Culture

    - The method for maintaining an ATCC stock culture is to subculture twice a month.

Stabbing:

      - Cultures are stabbed deeply into agar using an inoculating needle.

      - The stabs are incubated until visible cultures form and then sealed overlaid with half an ml of sterile paraffin and stored at -200C temperature.

Slant method:

      - Cultures are streaked on to the surface of the solid slant medium in a tube.

      - The slants are incubated until visible culture grows and then sealed overlaid with half an ml of sterile paraffin and stored at -200C temperature.

Sub-Culture of ATCC Strains:

      - Flame an inoculating loop and allow it to cool without touching any surface.

      - Touch multiple colonies from the ATCC culture and inoculate into Nutrient broth and incubate at 37°C for 4 hours or till growth matches 0.5 McFarland’s turbidity control. Then inoculate appropriate Agar plates and slants.

      - All biochemical tests are put up for identification twice a month.

      - Antimicrobial susceptibility testing is done using commercially acquired discs.

      - Incubate the inoculate slants at 37°C for 24-48 hours.

      - After incubation observe the slants for growth and perform Gram stain.

Few reliable methods for preserving some bacterial isolates are summarized as follows:

    - Escherichia coli ATCC 25922:

- Escherichia coli ATCC 25922 is sub-cultured on Nutrient agar & MacConkey agar once a week and then used for various purposes.

    - Pseudomonas aeruginosa ATCC 27853:

- Pseudomonas aeruginosa ATCC 27853 is sub-cultured on Nutrient agar & MacConkey agar once a week and then used for various purposes.

    - Staphylococcus aureus ATCC 29213:

- Staphylococcus aureus ATCC 29213 is sub-cultured on Nutrient agar & Blood agar once a week. A strain of Staphylococcus that produces β-haemolysin and is suitable for the CAMP test for identifying group B beta-hemolytic streptococci is desirable. Staphylococci will grow on slants of heart infusion agar, placed under oil, and held at 370C.

    - Enterococcus faecalis ATCC 29212

- Enterococcus faecalis ATCC 29212 is sub-cultured on Blood agar & Chocolate agar plates once in a fortnight for further use for various purposes.

Documentation:

- Updation will be done for all ATCC subcultures and ATCC Stock cultures in registers and are maintained permanently.

- Quality Control Procedure

- N/A

- Interfering Substances

- N/A.

- Result Calculation

- N/A

- Biological Reference Interval

- N/A.

- Reportable Interval

- N/A.

- Procedure when Results are not in Range

- N/A.

- Critical Alert Values

- N/A.

- Laboratory Clinical Interpretation

- N/A.

- Potential Source of Variability

N/A.

- References

Practical Medical Microbiology by Mackie & McCartney 14th Edition.