SOP - 009

PROCEDURE FOR

TYPHIDOT IGG/IGM  RAPID TEST

- TABLE OF CONTENTS

TABLE OF CONTENTS	2

APPROVAL AUTHORITY	3

RECORD OF AMENDMENTS	4

DISTRIBUTION LIST	5

LIST OF ABBREVIATIONS	6

1.	Objective	7

2.	Scope	7

3.	Responsibilities	7

4.	Definition	7

5.	Test Principle	7

6.	Specific Performance Characteristics	7

7.	Type of Sample	8

8.	Patient Preparation	8

9.	Type of Container & Additives	8

10.	Equipment & Reagents	8

11.	Safety	8

12.	Calibration Procedure	9

13.	Test Procedure	9

14.	Quality Control Procedure	9

15.	Interfering Substances	9

16.	Result Calculation	10

17.	Biological Reference Interval	10

18.	Reportable Interval	10

19.	Procedure when Results are not in Range	10

20.	Critical Alert Values	10

21.	Laboratory Clinical Interpretation	10

22.	Potential Source of Variability	11

23.	References	11

- APPROVAL AUTHORITY

| Prepared by: | Head of Department | Dr. Sadiya Shahnaz Name | Date: 14/11/2024 |
| --- | --- | --- | --- |
|  |  | Signature |  |
| Reviewed & Approved by: | Laboratory Head | Dr. Abdul Aleem Name | Date: 14/11/2024 |
|  |  | Signature |  |
| Issued by: | Quality Manager | Dr. Sai Varun Name | Date: 14/11/2024 |
|  |  | Signature |  |

- RECORD OF AMENDMENTS

| S. No. | Ver. No. | Ver. Date | Rev. No. | Rev. Date | Description of Changes |
| --- | --- | --- | --- | --- | --- |

| Document Copy No. | Name / Designation of the holder of the document copy | Type of Copy | Version No. | Revision No. |
| --- | --- | --- | --- | --- |
| 01/10 | Managing Director | Controlled Copy | 01 | 00 |
| 02/10 |  |  |  |  |
| 03/10 |  |  |  |  |
| 04/10 |  |  |  |  |
| 05/10 |  |  |  |  |
| 06/10 |  |  |  |  |
| 07/10 |  |  |  |  |
| 08/10 |  |  |  |  |
| 09/10 |  |  |  |  |
| 10/10 |  |  |  |  |

- DISTRIBUTION LIST

- LIST OF ABBREVIATIONS

| S. No. | Abbreviations | Expansion |
| --- | --- | --- |
| 1 | EDTA | Ethylenediaminetetraacetic Acid |
| 2 | ELISA | Enzyme-Linked Immunosorbent Assay |
| 3 | EQAS | External Quality Assurance Scheme |
| 4 | IgG | Immunoglobulin G |
| 5 | IgM | Immunoglobulin M |
| 6 | NABL | National Accreditation Board for Testing and Calibration Laboratories |
| 7 | SE | Serology |
| 8 | SLE | Systemic Lupus Erythematosus |
| 9 | SOP | Standard Operating Procedure |
| 10 | TDPL | TRUSTlab Diagnostics Private Limited |

- Objective

  - This SOP describes the procedure to test for Salmonella antibodies IgG/IgM antibodies by the rapid immunochromatography method.

- Scope

This procedure applies to all staff of the Serology Department tasked with the duty of performing these tests.

- Responsibilities

It is the responsibility of the technicians of the Serology Department to comply with this SOP.

HOD shall ensure implementation.

The Quality Manager is responsible for monitoring, reviewing and ensuring compliance with this procedure.

All other TRUSTlab staff involved in this process (including contract personnel) are responsible for complying with this procedure.

- Definition

The Typhoid IgG/IgM Rapid Test is a lateral flow immunoassay that detects and differentiates between IgG and IgM antibodies in the blood. It can be used to help diagnose typhoid fever.

- Test Principle

Monoclonal antihuman IgG and IgM are immobilized on a porous nitrocellulose membrane at individual test lines – G and M respectively. When assay buffer is added Colloidal gold conjugated inactivated Salmonella antigens in the antigen pad react with Salmonella serotype typhi IgG/ IgM antibodies if present in the Sample which further bind to immobilised antihuman IgG/ IgM antibodies giving a distinct pinkish purple band against a white background at the test region – IgG / IgM respectively. At the control region (‘C’) a “Built-in Quality Control band” has been devised to confirm the proper functioning of the device, reagent and correct procedural application.

- Specific Performance Characteristics

| Sample panel | S.Typhi IgG | S.Typhi IgM |
| --- | --- | --- |
| Sensitivity | 100% | 100% |
| Specificity | 100% | 100% |

- Type of Sample

- Serum or Plasma must be used for testing. Should a delay in testing occur, store the sample at 2-8°C. Samples can be stored for up to a week.

- Patient Preparation

- No special preparation of the patient is required prior to specimen collection by approved techniques.

- Type of Container & Additives

- Collect 2 ml of venous blood in a plain red topped/ yellow gel tube / EDTA vacutainer.

- Equipment & Reagents

Test Device

Timer

Buffer

- Safety

The use of disposable gloves and proper biohazardous clothing is STRONGLY RECOMMENDED while running the test.

In case there is a cut or wound on the hand, DO NOT PERFORM THE TEST.

Do not smoke, drink or eat in areas where specimens or kit reagents are being handled.

Tests are for in vitro diagnostic use only and should be run by competent personnel only.

Do not pipette by mouth.

All materials used in the assay and samples should be decontaminated in 1% sodium hypochlorite solution for 30-60 min. They should be disposed of in accordance with established safety procedures.

Wash hands thoroughly with soap or any suitable detergent, after the use of the kit. Consult a physician immediately in case of accident or contact with eyes, in the event that contaminated material is ingested or comes in contact with skin punctures or wounds.

Spills should be decontaminated promptly with Sodium Hypochlorite or any other suitable disinfectant.

- Calibration Procedure

- Centrifuge calibration is done on an annual basis by a NABL accredited calibration agency.

- Test Procedure

- Allow the test device and specimen to equilibrate to room temperature (15 - 300c) prior to testing.

Add one drop of serum or plasma or whole blood to the sample region

Add one drop of Buffer Solution in the sample region

READ RESULT at 20 minutes.

- IMPORTANT: It is important to allow each solution to soak in the test device before adding the next solution.

- Quality Control Procedure

Each testing device has an inbuilt Control Band to confirm the validity of the device. The appearance of the control dot after completing the procedural test indicates a valid test.

All kits and lots will be tested as per reagent acceptability criteria.

EQAS participation – Interlaboratory comparison.

- Interfering Substances

- Kit works best when used with fresh samples, however the frozen or viscous samples can also perform well if the following instructions are strictly adhered to

Frozen Samples

      - Allow the sample to thaw in a vertical position in the rack. Mix the sample thoroughly. If particles are seen, allow them to settle at the bottom or if a centrifuge is available, the sample can be centrifuged at 3500 rpm. for 15 minutes.

      - Insert the dropper just below the top surface of the sample and withdraw one drop of the sample.

Thick or Viscous Samples

      - Whenever possible, a clear specimen should be used. However, viscous, thick or turbid samples which may sometimes take more than 40-60 seconds to flow through the membrane should be centrifuged at 3500 rpm. for 15 minutes and retested on a fresh device to avoid inconsistent results.

- Result Calculation

NEGATIVE

- The control line (C) is only visible on the test device. No IgG and IgM antibodies were detected.

IgM POSITIVE

- The control line (C) and IgM line (M) are visible on the test device.

IgM and IgG POSITIVE

- The control line (C), IgM line (M) and IgG are visible on the test device.

IgG POSITIVE

- The control line (C) and IgG are visible on the test device.

INVALID

- The test is considered invalid if the control dot is not formed. Test should be repeated with a new device.

- Biological Reference Interval

- Non-Reactive

- Reportable Interval

- 4 Hours

- Procedure when Results are not in Range

- N/A.

- Critical Alert Values

- N/A.

- Laboratory Clinical Interpretation

- Typhoid disease is caused by a bacterium, Salmonella serotype typhi transmitted by contaminated food and water. The disease usually presents with high grade fever, headache, body pains, nausea, vomiting. The organism causes bacteremia and septicaemia which can result in complications like intestinal ulcers, meningitis, arthritis and liver abscess. The disease is usually identified by detection of antibodies or by isolation of the organism by blood culture. Results with immunochromatography which is a screening test have to be confirmed by ELISA method or by culture techniques.

- Potential Source of Variability

A negative result cannot exclude a recent Salmonella serotype typhi infection.

A negative result can occur if the quantity of antibody present is below the detection limits. Results should be correlated clinically and with other laboratory investigations and may be repeated after 3-5days.

Cross reactivity may be observed with high titre of Rheumatoid arthritis or SLE antibodies.

Optimum test performance depends on strict adherence to the test procedure as described in this manual. Any deviation from test procedure may lead to erratic results.

- References

- TYPHI IGG/IGM TEST – J’Mitra Kit Insert